Technical level: what it adds to the concept

SOW1 - SOW2 secure communication. The 30 technical requirements (TECH_CS_CONCEPT) compared with the 25 concept requirements (CS_CONCEPT).

1. In short

The concept level says what must be true. The technical level says how to verify it, with numbered test steps.

18 of the 30 have the same text as their concept requirement. 5 differ slightly. 4 add new detail. 3 are new (row configuration).

Of our 87 open SIQs, 3 are now answered and 15 are partly answered. 69 stay open.

The technical level does not define the registers, frame layout, timers or reactions that we ask about. It also adds new gaps (section 4).

2. The 30 technical requirements

Colour: green = same text, blue = small difference, yellow = adds detail, orange = new. Order as in your list (T1 to T30).

| # | Technical | Concept | Type | What it says or adds |
| --- | --- | --- | --- | --- |
| T1 | MVC-106582 | MVC-119709 | Same text as concept | Same text. Wrong footer tag and changed frame data must both fail; full 128-bit compare. |
| T2 | MVC-106575 | MVC-103031 | Adds detail (new information) | Freshness, ordering and sequence check with the header counter; "counter-validation policy" not defined. Content is closer to MVC-113703. |
| T3 | MVC-106563 | MVC-107642 | Adds detail (new information) | New: RSASSA-PSS with SHA-256 and a 2048-bit key. Concept MVC-107641 has the same topic but no linked technical requirement. |
| T4 | MVC-106560 | MVC-103042 | Same text as concept | Same text. Steps include injecting a TRNG failure (software level). |
| T5 | MVC-106578 | MVC-103034 | Small wording difference | Adds: over I2C, the term Row_info, and no transmission if authentication or encryption fails. Message layout is "specified" but not defined. |
| T6 | MVC-106569 | MVC-103051 | Small wording difference | Concept: configured maximum. Here: fixed three attempts. |
| T7 | MVC-106581 | MVC-103044 | Same text as concept | Same text. SSK not used for protected traffic; a new establishment can start again ("recovery conditions" not defined). |
| T8 | MVC-106562 | MVC-103063 | Same text as concept | Same text and criteria. One step is about Secure Boot, which is another feature. |
| T9 | MVC-106567 | MVC-103046 | Same text as concept | Same text. Counts requests on I2C; step 5 says "configured limit" while the text says three. |
| T10 | MVC-106566 | MVC-103053 | Small wording difference | Concept: retry the process. Here: retransmit the authentication request up to three times. |
| T11 | MVC-106573 | MVC-103036 | Same text as concept | Same text. Detailed steps per trigger; the old SSK must no longer be accepted. |
| T12 | MVC-106561 | MVC-107642 | Adds detail (new information) | New: SOW2 requests Device_n, KID and Sig_device over I2C after reset or before the session. Not in the concept. |
| T13 | MVC-113727 | MVC-113703 | Same text as concept | Same text. Boundary tests; LastAcceptedCounter is updated after the frame is authenticated. |
| T14 | MVC-106564 | MVC-119710 | Same text as concept | Same text. A rejected frame must not reach downstream processing as trusted. |
| T15 | MVC-106571 | MVC-113706 | Same text as concept | Same text. Lists the interfaces to check (diagnostic, logs, debug, communication, memory). |
| T16 | MVC-109120 | MVC-119708 | Same text as concept | Same text. Frames are trusted only after session set-up and the integrity and freshness checks. |
| T17 | MVC-109121 | MVC-113702 | Small wording difference | Says "protected" commands. Adds a counter maximum and rollover policy and the start value after a new session, but defines neither. |
| T18 | MVC-106568 | MVC-103057 | Same text as concept | Same text. New SSK per session; no key generation after a failed authentication. |
| T19 | MVC-113726 | MVC-103031 | Same text as concept | Same text. A malformed or missing header counter must not authorize a frame. |
| T20 | MVC-106572 | MVC-113705 | Same text as concept | Same text. Exactly 128 bits from the footer; a malformed footer is not treated as extracted. |
| T21 | MVC-106574 | MVC-103055 | New: row-information configuration | New: row range limit start + 1 + (interval + 1) x (total - 1) <= image + embedded data lines. |
| T22 | MVC-106580 | MVC-103055 | Same text as concept | Same text as the first part of the concept (the concept adds a Samsung note). Criteria need the private key to decrypt. |
| T23 | MVC-106576 | MVC-103055 | New: row-information configuration | New: row start 0 or 1. Start 0 puts the header line with the frame counter into the CMAC input. |
| T24 | MVC-108741 | MVC-103055 | New: row-information configuration | New: row interval at least 10 (merge modes) or 30 (line reorder RAW12). |
| T25 | MVC-106583 | MVC-103033 | Same text as concept | Same text. Needs a known SSK and an independent reference tag. |
| T26 | MVC-108740 | MVC-113701 | Same text as concept | Same text. Needs a known SSK; "specified authenticated input" is not defined. |
| T27 | MVC-106565 | MVC-107642 | Adds detail (new information) | New: the signature covers Device_n and KID; the root key is RSA-2048. |
| T28 | MVC-106570 | MVC-119707 | Same text as concept | Same text. The tag follows the command; the field order is "specified" but not defined. |
| T29 | MVC-109119 | MVC-119714 | Small wording difference | Says "protected" control commands; the concept says all control commands. |
| T30 | MVC-106577 | MVC-103040 | Same text as concept | Same text. A missing or malformed key must not be used; the "reference key" question returns. |

3. What it gives our open SIQs

3 answered, 15 partly answered. The comment "partly" means the technical requirement helps, but the value or policy is still not defined.

| Concept requirement | SIQ | Status | What the technical level gives |
| --- | --- | --- | --- |
| MVC-107642 | SIQ-2 | Answered | The signed message is Device_n and KID (MVC-106565). |
| MVC-107641 | SIQ-3 | Answered | RSA key size 2048 bits (MVC-106563). |
| MVC-113703 | SIQ-2 | Answered | LastAcceptedCounter is updated after the frame is authenticated (MVC-113727). |
| MVC-103053 | SIQ-1 | Partly | SOW2 retransmits the authentication request (MVC-106566). What counts as failed is still open. |
| MVC-103051 | SIQ-2 | Partly | A new establishment can start again, under undefined "recovery conditions" (MVC-106581). |
| MVC-103051 | SIQ-3 | Partly | The technical level says a fixed three attempts (MVC-106569); the concept says configured. |
| MVC-103044 | SIQ-1 | Partly | The SSK is not used for protected traffic (MVC-106581); deletion is not stated. |
| MVC-119709 | SIQ-3 | Partly | With row start 0 the header line with the frame counter is part of the CMAC input (MVC-106576). |
| MVC-119709 | SIQ-6 | Partly | "Incorrect" = changed footer tag, "modified" = changed frame data (MVC-106582); "invalid" is not covered. |
| MVC-103031 | SIQ-3 | Partly | Invalid counter data must not authorize a frame (MVC-113726, MVC-106575); the policy is not defined. |
| MVC-113705 | SIQ-1 | Partly | An invalid tag is not treated as extracted (MVC-106572); the policy is not defined. |
| MVC-119708 | SIQ-2 | Partly | Trusted only after session set-up plus integrity and freshness checks (MVC-109120); the start of the stream is open. |
| MVC-119714 | SIQ-1 | Partly | The technical level says "protected" commands only (MVC-109119). |
| MVC-119707 | SIQ-1 | Partly | The tag follows the command (MVC-106570); the field format is not defined. |
| MVC-103042 | SIQ-5 | Partly | Steps include injecting a TRNG failure (MVC-106560), so a software-level negative test is expected. |
| MVC-103046 | SIQ-1 | Partly | Retransmissions are counted on I2C with the response suppressed (MVC-106567); the expected response is not defined. |
| MVC-107641 | SIQ-5 | Partly | Inspection of the configuration plus a modified-data test (MVC-106563). |

4. New gaps found at the technical level

Concept MVC-107641 (RSASSA-PSS, SHA-256) has no linked technical requirement. MVC-106563, which covers it, is linked to MVC-107642.

Five technical requirements are linked from the concept but are not in our export: MVC-113724, MVC-106579, MVC-108739, MVC-113723, MVC-113725.

Fixed or configured retry limit: "three" in the text, "configured limit" in MVC-106567 step 5 and in concept MVC-103051. Three attempts (MVC-106569) and three retransmissions (MVC-106566) differ by one.

Three row-configuration technical requirements are linked to the encryption concept MVC-103055. The concept has no row-selection requirement.

MVC-106575 is linked to MVC-103031 (extract counter) but its content is freshness validation (MVC-113703).

Several criteria refer to things that do not exist yet: "approved interface specification" (MVC-106580), "specified message layout" (MVC-106578, MVC-106570), "counter-validation policy" (MVC-106575), "rollover policy" (MVC-109121).

MVC-106563 numbers its criteria 1, 2, 4, 5 (step 3 is missing). MVC-109119 says "protected" commands, the concept says all commands.

5. Next

Review the 30 technical requirements. For those with the same text, the comment is "Similar to concept REQ: MVC-xxxxx". Only differences and new items get their own SIQs.

Send the five missing technical requirements (MVC-113724, MVC-106579, MVC-108739, MVC-113723, MVC-113725).