# Repository analysis

Snapshot of main at the first push: 13,367 files, about 2.8 GB. All content is under Components Datasheets/.

## What is in the repo

| Folder | Size | What it is | Cybersecurity relevance |
| --- | --- | --- | --- |
| Gitlab Files/ | 2.5 GB | Copy of the HIL test-automation project 11_p_01467_mekra_vision_controller (iTestStudio, Python test pool, vendored libraries, bench-equipment manuals, past test reports) | High. It is the place to build cybersecurity test scripts. SOME/IP, DHCP and service-discovery libraries already exist |
| Functional Safety-Development/ | 130 MB | FuSa work: FSRs, TSRs, technical safety concept, DFMEA, DFA/FTA, SOW1 requirements on SOW2, OPL management sheets, Volvo/Samsung camera interface specs | High. The SOW1 interface spec defines the camera authentication scheme (see 02_Security_Attack_Surface.docx). Safe-state and DFMEA data feed fault-handling tests |
| SoC (CV22FS)/ | 89 MB | Ambarella CV22FS datasheets, Hardware Programming Reference Manual (v1.2 and v1.5), system hardware guide | High. Boot configuration (secure boot strap), OTP, JTAG/UART pins |
| SOW1/ | 23 MB | Samsung S5K3B6SA/SX image-sensor datasheets, app note, imager settings, Volvo camera interface draft | High. Sensor-side security (AES-CMAC, key fusing, signed EEPROM) |
| PHYs/ | 19 MB | NXP TJA1104, TJA1121 (with MACsec core user manual), Microchip LAN887x | Medium. MACsec capability. The board uses TJA1104A (100 Mbit) and TJA1121A (1 Gbit) |
| Test environment description/, Analysis/ | 9 MB | Bench description and seven analyses (equipment capability, interface/HW coverage matrix, TSR read/write, register map index) | High. Shows what the bench can already do |
| Schematics/ | 7 MB | MVCU01K BOM02K schematic prints | High. JTAG and UART nets, SPI NOR, test points |
| System Architecture/, TSRs/, SSRs/ | 11 MB | Polarion exports: architecture, technical safety concept, software requirements (SRS, includes chapter 23 Cybersecurity) | Very high. Main requirement source |
| Serializer/, Deserializer/, PMIC/, Flash/, DRAM/ | 16 MB | MAX96717, MAX96714 (plus safety manual), MPQ70165FS, IS25WP01G, DRAM datasheets | Medium to high. Interfaces and configuration |
| _page_links/ | 144 KB | HTML redirectors so Word links open a PDF at a given page | Keep as is, because Word documents depend on it |

## Findings

- The repo is mostly a document archive plus a test framework, with no cybersecurity artefacts yet. The only cybersecurity content is SRS chapter 23, the architecture elements UCM_OA and CS_APPL_OA, and the SOW1 interface spec. This workspace fills that gap.

- About 90% of the size is the vendored test framework (Gitlab Files): 1,294 DLLs, 103 EXEs, 1,466 .pyc files, a virtual environment, and binary tools (HexXtract_Tool.exe at 55.7 MB and avcodec-58.dll at 89.3 MB exceed GitHub's recommended 50 MB). It works, but it makes clones slow. Consider Git LFS or moving it to its own repo.

- Several documents exist in multiple versions with no marker of which is current: four revisions of the S5K3B6SA datasheet (plus an S5K3B6SX preliminary), two CV22FS HW programming manuals (1.2 and 1.5), five FSR versions (FSRs NEW, _EVEN_NEWER, _PREreviewed_v.0.1/0.4), two TSC versions. Cybersecurity work should cite the version it used. 03_Source_Index.docx lists the version chosen.

- Leftover and local-only files are in the repo: .drawio.bkp and .dtmp lock files, *.bak result files, New Text Document.txt, and a 12.9 MB Outlook .msg at the top level. A .gitignore for these would help.

- Confidential material. Several documents are customer or supplier internal (Volvo interface spec, Samsung security policy references, Ambarella manuals, Polarion exports). The repository must stay private.

- Some referenced documents are not in the repo: Security_Policy_REV1.00.pdf and CYBERSECURITY_KEY.FUSING_GUIDE.pptx (Samsung key provisioning, cited in the Volvo interface spec). They are needed to test the SOW1 authentication chain.

- The SRS is Draft. All 249 cybersecurity requirements are Draft, and 52 have open [X] parameters. Test design can start, but thresholds cannot be final.

## Not changed

No file was moved, renamed or deleted. If you later want a physical restructuring of the document folders, the risk is the relative links in the Word documents (_page_links and the docx files in Analysis/ and Functional Safety-Development/). A restructure would need those links rewritten as part of the same change.