# Security attack surface of the MVC (SOW2)

Derived from the architecture export (System Architecture rev. 670544, 2026-08-02), the SRS (rev. 731696, 2026-10-08), the schematics (MVCU01K BOM02K), the CV22FS HW Programming Reference Manual 1.5 and the SOW1 Camera Interface Specification. Where something is my inference and not stated in a document, it says (to confirm).

## Architecture elements that are already cybersecurity-specific

| Element | Source | Meaning |
| --- | --- | --- |
| UCM_OA with ppSecurity (CSRuntime-SoftwareIF) and ppFlash | System Architecture | Update and configuration management (software download). Talks to the security runtime and to the flash |
| CS_APPL_OA with ppSecureStorage, ppSecureComm. | System Architecture | Cybersecurity application: secure storage and secure communication |
| SecurityData (CSProgrammable) on flash and on the flash-access port | System Architecture | Security data region in the SPI flash |
| ppSPIneedleAdapter and ppFlashAccess (SPI) | System Architecture | Physical SPI access to the flash. This is a production or service access path |
| ProductionData, RuntimeData | System Architecture | Production data block (integrity) and runtime data in NVM |

## Interfaces and what to test

| # | Interface | Evidence | Threat classes | Requirement source | Test area |
| --- | --- | --- | --- | --- | --- |
| 1 | Ethernet to vehicle ECU (100/1000BASE-T1, PHY TJA1104A / TJA1121A, RGMII to SoC) | Schematic, architecture ppEthernet, SRS ch.5 to 8 | Spoofing, tampering, replay, flooding, malformed input | SRS ch.5 (Ethernet, DHCP client), ch.6, ch.7 (SOME/IP), ch.23.1 (DTLS 1.3) | CS-DTLS, CS-SOMEIP, CS-ETH |
| 2 | DTLS 1.3 session with the vehicle ECU (PSK, ECDHE, TLS_AES_128_GCM_SHA256) | SRS 23.1, 90 requirements | Handshake downgrade, replay, key reuse, weak randomness, fragmentation abuse | SRS 23.1.1 to 23.1.12 | CS-DTLS |
| 3 | SOME/IP camera service (getCamIdentification, setCamControl, setDataBlock, subscribeRoiVideo, etc.) | SRS 7.3 | Unauthenticated control, malformed headers, exclusivity lock abuse, flood | SRS 7.1.3, 7.3.x | CS-SOMEIP (existing tests cover header validation and flood) |
| 4 | Software download and rollback (UCM: TransferStart, TransferData, TransferExit, ProcessSwPackage, Activate, Rollback, RevertProcessedSwPackages) | SRS 7.4.2 | Unsigned or modified package, downgrade, interrupted update, unauthorised rollback | SRS 7.4.2, 23.4 (boot verification, anti-rollback) | CS-UCM, CS-BOOT |
| 5 | GMSL2 link to the SOW1 camera (MAX96714 deserializer, MAX96717 serializer): I2C tunnel, GPIO forwarding, video | Schematic, SRS ch.15, SOW1 interface spec | Spoofed camera, manipulated I2C configuration, injected frames | SRS 23.2 (57 requirements), ch.20.3 (I2C manipulation use cases) | CS-SOW1, CS-GMSL |
| 6 | SOW1 authentication chain: Samsung Root public key verifies the imager's RSA device key, ECU generates session key plus row selection and sends it RSA-encrypted, imager embeds AES-CMAC in each frame footer (from frame 18 on), signed EEPROM verified with the Bosch public key | SOW1 Camera Interface Spec (Volvo D040574), SRS 23.2 | Counterfeit camera, replayed frames, stale session key, unsigned EEPROM | SRS 23.2.1 to 23.2.7 | CS-SOW1 |
| 7 | Debug ports: ISP_JTAG (test points TP1200 to TP1203), the UART (APB) and UART0/UART1 nets on the SoC. UART2 and UART3 appear only as alternate functions on SMIO pins | Schematic | Unauthorised debug access, unlock bypass | SRS 23.3 (39 requirements): JTAG locked by default in production, unlock only through UART challenge-response with fresh challenge, nonce, salt and session ID, validity period 30/60/120 s [X] | CS-DBG |
| 8 | External SPI NOR flash (IS25WP01G-RILA3, NORSPI_* nets) and secure boot | Schematic, SoC HW PRM section 2 (POC bits, secure boot strap, signed image size field, OTP ROM), SRS 23.4 (35 requirements) | Modified boot image, rollback, flash read-out | SRS 23.4: SHA-256, RSA 2048/3072/4096 [X], public key in OTP, boot stages Safety BST, Safety BLD, Safety OS, A53 BLD, ATF, OS image | CS-BOOT |
| 9 | DRAM (scrambling AES engine, write-only key register, address translation tables per DRAM client) | SRS 23.5 (28 requirements) | Cold-boot or probing attack, DMA access outside range | SRS 23.5 | CS-DRAM |
| 10 | NVM data blocks (production data, software configuration, ImVer) | SRS ch.13, architecture | Tampering with production or configuration data | SRS ch.13, 23.4 | CS-NVM |
| 11 | MACsec capability of the PHYs (TJA1104A, TJA1121A) and MACsec bench switch (Technica TE-2004, MKA) | PHY datasheets, bench description | Layer-2 spoofing, unauthorised links | (to confirm) whether MACsec is used on this product. No SRS requirement for it was found | CS-ETH (only if used) |
| 12 | Time sync (gPTP) and FSYNC | SRS ch.9, 10 | Spoofed time master, frame-timing manipulation (also a safety goal) | SRS ch.9, 10 | CS-ETH |
| 13 | Video stream (RTP) and SEI frame tagging | SRS ch.8, 11 | Stream injection, tag manipulation | SRS ch.8, 11, 23.1 (to confirm whether the stream is inside DTLS) | CS-DTLS / CS-ETH |

## Open points

- Which traffic runs inside the DTLS session: only control and SOME/IP, or also RTP video? SRS 23.1 says "SOW2 to Vehicle ECU secure communication interface" without naming the payloads.

- Is MACsec used? Hardware and bench support it, but no requirement was found.

- ECU lifecycle states (production, development, service). SRS 23.3 refers to "production mode" and a "lifecycle state that permits debug unlock", and the states and their transitions are not defined in the documents I read.

- Samsung key-provisioning documents (see 01_Repo_Analysis.docx, finding 6) are missing.

- 52 SRS requirements still carry open [X] parameters. See 03-Traceability/.

- DTLS role: the SRS says SOW2 is the DTLS client. A vehicle-ECU simulator acting as the DTLS server is needed on the bench (see 04-Test-Environment/).