# Cybersecurity test bench: what exists and what is missing

Sources: Test environment description/Description_of_test_environment_SOW-2.docx (v0.1.3), Analysis/Technica_Bench_Equipment_Capability_Analysis.docx, Analysis/Interface - HW Coverage Matrix.docx, and the framework in Gitlab Files/…/ (its CLAUDE.md). The "needed" column is my assessment for cybersecurity testing and to confirm with you.

## Already available

| Item | Use for cybersecurity |
| --- | --- |
| iTestStudio HIL framework with Python test pool, ttk_* libraries, report generation | Run cybersecurity scripts in the same format and reports as the functional tests |
| Python libraries for SOME/IP, service discovery, DHCP server, RTP sniffer, Technica switch control | SOME/IP robustness, DHCP hostile-server tests, traffic analysis |
| 20+ SOME/IP and camera-service test scripts | Reuse as the robustness baseline |
| Technica TE-1177 / TE-2004 (MACsec, MKA) Ethernet switch and capture modules, TE-1190 GMSL capture | Traffic injection, capture, MACsec tests (if used), GMSL observation |
| Wireshark 4.4.9 with Technica dissector | Protocol analysis of DTLS handshakes and SOME/IP |
| Lauterbach (via SoC) | JTAG state checks, register and I2C access, DRAM-range access attempts |
| Saleae Logic 2 | Capture of I2C, UART and SPI lines |
| R&S NGP824 power supply, Siglent DMM, Vötsch climate chamber | Power-cycle and reset tests, interrupted-update tests |

## Missing or to confirm

| Item | Needed for | Status |
| --- | --- | --- |
| Test root key and a way to provision it (development OTP or equivalent), so SOW1 authentication can be tested with valid and modified signatures | CS-SOW1-001 to 004 | Blocker. Only Samsung holds the real root private key |
| Controllable SOW1 responder (withhold, modify, delay, truncate responses), test frames with known AES-CMAC and frame counters, and a reference CMAC calculator | CS-SOW1-002 to 030 | Missing. A real imager cannot do this. The in-line I2C injector is listed as Missing in the coverage matrix |
| Scriptable DTLS 1.3 server and fault-injecting peer (PSK, ECDHE, TLS_AES_128_GCM_SHA256) | CS-DTLS (90 requirements) | Not in repo. Build or adopt |
| UART host tool and scripts for the debug unlock service, plus key material | CS-DBG | Not in repo. Unlock protocol details come from SRS 23.3 and the remaining requirement text |
| Defined ECU lifecycle states and a way to switch them on the bench | CS-DBG, CS-BOOT | Open question |
| Image-signing toolchain, keys, OTP/eFuse provisioning procedure | CS-BOOT, CS-UCM | Not in repo |
| SPI flash access through the needle adapter (programmer or clip) and a way to read back the flash | CS-BOOT, CS-NVM | Architecture lists the port. Equipment to confirm |
| SOW1 camera with fused security keys, or a simulator, plus Samsung key documents | CS-SOW1 | Documents missing (Security_Policy_REV1.00.pdf, CYBERSECURITY_KEY.FUSING_GUIDE.pptx) |
| In-line I2C injector (FPGA man-in-the-middle) | CS-GMSL | Listed as Missing in the Interface / HW Coverage Matrix |
| Packaging tool for software-update packages | CS-UCM | Not in repo |
| Fuzzing framework for SOME/IP payloads and DTLS records | CS-SOMEIP, CS-DTLS | Not in repo |
| Hardware fault-injection (voltage or clock glitching) | Optional extension of CS-BOOT | Not requested. Only if the SRS or your plan asks for it |

## Practical suggestion

Start with what needs no new hardware: CS-SOMEIP (reuse), CS-ETH, CS-NVM, and the log-checking parts of CS-DTLS using a simple Python DTLS peer. In parallel, close the three blockers with the longest lead time: the lifecycle definition, the signing and key material, and the Samsung key documents.