Interface & Feature — HW Coverage Matrix

Per action on each interface — HW needed and current availability on the MVCU01 bench

| Role (action on the interface) | HW needed | Status | TSR |
| --- | --- | --- | --- |
| SoC ↔ Deserializer — primary I²C @0x50 + ERRB/LOCK |  |  |  |
| Read I²C bus (SoC↔Des) | Logic analyzer (Saleae) | Available |  |
| Write I²C register values (config / read-back) | Lauterbach via SoC I²C (or external master with SoC halted) | Available |  |
| Inject I²C CRC / message-counter error | In-line I²C injector (FPGA MITM) | Missing |  |
| Read ERRB / LOCK (Des→SoC) | Logic analyzer | Available |  |
| Drive DES_EN / DES_XRES (enable / reset) | Lauterbach via SoC GPIO | Available |  |
| Control V of VDDIO / VTERM (deser rails) | NGP824 + PCB cut / rail isolation | Available |  |
| Deserializer ↔ Serializer — GMSL2 coax (tunnel) |  |  |  |
| Read / write Serializer registers (tunneled, EXT11 0x383) — @0x80 | Lauterbach via SoC I²C over the GMSL2 control channel | Available |  |
| Inject coax line-fault (short-batt / GND / open) | Solectrix PoC Fault Injection Unit (short-to-GND + short-to-VEXT; “open” TBC) | Planned (on order) |  |
| Inject coax BER / attenuation (PRBS) | TE-1190 Serdes Diagnostics PRBS mode covers BER pass/fail; RF step attenuator + bias-tee still needed to sweep link margin | Partial |  |
| Observe GMSL LOCK / link | Logic analyzer | Available |  |
| Force serializer / deserializer MFP pins (Low / High / High-Z / Bridge) | TE-1190 GPIO switching via the internal MAX10 — Status-tab configuration, so it needs Save & Apply plus a device restart. Preconditioning only until the API is in use. | Preconditioning |  |
| Capture GMSL control-channel and pass-through I²C | TE-1190 PLP I²C logging to SFP+ (separate Bus IDs per bus) | Available |  |
| Control PoC power on coax (off/on recovery) | Solectrix PoC Fault Injection Unit. The TE-1190 can set the steady-state PoC supply (HW v2.1+, max 600 mA) but not toggle it during a run — the setting needs Save & Apply plus a restart. | Planned (on order) |  |
| Serializer ↔ Imager — pass-through I²C (camera head) |  |  |  |
| Read / write Imager registers (pass-through) — @0x20 | Lauterbach via SoC I²C, pass-through enabled (REG1::IIC_1_EN / IIC_2_EN) | Available |  |
| Camera head connected (terminates the GMSL2 link) | SOW1 camera head — serializer 0x80, imager 0x20, PMIC 0xE0, EEPROM 0xA0 | Available |  |
| SoC ↔ Ethernet PHY — MDIO + INT_N / RST_N |  |  |  |
| Read MDIO bus (SoC↔PHY) | Logic analyzer | Available |  |
| Write PHY config via MDIO | MDIO via SoC (Lauterbach) | Available |  |
| Read INT_N / RST_N | Logic analyzer. TE-1406 MQS PIN 2 gives a hardware link-status output (0 V = link up, 3.3 V = no link) that can be wired to a Saleae channel as t0. | Available |  |
| Drive PHY RST_N | Lauterbach via SoC GPIO | Available |  |
| Inject PHY link fault (break / disturb T1) | TE-1406 console: phy mute / phy unmute (immediate). Hardware alternative: MQS PIN 3 shorted to PIN 1. TE-2004 port disable works only as an API on-the-fly command — from its web page it needs a restart. | Available |  |
| Inject PHY ref-clock fault (25 MHz) | Programmable clock source | Missing |  |
| Control V of PHY VREGA / VDDIO | NGP824 + PCB cut / rail isolation | Available |  |
| SoC ↔ DRAM (LPDDR4) |  |  |  |
| Read back REG_ECCH_MODE post-boot, confirm ECC engine enabled | SoC register readback via JTAG (Lauterbach) | Available |  |
| Inject double-bit ECC error into 32-byte codeword, read back | SoC DDR controller register write/read via JTAG (Lauterbach) | Available |  |
| Read DRAM bus signals directly during error injection | High-speed — no test point (BGA-to-BGA, interposer/probe not available) | N/A |  |
| Capture timing of trigger start and assertion (CEHU safety-error assertion on double-bit error) | GPIO marker toggled by SoC SW via JTAG & CEHU_SAFETY_ERR_L_0/1 (B10/P17), Analog Discovery digital channel | Available |  |
| Read back WATCHDOG_TIMER_0/1_CONFIG values post-boot | JTAG (Lauterbach) register readback | Available |  |
| Confirm safe-state transition ≤200ms after WDT error | JTAG (Lauterbach) timer starts; safe-state observable | Available (pending on safe-state observation if it’s external behavior or it’s an internal SW status flag?) |  |
| Read back DRAM segmentation register config post-boot & confirm non-overlapping segment allocation (≥256KB/client) | JTAG (Lauterbach) register readback | Available |  |
| Inject address-stuck error, read DTTE_ERR_VLD/DTTE_ASTUCK_ERR_VLD, confirm safe-state | JTAG (Lauterbach) injection + register read; safe-state observable TBC | Available (pending on safe-state observation if it’s external behavior or it’s an internal SW status flag?) |  |
| Read back DBSE_MODE / DBSE_BIST_PATTERN config, confirm sequencing before first client read | JTAG (Lauterbach) register readback + boot-sequence trace | Available |  |
| Fault initialization injection by skipping OR miss configuring DRAM BIST and DBSE. | Flashing a specific for debug build | Available |  |
| SoC-internal — CEHU / ECRU / IST / clock & power monitors |  |  |  |
| Read registers back (IST control register, PSMON, internal safety error registers,..) | JTAG (Lauterbach) register readback | Available |  |
| Fault injection to LBIST/ MBIST | Open So far, we have 2 workarounds: - Verifying the reaction of the failure by injecting the reaction to be in the expected mode it failure is detected as what’s agreed to be done in MVC-99665 - Corrupting the vector data pre start | N/A |  |
| Fault injection to CEHU & ECRU | CEHU safety_error_inj_0..4 (0x028–0x038, RW) via Lauterbach | Available |  |
| Post Boot configuration readback | JTAG (Lauterbach) | Available |  |
| CLKs enable bit readback | JTAG (Lauterbach) | Available |  |
| Inject out-of-bounds or stuck clocks error in runtime/ after boot | JTAG (Lauterbach) if it’s not locked protected after boot | Available if it’s not locked protected after boot |  |
| Control SoC CLKIN 24 MHz (not needed so far, but if XIN failure req has been added, it’ll be needed to control it to trigger different XIN cases) | signal/function generator | N/A |  |
| Injecting power monitor latent fault test (uvov_swap) | JTAG (Lauterbach), write then read | Available |  |
| Control V of SoC rails VDDI (power monitor) | Analog discovery | Available |  |
| PMIC I²C Communication & Register Access |  |  |  |
| Read PMIC I²C Registers | Lauterbach | Available |  |
| Write / config PMIC registers via I²C | Lauterbach | Available |  |
| I²C CRC error injection | In-line I²C injector (FPGA) | Missing |  |
| Verify write / read-back / re-write timing (<50ms) | Open (Lauterbach) | Available |  |
| PMIC Rail Voltage Control & Measurement |  |  |  |
| Measure buck output voltage (nominal) | Scope / DMM | Available |  |
| Control V of PMIC Vin (cut-off test) | NGP824 (bench is the source — no sink needed) | Available |  |
| Control V of PMIC buck1–6 (OV/UV perturbation) | Command buck VOUT setpoint via I²C (Lauterbach) — no sink required | Available (confirm runtime setpoint) |  |
| Measure switching-node voltage collapse to 0V | Scope / DMM (multi-channel) | Available |  |
| PMIC ALERT# / PG Safety Signaling |  |  |  |
| Force ALERT# / PG pin low (fault simulation) | I²C write ALERT#_PG_FORCE_ASSERT (C0h) b1/b0 via Lauterbach | Available |  |
| Measure PG / ALERT sub-µs timing (<50µs) | Saleae (500 MSPS) on the physical pin — NOT Lauterbach | Partial (no TP — probe access) |  |
| Measure PG→power-cutoff timing (<1ms) | Scope/DMM + timer | Available |  |
| PMIC Built-In Self-Test (ABIST / LBIST) |  |  |  |
| Inject ABIST fault (force OV beyond threshold) + read status register | NGP824 + I²C read — on-demand ABIST trigger needs PMIC Safety Manual §5.3 | Partial (doc needed) |  |
| PMIC FuSa Error Handling / SafeState Transition (SW) |  |  |  |
| Inject SoC-side CRC error + measure time to PHY inactivity | Open (SW fault-injection via JTAG (Lauterbach) + timing capture on PHY) | Available |  |
| Force ALERT#/PG low, observe SW SafeState transition + timing | C0h register write + SafeState observation (PHY inactive / PMIC_ISP_RST_N TP1207) | Available |  |

SafeState — How to Check

1.  Signals to probe

| Signal | Where to probe | Healthy | SafeState / fault | Meaning |
| --- | --- | --- | --- | --- |
| Ethernet image stream | RGMII TX_EN, or capture at TE-1406 / TE-2004 | Packets present | Packets stop | The definitive observable — this IS the safe state |
| CEHU_SAFETY_ERR_L_0 | TP1228 (SoC ball B10) | HIGH | LOW — sticky | SoC detected a safety error. Latched: does not self-clear |
| CEHU_SAFETY_ERR_L_1 | TP1229 (SoC ball P17) | HIGH | LOW — sticky | Redundant second copy — both pins must agree |
| PHY RST_N | TP709 | HIGH | LOW, held | Mechanism 1: SoC stops the PHY |
| PHY INT_N | TP710 | HIGH | LOW while pending | PHY-side error indication |
| PMIC_ISP_RST_N | TP1207 | HIGH | LOW — pulse or held | Mechanism 2: second PMIC resets the SoC. May be a short pulse — capture with the logic analyzer |

Missing for Full Test Coverage

Everything not yet available that is required to execute the full set of test cases — hardware, test actions not covered by the sections above, missing documents, and open confirmations.

| Role (action / item still needed) | HW / information needed | Status |
| --- | --- | --- |
| Hardware still to procure / build |  |  |
| Inject I²C CRC / message-counter error (deser + PMIC) | In-line I²C injector — FPGA MITM (e.g. Digilent Arty A7-100T). Only device able to corrupt a frame in transit. | Missing |
| Inject coax BER / attenuation (PRBS / link margin) | PRBS mode now available via the TE-1190 Serdes Diagnostics tab. Still needed to sweep link margin: RF step attenuator + bias-tee (Mini-Circuits RCDAT-6000-90 + ZFBT-6GW+) | Partial |
| Inject clock fault — PHY 25 MHz (and SoC 24 MHz if an XIN requirement is added) | Programmable clock source (Si5341-EVB / Keysight 33500B) | Missing |
| Coax “open circuit” line-fault (LF_x = 0b011) | Solectrix unit documents short-to-GND and short-to-VEXT only — confirm whether “open” is supported | Open (confirm) |
| Probe PMIC ALERT# / PG pins for the <50 µs timing | No test point on Sheet 5 (ALERT = U500 pin 16, net PG_PMIC) — needs probe access / tack-solder | Missing |
| Break / disturb the 100BASE-T1 link | Resolved manually: TE-1406 console phy mute / phy unmute asserts and releases TX_DISABLE immediately, and MQS PIN 2 provides a hardware link-status observable [MC UM v1.3 §4.2 pp.11–12, §4.3 p.17]. No break relay, no API. | Resolved |
| Test actions not covered by the sections above |  |  |
| Single-bit (correctable) ECC error test — corrected, counter increments, NO safe-state | JTAG (Lauterbach) injection + register read | To be added (DRAM) |
| Force a hang in a DRAM functional block (HW watchdog 3 ms detection) | Injection method undefined | Open — MVC-99256 |
| Characterize GPIO-marker latency used as t0 | Saleae cross-check against the injection instant | To be added |
| Confirm whether ECRU has its own injection register or is CEHU-only | Ambarella CV2FS HW Safety Manual (available) - review | Open |
| Missing documents |  |  |
| MAX96714 Safety Manual (32-100741 Rev D) - section 8 tables SF1-SF4, section 9.2 measures SM1-SM20 | Received and stored in the Deserializer folder. Fault reaction (ERRB or LOCK, dedicated register, end-to-end CRC), 15 ms element FTT, and the SM1-SM20 list with Default vs Optional status are now defined. | Available |
| PMIC Safety Manual §5.3 — on-demand ABIST trigger | Present in PMIC folder — review §5.3 for the trigger mechanism | Available — to review |
| Camera Interface Specification (0220DN00JX) | Available in Functional Safety-Development — I²C addresses §3, serializer MFP map §5 | Available |
| Open confirmations |  |  |
| Is the IST STATUS register writable? (decides the LBIST “force status” workaround) | Ambarella CV2FS HW Safety Manual (now available) — review | Open |
| Is clock-monitor configuration lock-protected after boot? | Ambarella CV2FS HW Safety Manual — review | Open |
| Does the PMIC allow runtime VOUT setpoint change? (gates the 6 buck OV/UV reqs) | PMIC datasheet / vendor | Open |
| Are ALERT# / PG routed to SoC GPIOs? (SW detection path, MVC-108192) | Schematic / SW confirmation | Open |
| Can the SoC I²C/PMBus driver emit a wrong PEC/CRC byte? (would remove the FPGA injector need for MVC-99243) | SW team confirmation | Open |
| Does firmware expose an I²C helper callable from Lauterbach? | SW team — determines whether an external I²C master is needed at all | Open |
| Which TE-1190 hardware version is on the bench? | PoC injection needs HW v2.1+; external / configurable source needs v3.0. Check the label on the device underside. | Open |
| Can the TE-1190 Serdes Diagnostics tab reach the DUT deserializer, and does it take 16-bit register addresses? | The Serdes Diagnostics tab is the one live control on the module (no restart). If it reaches the DUT deserializer at 0x50 with 16-bit addressing, it replaces an external I²C master for tunnel-side writes. Bench check. | Open |
| What is the TE-1190 forwarding latency, camera port to ECU port? | Not published. It sits inside any FTTI interval whose t0 is taken on the camera side. | Open |