Register Map & Interface Index

1.  Register maps — per component

| Component | What is at that location | Open |
| --- | --- | --- |
| SoC — Ambarella CV22FS | CEHU registers (safety_mode, safety_error_inj 0x028–0x038, error_bit_vector) | Register map — p.588 ↗ |
| Deserializer — MAX96714 (U600) | Register map — REG1 0x01, REG5 0x05, CTRL3 0x13, REG26 0x26, PWR0 0x08, CRC 0x3000–0x300F, TUN_EN 0x474 | Register map — p.53 ↗ |
| Serializer — MAX96717 (camera head) | Register map — EXT11 0x383 (Tun_Mode), I²C addr translation 0x42–0x45 | Register map — p.76 ↗ |
| Ethernet PHY — TJA1104 (U700) | §7 Registers (CL22) — ALWAYS_ACCESSIBLE 801Fh, DEVICE_STATUS 42h, OP_COND_IRQ 90h/91h, EPHY_FUNC_IRQ 80A0h/80A1h | Register map — p.83 ↗ |
| Ethernet PHY — TJA1121 (alt.) | Register tables (bit / symbol / access) | Register map — p.78 ↗ |
| Ethernet PHY — LAN8870/71/72 (alt.) | MMD register document | Register map — p.9 ↗ |
| PMIC — MPQ70165FS (U500) | REGISTER MAP — PG_CONFIG 0xCA, ALERT#_PG_FORCE_ASSERT 0xC0, ALERT_MAPPING 0xC4 | Register map — p.82 ↗ |
| Imager — Samsung S5K3B6SA | CCI (I²C) register access; I²C ID table 4-5 on p.36 | Register map — p.33 ↗ |
| DRAM — ISSI LPDDR4 | Mode registers — 3.4.1 MR0 Register Info | Register map — p.15 ↗ |
| Flash — ISSI IS25WP01G | 6. REGISTERS — status & configuration registers | Register map — p.21 ↗ |
| Imager — SFR / embedded register list v0.7 | Newer register list (supersedes v0.5) — sheets per HDR setting | Open file ↗ |

2.  Interfaces & I²C addresses

Camera-head addresses are taken from the Camera Interface Specification (0220DN00JX v1.0, §3, p.7). Board addresses are taken from the MVCU01 schematic and the component datasheets.

| Device | Location | Interface | 7-bit | 8-bit W / R | Source |
| --- | --- | --- | --- | --- | --- |
| Deserializer MAX96714 (U600) | MVCU01 board | I²C — primary control | 0x28 | 0x50 / 0x51 | CFG0 strap; datasheet REG0 p.68 · schematic Sheet 6 |
| Serializer MAX96717 | Camera head | I²C over GMSL2 control channel | 0x40 | 0x80 / 0x81 | Camera Interface Spec §3; CFG0_FSYNC strap = I²C, XTAL, 0x80 |
| Image Sensor S5K3B6SA | Camera head | I²C pass-through | 0x10 | 0x20 / 0x21 | Camera Interface Spec §3 (differs from datasheet default 0x2D) |
| Camera-head PMIC | Camera head | I²C pass-through | 0x70 | 0xE0 | Camera Interface Spec §3 |
| Camera-head EEPROM | Camera head | I²C pass-through | 0x50 | 0xA0 | Camera Interface Spec §3 |
| PMIC MPQ70165FS (U500) | MVCU01 board | I²C / PMBus | 0x03 | — | TSRs / schematic Sheet 5 |
| IMU | MVCU01 board | I²C — management | 0x4E | — | schematic Sheet 12 |
| Microphone | MVCU01 board | I²C — management | 0x35 | — | schematic Sheet 12 |
| Ethernet PHY TJA1104 (U700) | MVCU01 board | MDIO (Clause-22/45) — not I²C | — | — | schematic Sheet 7 · TP713 / TP714 |

3.  Interface & safety specifications

| Document | Contents | Open |
| --- | --- | --- |
| Camera Interface Specification | §3 I²C addresses — the authoritative camera-head address table | Open — p.7 ↗ |
| Camera Interface Specification — MFP map | §5 Serializer configuration — MFP0 IMG_RST_N, MFP1 CFG0_FSYNC, MFP2 CFG1, MFP3 SER_LH_EN, MFP4 SER_CLKOUT (25 MHz), MFP5 PMIC_ERROR_N, MFP6 VARIANT_CODE | Open — p.9 ↗ |
| Ambarella CV2FS / CV22FS HW Safety Manual | Safety requirements, safety mechanisms, integration & analysis (rev. 1522577) | Open file ↗ |
| Ambarella CV2FS CV22FS Safety Case | HW-SM-005 v1.3 — supplier safety case | Open file ↗ |
| MAX96714 / MAX96714F Safety Manual (GM16) | Doc 32-100741 Rev D. Section 8 Functional Safety Concept (pp.14-16): SF1-SF4 safe-state tables, element Fault Tolerant Time 15 ms. Section 9.2 (pp.24-25): Safety Measures SM1-SM20 with diagnostic intervals, Default/Optional status per mode, and ERRB routing. | Open - p.14 |
| PMIC MPQ70165FS Safety Manual | §5.3 internal safety mechanisms (referenced by the PMIC TSRs) | Open file ↗ |
| MVCU01 schematic (11-E-01405, BOM02I) | Sheet 6 GMSL2/deser · Sheet 7 PHY · Sheet 12 SoC · Sheet 5 PMIC · Sheet 4 power | Open — p.6 ↗ |