TSR Interface & Read/Write Analysis

Which components must read/write to each other — and the test-bench hardware each interface needs

| Field | Value |
| --- | --- |
| Source | Technical Safety Concept — 11-P-01467 (System Allocation Requirements), 180 pp |
| Parsed | 225 requirements · 158 testable (checked by Test) · 58 review (not testable) · 9 unclassified |
| Scope | Only 'Testable, checked by Test' TSRs (the ones that become executable TCs) are analysed here |
| Method | Each TSR grouped by its allocation section (= target component); its Verification Criteria scanned for inject / read / physical-stimulus / timing |
| Prepared for | alaa.said@brightskiesinc.com — FuSa Validation |

1.  Key finding — the access topology is a star centred on the SoC

Across all 158 testable TSRs, the actor is the SoC (SoC SW / SoC HW). There are no peer-to-peer safety interactions (e.g. Deserializer↔PHY). Every read/write relationship is SoC↔component. Therefore the test bench interfaces with the SoC and taps each SoC↔component bus — it never needs to sit between two peripherals.

“Inject” = the test must WRITE a fault value on that interface (needs an active device / debug write / physical stimulus). “Physical” = the fault is a real electrical stimulus (clock, supply, link, hang), not a register write. “Timed” = has a ms reaction budget to measure.

| Relationship (SoC ↔ target) | # testable TCs | Need WRITE / inject | Need READ / sniff | Physical stimulus | Timed (ms budget) |
| --- | --- | --- | --- | --- | --- |
| SoC-internal (CEHU/ECRU/IST/CKMON/PSMON) | 52 | 21 | 52 | 12 | 17 |
| PMIC | 38 | 17 | 38 | 18 | 15 |
| Deserializer (+ Serializer, Imager) | 29 | 7 | 29 | 4 | 18 |
| Ethernet PHY | 21 | 13 | 21 | 5 | 9 |
| DRAM | 14 | 8 | 14 | 3 | 4 |
| Flash | 4 | 0 | 4 | 0 | 0 |

Read/sniff is required by every TC (all establish a baseline and read the reaction). Write/inject and physical-stimulus counts drive the specialised bench hardware.

2.  Read/Write relationship → interface → test-bench hardware

| SoC ↔ target | Interface (test point) | What the test must DO | Test-bench hardware needed |
| --- | --- | --- | --- |
| SoC-internal (CEHU, ECRU, IST/BIST, clock & power monitors) | SoC internal registers (no external bus) + clock & supply rails | Read CEHU/ECRU/IST status; INJECT corrupted values into safety-error / BIST-status registers; for clock/power monitors apply real clock & supply faults | JTAG/debug probe (Lauterbach TRACE32 / SEGGER J-Link) or SoC SW test hook to read+write internal regs; programmable clock source; programmable rail supplies |
| SoC ↔ PMIC | PMIC I²C/PMBus (TP507/TP508) + WDI (TP510/514) + power rails | Read PMIC status; INJECT via I²C; manipulate Vin & buck rails (UV/OV); drive/starve the watchdog (WDI); observe power-good / rail cut-off & timing | Active I²C/PMBus device on PMIC bus; programmable supplies (Vin + bucks); GPIO/DAQ for WDI; scope/LA for rail cut-off timing |
| SoC ↔ Deserializer (and via it → Serializer, Imager) | Primary I²C @0x50 (TP1211/TP1212); tunnel/pass-through to Ser (EXT11 0x383) & Imager; ERRB/LOCK (TP1232/TP1215) | Read/write deser, serializer (tunnel) & imager (pass-through) registers; INJECT I²C bit/CRC errors; coax & supply faults; observe ERRB/LOCK | Active in-line I²C device (inject + CRC/msg-counter); coax fault switch + attenuator; programmable VDDIO/VTERM; ERRB/LOCK probes |
| SoC ↔ Ethernet PHY | MDIO (TP713/TP714) + INT_N (TP710) + RST_N (TP709) | Read/write PHY config regs via MDIO (config regs ARE writable); status/IRQ regs are RO/W1C → faults must be PHYSICAL (link, clock, supply); observe INT_N & RST_N | MDIO master; 100BASE-T1 link partner + media-fault harness; 25 MHz clock stop; PHY supply control; INT_N/RST_N probes |
| SoC ↔ DRAM | SoC DDR controller / DRAM (no external test point) | Read ECC/error status; INJECT ECC errors via SoC controller; force functional-block hang for HW watchdog; measure detection | SoC DDR ECC error-injection (via debug/SW); no external bus — driven through the SoC; timing capture |
| SoC ↔ Flash | QSPI (TP1002–TP1008) | Read back image & compare CRC-32 (verification only — NO fault injection required on the flash bus) | QSPI sniffer (optional) + flash tool; no active injector needed |

3.  Where you need an ACTIVE (write/inject) device vs. read-only

Directly answering the bench question: an active injection device is mandatory only on the interfaces below; the rest are read/observe or physical-stimulus.

Active in-line I²C device — on the PRIMARY I²C (Deserializer bus, TP1211/12): 7 inject TCs, plus CRC / message-counter error tests, plus reaching Serializer (tunnel) and Imager (pass-through).

Active I²C / PMBus device — on the PMIC bus (TP507/08): 17 inject TCs.

JTAG / debug write (or SoC SW hook) — for SoC-internal register injection (CEHU/ECRU/IST): 21 inject TCs. This is the 'active device' for everything inside the SoC — not a bus sniffer.

MDIO master (write) — for PHY CONFIG registers (writable). But PHY fault STATUS cannot be injected by register write (RO/W1C) — those 13 'inject' TCs are PHYSICAL (link/clock/supply).

No active bus device needed for Flash (read-only CRC check) or DRAM external bus (injection is internal to the SoC DDR controller).

Physical-stimulus rigs are additionally required: programmable supplies (PMIC 19, SoC-internal 10, PHY 8, deser 6), a clock-fault source (SoC clock monitors, PHY 25 MHz), a 100BASE-T1 link-fault harness (PHY), and a coax fault/attenuator (deserializer link).

4.  Shared reaction observables & timing (needed by the timed TCs)

SafeState (system): FAULT_CAM (TP403) / FAULT_SHUT (TP404) — the end reaction for most 'go to safe state within N ms' TSRs.

SoC safety error pins / CEHU error propagation — for the SoC-internal safety-mechanism TSRs.

Deserializer ERRB (TP1232) / LOCK (TP1215); PHY INT_N (TP710) / RST_N (TP709).

All timed TCs (17 SoC-internal, 15 PMIC, 18 deser, 9 PHY, 4 DRAM) need synchronized capture: shared hardware trigger from the injector + reaction on one time base (per the Test-Environment spec).

5.  Traceability — testable TSR IDs per relationship

SoC-internal (CEHU/ECRU/IST/CKMON/PSMON)  (52 TCs):  MVC-99664, MVC-99667, MVC-99668, MVC-99669, MVC-99671, MVC-99670, MVC-99673, MVC-99675, MVC-99674, MVC-99679, MVC-99678, MVC-99681, MVC-99680, MVC-99683, MVC-99684, MVC-99783, MVC-99784, MVC-99785, MVC-99786, MVC-99788, MVC-99789, MVC-99790, MVC-99791, MVC-99806, MVC-99805, MVC-99807, MVC-99809, MVC-99808, MVC-99810, MVC-99811, MVC-99812, MVC-99875, MVC-99874, MVC-99873, MVC-99872, MVC-100070, MVC-100073, MVC-100076, MVC-100075, MVC-100066, MVC-100068, MVC-100067, MVC-100755, MVC-100092, MVC-99800, MVC-100091, MVC-99795, MVC-99797, MVC-99799, MVC-100107, MVC-100098, MVC-100097

PMIC  (38 TCs):  MVC-105396, MVC-99236, MVC-99238, MVC-105400, MVC-105401, MVC-99239, MVC-105402, MVC-99241, MVC-99243, MVC-107742, MVC-99237, MVC-99250, MVC-99249, MVC-99252, MVC-99251, MVC-99246, MVC-99245, MVC-99248, MVC-99315, MVC-99316, MVC-99320, MVC-99322, MVC-99321, MVC-99324, MVC-99323, MVC-99326, MVC-99325, MVC-99317, MVC-99319, MVC-108191, MVC-108193, MVC-108192, MVC-100138, MVC-100140, MVC-100139, MVC-100142, MVC-100144, MVC-100146

Deserializer (+ Serializer, Imager)  (29 TCs):  MVC-100103, MVC-105968, MVC-105969, MVC-100121, MVC-100223, MVC-100222, MVC-106886, MVC-106887, MVC-100113, MVC-100112, MVC-100115, MVC-100114, MVC-100120, MVC-100220, MVC-100117, MVC-100116, MVC-100118, MVC-100127, MVC-100128, MVC-100214, MVC-100213, MVC-100218, MVC-100219, MVC-106124, MVC-106125, MVC-106126, MVC-106127, MVC-106128, MVC-106130

Ethernet PHY  (21 TCs):  MVC-108301, MVC-99583, MVC-99298, MVC-99575, MVC-99577, MVC-99576, MVC-99579, MVC-99578, MVC-99580, MVC-99582, MVC-99581, MVC-99584, MVC-99600, MVC-99604, MVC-99603, MVC-99606, MVC-99605, MVC-99627, MVC-99626, MVC-99637, MVC-99636

DRAM  (14 TCs):  MVC-99256, MVC-99143, MVC-99144, MVC-99257, MVC-99260, MVC-99258, MVC-99259, MVC-99151, MVC-99152, MVC-99153, MVC-99158, MVC-99159, MVC-99205, MVC-99206

Flash  (4 TCs):  MVC-99213, MVC-99214, MVC-99216, MVC-99218

Note: section→component grouping follows the TSR document's own allocation chapters. A few TSRs in the Deserializer chapter actually target the Serializer (tunnel, e.g. EXT11 0x383) or Imager (pass-through) — reached over the same SoC↔Deserializer I²C path, so they share its bench hardware.

6.  Hardware Resources Needed  (test-bench bill of materials)

Consolidated from the read/write + physical-stimulus analysis above. Part numbers are REPRESENTATIVE examples to size and budget the bench — substitute your lab's equivalent. ‘Active’ = writes/injects on a bus; ‘Capture’ = observe + measure timing; ‘Stimulus’ = physical fault.

| Resource (role) | Description — what it enables | Sample part number (representative) |
| --- | --- | --- |
| Logic analyzer / MSO (Capture) | Synchronized capture of the injection trigger + all reactions (ERRB, LOCK, INT_N, RST_N, PHY-inactive, PMIC_ISP_RST_N) with I²C/MDIO decode; measures the 10/50/200/300/350 ms budgets on one time base. | Saleae Logic Pro 16 |
| Oscilloscope ≥200 MHz (Capture) | Analog cross-check of fast edges, rail cut-off, ALERT#/PG transitions and glitches. | Tektronix MSO44 (or Keysight DSOX3024T) |
| Active I²C host adapter (Active) | Read/write registers on the primary I²C (deser @0x50, +tunnel Ser, +pass-through Imager) and the PMIC I²C @0x03; baseline config & read-back. | Total Phase Aardvark I2C/SPI (P/N TP240141) |
| In-line I²C fault injector (Active) | Man-in-the-middle that corrupts a data/CRC byte or drops/duplicates a message for the CRC & message-counter TCs (deser 7 inj, PMIC 17 inj). | FPGA: Digilent Arty A7-100T (or Total Phase Promira Serial Platform) |
| MDIO (Clause-45) master (Active) | Read/write PHY MMD registers (config-enable, config regs). PHY status/IRQ regs are RO/W1C → not injectable here. | Total Phase Promira + MDIO app (or FPGA-based MDIO) |
| JTAG / debug probe (Active) | Read/write SoC-internal safety registers (CEHU, ECRU, IST/BIST) and INJECT corrupted values; halt/resume for the 21 SoC-internal inject TCs. | Lauterbach µTrace for Cortex-R (or SEGGER J-Link PRO) |
| Programmable supply / SMU, multi-rail (Stimulus) | Command UV/OV sag & overshoot on VDDIO/VTERM/VREGA/Vin/PMIC bucks; measure rail cut-off timing (PMIC 19 phys, SoC 10, PHY 8, deser 6). | Rohde & Schwarz NGU401 (or Keysight N6705C + SMU) |
| Bench multi-output supply (Support) | Interposer / auxiliary rail power. | Keysight E36313A |
| Programmable RF step attenuator (Stimulus) | Raise link BER for the GMSL serial-link PRBS / BIST TCs. | Mini-Circuits RCDAT-6000-90 (USB, DC–6 GHz) |
| Bias-tee (Support) | Attenuate/inject on the coax RF while preserving Power-over-Coax DC. | Mini-Circuits ZFBT-6GW+ |
| Coax fault-switch / relay matrix (Stimulus) | Short-to-battery / short-to-GND / line-open on the GMSL coax; sensed by deser LMN0/LMN1. | Pickering PXI 40-190 (or Coto 9007-05-00 reed relays) |
| USB relay board (Support) | Software-drive the coax fault relays and other switches. | Numato 8-Channel USB Relay Module |
| USB-GPIO / DAQ (Support) | Drive DES_EN / XRES / PHY RST_N / PMIC WDI; read discrete lines; emit the shared TRIG. | NI USB-6501 (or Numato 16-ch USB GPIO) |
| Programmable clock source (Stimulus) | Stop / perturb the 24 MHz SoC CLKIN and PHY 25 MHz for clock-monitor & REFCLK TCs. | Silicon Labs Si5341-EVB (or Keysight 33500B) |
| Thermal forcing system (Stimulus) | Drive junction temperature (–40…+125 °C) for temperature-sensing / TEMP_WARN TSRs. | inTEST ThermoStream ECO-560 (or Thermonics T-2500SE) |
| 100BASE-T1 link partner + media-fault (Stimulus) | Establish then break/disturb the PHY link (link/training/polarity faults) and observe Ethernet-inactive SafeState. | Technica Engineering 100BASE-T1 Media Converter + break-out |
| GMSL2 remote end / camera head (Support) | Terminate the coax for tunnel, pass-through and imager-readback TCs. | SOW1 camera head (or ADI MAX96717 + MAX96714 eval kits) |
| Signal-access interposer / break-out (Support) | Cut-jumpers on primary I²C, rails and coax so injectors insert and rails isolate to one device. | Custom (in-house PCB) |

Roles: Active = mandatory write/inject device; Capture = observe + timing; Stimulus = physical fault; Support = infrastructure. Minimum set to start the majority of fail-path TCs: logic analyzer, active I²C host, in-line I²C injector, JTAG probe, one multi-rail programmable supply, and the coax fault-switch — see §3.