FuSa Validation — Test Environment Specification

Test-bench instrumentation, fault-injection capability, and timing-measurement method

| Field | Value |
| --- | --- |
| Board / DUT | MVCU01 — Option I (BOM02I), “SOW2 Rear and Trailer Camera 02 [100Base-T1 STD]”, drawing 11-E-01405 |
| Components in scope | Deserializer MAX96714F (U600, Sheet 6); Ethernet PHY TJA1104A (U700, Sheet 7); remote Serializer MAX96717 + Imager S5K3B6SA (camera head) |
| Purpose | Define the equipment, fault-injection means, probe points and timing-measurement method needed to execute the FuSa test cases |
| Companion docs | MAX96714_FuSa_TestAccess_and_Register_Reference.docx; component datasheets; schematic Sheet 6 / 7 / 4 |
| Prepared for | alaa.said@brightskiesinc.com — FuSa Validation |
| Status | Working spec — resolve Open Dependencies (§11) before bench freeze |

Guiding principle:  Every timed requirement is verified by capturing the fault-injection instant (t0) and the DUT reaction on ONE synchronized time base and measuring Δt. Because most PHY/deserializer status bits are read-only or write-1-to-clear, faults must be injected PHYSICALLY, not by register writes — the bench is built around that fact.

1.  Purpose & Scope

This document specifies the test environment required to execute the functional-safety test cases for the MVCU01 board. It answers three questions for the bench builder: (a) what instruments/devices are needed, (b) how each fault is injected and where, and (c) how the millisecond reaction budgets (10 / 50 / 200 / 300 / 350 ms) are measured with traceable accuracy.

2.  Why Physical Fault Injection (not register writes)

Analysis of the deserializer and PHY register maps shows the diagnostic STATUS/SOURCE bits cannot be used to create a fault:

| Register (example) | Addr | Access as documented | Consequence for testing |
| --- | --- | --- | --- |
| TJA1104 EPHY_FUNCTIONAL_IRQ_SOURCES | 80A0h | “R/W” but Write-1-to-Clear, latched-high | Writing clears the event; HW sets it. Cannot inject by write. |
| TJA1104 OP_CONDITIONS_IRQ_SOURCES | 90h | “R/W” but Write-1-to-Clear, latched | Same — clear-only. |
| TJA1104 ALWAYS_ACCESSIBLE::REFCLK_WARN | 801Fh b5 | R (read-only) | Cannot write at all; reflects OSC_WARN. |
| TJA1104 DEVICE_STATUS::FUSA_STATUS | 42h b15 | R (read-only) | BIST result; cannot be forced by write. |
| MAX96714 CTRL3 (LOCKED/ERROR/CMU) | 0x13 | Read Only | Status only; faults come from the link/supply. |

Therefore the enable registers (writable) are used to arm the mechanism, and the fault itself is applied physically on the link, clock, or supply. The bench must provide those physical stimuli.

3.  Test-Bench Architecture

Core building blocks: (1) the DUT board on a signal-access interposer, (2) fault injectors on each stimulus domain, (3) a synchronized capture instrument sharing one hardware trigger with the injectors, (4) a host PC automating instruments + register access, and (5) the remote camera head (or emulator) for the GMSL2 path.

Single shared hardware trigger (“TRIG”) line: each injector emits a trigger edge at the injection instant = t0. This edge is captured on the same instrument as the reactions, removing software-timestamp jitter.

All reaction signals (ERRB, LOCK, INT_N, RST_N, SafeState) and the control buses (primary I²C, MDIO) are wired to the capture instrument simultaneously.

Rails and buses are broken out through cut-jumpers on the interposer so each injector inserts cleanly and each supply can be isolated to a single device.

4.  Required Instruments & Devices

| Device (example product) | Purpose | Connects to (board node) | Automation-ready? (API/SCPI) |
| --- | --- | --- | --- |
| Mixed-signal capture — logic analyzer / MSO (Saleae Logic Pro 16; or Tektronix MSO) | Synchronized timing capture of trigger + reactions + bus decode. Primary timing instrument. NOTE: input-only — cannot drive lines. | TRIG line; ERRB (U600 p32), LOCK (U600 p7), INT_N & RST_N (U700), SafeState; I²C p4/p6; MDIO | Yes — Saleae Automation API (Python) |
| In-line I²C fault injector — FPGA man-in-the-middle (Digilent Arty-A7) or bus-switch + pattern gen | Corrupt a data/CRC byte or drop/duplicate a message on I²C; emits TRIG at injection | Primary I²C in-line: U600 p4 (SDA), p6 (SCL); also PMIC I²C | Yes — HDL / SDK scriptable |
| Coax fault-switch matrix — SSR/reed relays (Coto/TE) on USB relay board (Numato) or PXI (Pickering) | Short-to-batt / GND / line-open on the GMSL2 coax; emits TRIG | Coax at connector → U600 SIO p12/13; sensed by LMN0/1 (p18/19). Probe TP600 | Yes — Numato serial API / Pickering driver |
| Programmable RF step attenuator + bias-tee (Mini-Circuits RCDAT-6000-90) + noise source | Raise BER on the serial link for PRBS/BER; bias-tee preserves PoC DC | In series with coax (TP600) | Yes — Mini-Circuits USB DLL/API |
| Programmable supplies / SMU (R&S NGU401; Keysight N6705C / E36313A) — transient + sink capable | Command UV/OV on individual rails; measure rail cut-off timing | U600 VDDIO(p17)/VTERM(p20)/VDD18(p11)/VDD(p15); U700 VREGA/VDDIO; PMIC Vin+bucks | Yes — SCPI (USB/LAN) |
| Thermal forcing system (inTEST ThermoStream / Thermonics) | Drive junction temperature (−40…125 °C) for temperature-sensing TSRs | Localized over U700 / U600 | Yes — GPIB/serial API |
| Active I²C host (Total Phase Aardvark P/N TP240141 / Promira) | Register R/W: arm mechanisms, enable IRQs/CRC, read POST/FUSA_STATUS, config PMIC/deser/imager | Primary I²C p4/p6 (deser @0x50); PMIC I²C @0x03 | Yes — Python/C API (Control Center GUI) |
| MDIO master (via SoC, or a Clause-45 MDIO tool) | Read/write PHY MMD registers (enables, sources, DEVICE_STATUS) | U700 MDC/MDIO (TP713/714) | Yes — API, or via SoC/Lauterbach |
| Driven digital I/O — USB-GPIO/DAQ (NI USB-6501) or multi-tool (Digilent Analog Discovery 3) | Drive DES_EN/XRES/PHY RST_N/PMIC WDI; emit the shared TRIG (t0). AD3 also = slow scope + basic I²C master. Needs 1.8 V level shifting. | U600 PWDNB(p31)/XRES(p10); U700 RST_N; PMIC WDI (TP510) | Yes — NI-DAQmx / WaveForms SDK |
| 100BASE-T1 link partner + media-fault (Technica media converter + break harness) | Establish then break/disturb the PHY link; observe Ethernet-inactive SafeState | U700 MDI pair (Sheet 7) | Partial — link manual; fault via relay API |
| GMSL2 remote camera head (real ser+imager) or link emulator | Terminate the coax so tunnel / pass-through / imager-readback TCs run end-to-end | Coax link | N/A — DUT remote end |
| Bench oscilloscope (Tektronix / Keysight, ≥200 MHz) | Fast analog edges & sub-µs timing (PG < 50 µs); cross-check the LA | Shared probe points | Yes — SCPI |

5.  Fault-Injection Capability by Domain

| Domain / requirement class | What to inject | Injector | DUT detection → observable |
| --- | --- | --- | --- |
| Primary-I²C CRC / message-counter (deser) | Bit/CRC corruption; message drop/duplicate | In-line I²C injector on p4/p6 | CRC_ERR_CNT(0x300D)/MSGCNTR regs → ERRB (p32); SW retry on bus |
| Coax line-fault (deser #3, part of #6) | Short-to-batt, short-to-GND, line-open | Coax fault-switch matrix | REG26::LF_0/LF_1 → ERRB (p32) |
| Deser supply (VDDIO/VTERM) | UV below threshold (VDDIO<1.617 V, VTERM<1 V) | Programmable supply on the rail | PWR0(0x08)/REG7(0x07) → ERRB |
| Serial-link BER (PRBS diag) | Attenuation / noise until BER trips | Attenuator + bias-tee on coax | Link PRBS/BER result register → ERRB / LOCK |
| PHY link failure (#1,#2) | Break/disturb T1 MDI; force training failure | T1 link partner + media-fault harness | EPHY_FUNCTIONAL_IRQ_SOURCES(80A0h) → INT_N → RST_N low |
| PHY reference-clock (#4) | Stop / perturb the 25 MHz clock | GPIO/relay on clock net; or gate the oscillator | OSC_WARN(90h) → REFCLK_WARN(801Fh) → INT_N → RST_N |
| PHY operating-condition (#3) | Supply-warn (sag VREGA/VDDIO/core); over-temp | Programmable supply / thermal forcing | OP_CONDITIONS_IRQ_SOURCES(90h) → INT_N → RST_N |
| BIST fail (deser LBIST, PHY #6) | Not physically inducible | — | Cover by CODE REVIEW + vendor FMEDA / Safety-Manual evidence |

6.  How to Measure the Timing Budgets (10 / 50 / 200 / 300 / 350 ms)

6.1  Method

Define, per requirement, the START reference t0 (the fault instant) and the STOP reference t_react (the reaction edge). Reaction edges are hardware-observable: RST_N falling, ERRB asserting, LOCK de-asserting, or the SafeState line changing.

Wire the injector’s TRIG output and every reaction signal to the SAME capture instrument. Δt = t_react − t0 is then read directly from one waveform, with no cross-instrument or software-timestamp error.

Arm the mechanism first via the writable ENABLE registers (e.g., 80A1h, 91h, CC_CRC_EN), run a clean baseline to confirm NO reaction, then inject and capture.

Repeat each case N ≥ 20 times and report the worst-case Δt against the budget; note measurement uncertainty (sample period + trigger propagation).

6.2  Instrument settings

Sample rate: even 1 MSa/s gives 1 µs resolution — ~300,000× margin on a 300 ms budget. Choose rate × memory-depth so the full window (up to ~400 ms) is captured in one acquisition.

Trigger: start acquisition on the injector TRIG edge (t0). Use a long post-trigger capture to see the reaction anywhere within the budget window.

Cross-check the fastest reaction edges (e.g., the 10 ms ERRB) with an oscilloscope on the same node.

6.3  Detection vs. reaction split

Several requirements decompose the budget (e.g., “detection < 100 ms” inside a 200 ms total, or “test + signaling + detection + reaction” in 350 ms). To measure the DETECTION point separately from the REACTION, you need an observable at the moment SW recognizes the fault — either (a) a dedicated SW debug GPIO toggled by the ISR/monitor, or (b) the I²C/MDIO transaction where SW reads the status register (visible on the bus decode). If neither exists, only the end-to-end t0→reaction can be measured; the internal split must be shown by code review. Raise this with SW (see §11).

6.4  Reference points per requirement class

| Requirement class | Budget | t0 (start) | t_react (stop) — observable | Board node |
| --- | --- | --- | --- | --- |
| Deser detects FuSa problem → ERRB/LOCK | ≤ 10 ms | Physical fault instant (TRIG) | ERRB asserts / LOCK de-asserts | U600 p32 / p7 |
| I²C CRC error → re-read | ≤ 50 ms | I²C corruption instant (TRIG) | Repeated read on I²C decode | U600 p4/p6 |
| CRC re-fail → SafeState | ≤ 200 ms | 2nd corruption instant | PHY inactive (Eth TX stops) OR PMIC_ISP_RST_N (TP1207) asserted | see §10 |
| Imager readback mismatch → SafeState | ≤ 200 ms (detect <100 ms) | Forced mismatch instant | PHY inactive; detection via SW GPIO / bus | §10 + SW hook |
| PHY fault → hold RST_N low | ≤ 300 ms (FHTI) | Physical PHY fault (TRIG) | RST_N falling edge | U700 RST_N |
| Deser start-up BIST fail → SafeState | ≤ 350 ms | Reset release (start-up) | PHY inactive OR PMIC_ISP_RST_N (TP1207) | see §10 |

6.5  Defining t0 per fault type — the capture instrument is INPUT-ONLY

The logic analyzer (Saleae) captures but cannot drive: t0 must itself be a captured edge, never ‘when you clicked inject’. For faults with no natural electrical edge (register / software injections), toggle a shared TRIG marker at the injection instant (from the driven-I/O device) and capture it alongside the reaction.

| Fault type | t0 (start) = captured edge | t_react (stop) = captured edge |
| --- | --- | --- |
| Supply UV/OV | Rail crossing the threshold (analog channel on the rail) — NOT the PSU command | PG / ALERT# low, or PHY-inactive / reset |
| Clock fault | Clock stopping / crossing the threshold | CEHU error / safe-state signal |
| Coax line-fault | Relay-drive edge (fault applied) | ERRB assert / LOCK de-assert |
| I²C error injection | Corrupt transaction on the bus (LA decodes → timestamp) | Retry read / ERRB |
| Register-write injection (Lauterbach) | Driven GPIO marker at the write, OR Lauterbach trace timestamp | Reaction signal |

7.  Observability / Probe-Point Map

| Signal | Component / pin | Net | Test point | Use |
| --- | --- | --- | --- | --- |
| ERRB | U600 pin 32 | DES_ISP_MFP8 | — | Deser fault reaction (10 ms) |
| LOCK | U600 pin 7 | DES_ISP_MFP5 | — | Link-lock status |
| Primary I²C SDA/SCL | U600 pin 4 / 6 | ISP_DES_I2C.DAT/.CLK | — | Bus decode + injection |
| Coax link | U600 pin 12/13 | DES.SIO_P/_N | TP600 | Line-fault / BER injection point |
| VDDIO / VTERM | U600 pin 17 / 20 | V_DES_VDDIO / V_DES_VTERM | TP605 / TP606 | Supply UV injection & measure |
| Reset / Enable | U600 pin 10 / 31 | DES_XRES / DES_EN | — | Force start-up / power-down |
| PHY INT_N | U700 | PHY_INT_N | — | PHY interrupt reaction |
| PHY RST_N | U700 | PHY.RST_N | — | PHY reset reaction (300 ms budget) |
| PHY ref-clock | U700 | PHY_25MHZ / Y700 | — | Reference-clock fault |
| PHY supply | U700 | V_PHY_VREGA | — | Operating-condition fault |
| PHY MDIO | U700 | RGMII.MDC / MDIO | — | PHY register access |
| SafeState — primary | PHY / Ethernet | PHY link disabled (no Eth TX) | TP709 / TP710 | Definitional SafeState: image transmission stopped (MVC-100138/142) |
| SafeState — PMIC path | PMIC MPQ70165 | ALERT# (U500 p16), PG_PMIC | — (see §10) | PMIC signals FuSa error / triggers reset |
| SafeState — reset path | PMIC → SoC | PMIC_ISP_RST_N | TP1207 | PMIC resets system (variant B) |

8.  DUT Control, Bus Arbitration & Signal Access

Primary I²C has a single master (the SoC). For external register access or in-line injection, the SoC must be taken off the bus (held in reset / a bring-up mode) — confirm the mechanism with the SW/integration team.

Provide cut-jumpers / 0-Ω links on the interposer for: primary I²C (p4/p6), deser rails (p11/p15/p17/p20), the coax, and the PHY rails — so injectors insert and rails isolate cleanly.

Reset/enable control (DES_EN, DES_XRES, PHY RST_N) on GPIO so each start-up/self-test iteration is scripted and repeatable.

9.  Automation Roadmap  (manual now → automated test-bench)

Principle: buy automation-ready instruments (SCPI / USB / API) from the start — use them manually first, wrap them in scripts later, with NO rebuy. Every device in §4 is scriptable (see its Automation-ready column).

| Phase | What you run | Manual vs automated | Add this phase |
| --- | --- | --- | --- |
| 0 — Manual (now) | Saleae capture, Lauterbach reg R/W + inject, PSU set by hand, Technica link | Human triggers fault, reads Δt off the capture. OK for big budgets (200–350 ms); risky for 10 ms / 50 µs (needs an instrument-emitted t0) | — (use current kit) |
| 1 — Scripted instruments | Same devices driven by a Python host + shared trigger | Inject + capture + Δt + verdict scripted per TC; still supervised. Precise t0 for register / GPIO injections | Analog Discovery / USB-GPIO (driven I/O + trigger) + active I²C host (Aardvark) + interposer |
| 2 — Automated injectors | Add the fault injectors, all USB/SCPI/GPIO-controlled | Per-domain sequences run unattended: set-up → inject → capture → read-back → log | In-line I²C injector (FPGA), coax fault-switch + relays, programmable clock source, RF attenuator |
| 3 — Full automated bench | Test executive over a routing / switch matrix | Whole regression suite runs hands-off; CI-style evidence reports | Switch/relay matrix, productized interposer/fixture, test-executive (pytest / NI TestStand) |

9.1  Automation-readiness notes

Foundational — add in Phase 1: the shared TRIG marker and the signal-access interposer. Automation is impossible without a repeatable t0 and clean signal access.

Design the 1.8 V ↔ 3.3 V level shifting once (Phase 1) so every later driver / injector plugs in without rework.

Scriptable interfaces already available: Saleae Automation API, Lauterbach PRACTICE + remote API, PSU SCPI, Analog Discovery WaveForms SDK, Aardvark Python API, Mini-Circuits attenuator DLL, Numato relay serial API.

10.  How to Confirm SafeState  (per Technical Safety Concept, MVC-100106)

Definition: the system is in SafeState when it STOPS transmitting image data over Ethernet — the PHY link is disabled (MVC-100138). This is achieved by AT LEAST ONE of two mechanisms (MVC-100142), so a SafeState test must confirm the PHY went inactive by one of them:

Correction to earlier drafts: FAULT_CAM / FAULT_SHUT (Sheet 4) are camera/shutter power-branch fault flags — they are NOT the system SafeState indicator. The authoritative SafeState observable is PHY-inactive plus the ALERT#/PG/PMIC-reset signals below.

10.1  The two SafeState realizations

| Variant | Mechanism | What you observe on the bench |
| --- | --- | --- |
| A — PHY link disable | SoC / SafetyCore (Cortex-R52) commands the PHY to disable its link | PHY inactive (no Eth TX), SoC still running. Probe PHY RST_N (TP709) / INT_N (TP710) / MDIO link status (TP713/14) / Ethernet output |
| B — PMIC system reset | 2nd PMIC resets / cuts power to the SoC (Q&A watchdog or voltage violation) | PMIC_ISP_RST_N (TP1207) asserted, SoC reset, rails drop → PHY stops |

10.2  Observable signals (ranked)

PRIMARY (definitional): PHY inactive — no image data on the 100BASE-T1 output / PHY TX. Capture the Ethernet line; link down / no packets = SafeState.

PMIC ALERT#  (MPQ70165FS U500 pin 16) — low = PMIC signalled a FuSa error. SoC reacts: ALERT# low → SafeState (det 50 ms + react 300 ms, MVC-108192).

PMIC PG (power-good), net PG_PMIC (Sheet 5) — low = OV/UV/FuSa error → reset / power-cut path.

SoC safety error pin — asserted LOW, routed via the CEHU (“Verify the SoC safety pin is asserted low”).

PMIC_ISP_RST_N (TP1207) — asserted = PMIC reset the SoC (Variant B). PMIC watchdog stimulus at WDI (TP510/TP514).

Cause signals the SafetyCore monitors (MVC-100147), not SafeState itself: PMIC PG & ALERT#, PHY error indication, Deserializer ERRB (TP1232) / LOCK (TP1215).

10.3  Design & recovery notes

Measure the common endpoint (PHY inactive) for the timing budget, then classify which path fired via TP1207 (reset) vs. ALERT#/PHY-command (link disable).

Stays in SafeState while the trigger persists (MVC-100139); PHY re-activates when the fault is removed. Camera-head recovery = SoC power-cycles PoC over the coax (MVC-100224).

For ‘log a trouble code’ requirements, define how the code is read back (diagnostic interface / DTC memory) so the log-only checks can be verified.

11.  Open Dependencies (resolve before bench freeze)

MAX96714 Safety Manual (DOC 32-100741 Rev D) — fault list / diagnostic / FTTI (SF2–SF4); and TJA1104 Safety Manual — for the exact PHY fault set and any interrupt-force / self-test injection hook.

MVC-100121 — SOW1 peripheral (imager) access-channel definition; and the MFP9/MFP10 vs MFP1/MFP2 pass-through clarification.

SafeState now defined (§10): PHY-inactive via SoC PHY-disable OR PMIC reset. Still confirm: routing/polarity of PMIC ALERT#/PG to a probe header, and the SoC safety-error pin location.

SW debug hooks for detection-vs-reaction split (§6.3), and the SoC bus-relinquish mechanism (§8).

Confirm the real remote end (camera head) vs. a link emulator for GMSL2 tunnel / imager-readback TCs.