MAX96714 Deserializer

Functional-Safety Test-Access & Register Reference

Prepared by  alaa.said@brightskiesinc.com

Table of Contents

1.  Purpose & Scope	1

2.  Device Identification & Start-up (Strapped) Configuration	2

2.1  Identity, reset and mode registers	2

3.  Control-Bus Architecture — How the SoC Talks to the Deserializer	3

4.  Physical Signals, Pins & Test Points (U600, Sheet 6)	3

5.  How to Read & Write — Primary I²C Access Method	4

5.1  Addressing & transaction format	4

5.2  Safety of the primary I²C link — CRC + Message Counter (control channel)	4

6. Safety-Relevant Register Reference	4

6.1 LOCK & ERRB — enable and status	4

6.2 ERRB interrupt sources — error-enable (OEN) and flags	5

6.3 Line-fault status	5

6.4 Supply monitoring — VDD / VDD18 / VDDIO / VTERM  (map to voltage-range SafeState requirements)	5

6.5 Pass-through I²C & control-channel enables (REG1 / REG3)	5

6.6 Tunnelling mode	6

6.7 Register-map integrity (Register CRC) & FuSa interrupt aggregation	6

6.8 Power-On Self-Test / BIST (“At start-up the deserializer shall run …”)	6

7.  Requirement → Register / Signal / Test-Point Traceability	7

8.  Notes, Assumptions & Open Points for TC Design	8

# 1.  Purpose & Scope

This document supports design of Functional-Safety (FuSa) test cases for the MAX96714 deserializer (U600) as fitted on the MVCU01 vision-controller board. For each safety mechanism it answers the three questions needed to write a test case:

Which register / bit-field carries the information, its address, and whether it is readable, writable, or read-only.

How the SoC accesses it — which bus, which I²C address, and the transaction format (including the safety CRC / message-counter scheme).

Which physical signal or test point to observe on the board when the mechanism has to be stimulated or measured on the bench.

Scope is limited to the deserializer and the buses/signals that terminate on it. The remote serializer (MAX96717) and imager (S5K3B6SA) are reachable only through this deserializer and are covered only where the access path matters.

# 2.  Device Identification & Start-up (Strapped) Configuration

At power-up the device latches two configuration pins (CFG0, CFG1). These fix the I²C address and the link mode before any software runs — they are the baseline your start-up test cases check.

| Strap pin | IC pin | Net | Configured value (per schematic note) | Effect | Verify in register |
| --- | --- | --- | --- | --- | --- |
| CFG0 | 5 | DES_CFG0 | I²C mode, device address 0x50 | Selects I²C control and 7-bit address 0x28 → 8-bit write 0x50 / read 0x51. Strap set by R600/R601. | REG0(0x00)::DEV_ADDR ; REG6(0x06)::I2CSEL |
| CFG1 | 16 | DES_CFG1 | Coax, Tunnel mode, 3 Gbps | Selects coax (vs STP), GMSL2 tunnelling and 3 Gbps line rate. Strap set by R602/R603. | REG6(0x06)::GMSL2_A ; CTRL1(0x11)::CXTP_A ; RX_RATE ; MIPI_TX52(0x474)::TUN_EN |

## 2.1  Identity, reset and mode registers

| Register | Addr | Field[bits] | Access | Reset | Function / notes |
| --- | --- | --- | --- | --- | --- |
| REG13 | 0x0D | DEV_ID[7:0] | RO | 0xCA | Device variant identifier. 0xC9 = MAX96714, 0xCA = MAX96714F (this board), 0xCB = MAX96714R. Good first read to prove the bus works. (p.71) |
| REG14 | 0x0E | DEV_REV[3:0] | RO | 0x1 | Silicon revision. (p.71) |
| REG6 | 0x06 | GMSL2_A[6] | RW* | latched | 0 = GMSL1, 1 = GMSL2. Latched from CFG at power-up. (p.70) |
| REG6 | 0x06 | I2CSEL[4] | RW* | latched | 0 = UART, 1 = I²C. Latched from I2CSEL pin at power-up. (p.70) |
| CTRL1 | 0x11 | CXTP_A[0] | RW* | latched | Coax / twisted-pair select for Link A (“Coax cable select”). Latched from CXTP pin. (p.73) |
| REG0 | 0x00 | DEV_ADDR[7:1] | RW | 0b0101000 | Live 7-bit device address (0x28). Default from CFG0. (p.68) |
| REG0 | 0x00 | CFG_BLOCK[0] | RW | 0b0 | 1 = freeze/lock the register map read-only until power-cycle. (p.68) |
| CTRL0 | 0x10 | RESET_ALL[7] | RW | 0b0 | Write 1 = full device reset (equiv. to PWDNB toggle); self-clears. (p.72) |
| CTRL0 | 0x10 | RESET_LINK[6] | RW | 0b0 | Reset GMSL Link-A data path, keep register settings (“Changes to GMSL link”). (p.72-73) |
| CTRL0 | 0x10 | RESET_ONESHOT[5] | W/RO | 0b0 | One-shot link data-path reset; self-clears. (p.73) |

* RW but the reset value is loaded from a configuration pin at power-up; treat as “read to confirm the strap” in start-up TCs.

# 3.  Control-Bus Architecture — How the SoC Talks to the Deserializer

The Ambarella CV22FS SoC (labelled “ISP”) is the single I²C master. Three logical paths exist through this deserializer:

| Path | Direction | Signals / where to probe | Purpose & related requirement |
| --- | --- | --- | --- |
| Primary I²C (control channel) | SoC → U600 | ISP_DES_I2C.CLK / .DAT (net); IC pins 6 / 4 | Direct register read/write of the deserializer at address 0x50. This is the “primary-I2C bus” in the requirements; protect with CRC + message counter (Section 5). |
| Pass-through I²C (Ch.1 / Ch.2) | SoC → (through link) → remote side | MFP1/MFP2 (Ch.1) and MFP3/MFP4 remote; enabled by REG1 | Transparent I²C bridge to the remote serializer / imager. Must be enabled (IIC_1_EN / IIC_2_EN). Used for “read SOW1 parameter via pass-through I2C”. |
| GMSL2 tunnel (control forwarding) | SoC → U600 → coax → MAX96717 | SIO_P/SIO_N (pins 13/12), TP600 on coax | In tunnelling mode the reverse control channel carries register traffic to the remote serializer/imager over the single coax. “Read back data from serializer / imager” rides this path. |
| MIPI CSI-2 (video, not registers) | U600 → SoC | MIPI_CSI2.* (pins 21-30) | Pixel data only — no register access. Listed for completeness. |

# 4.  Physical Signals, Pins & Test Points (U600, Sheet 6)

Use this table to decide what to wire up on the bench and where to probe. Supply test points TP603–TP606 are the direct measurement points for the VDDIO / VTERM safety requirements.

| Signal / function | IC pin | Pin name | Net name | Test point | How it is used / measured |
| --- | --- | --- | --- | --- | --- |
| ERRB (fault out) | 32 | MFP8/ERRB/GPIO8 | DES_ISP_MFP8 | — (named net to SoC) | Active-low fault interrupt to SoC. Enabled by REG5::ERRB_EN. Probe the net at the SoC GPIO or IC pin 32. |
| LOCK (link locked) | 7 | MFP5/LOCK/GPIO5 | DES_ISP_MFP5 | — (named net to SoC) | Link-lock status to SoC. Enabled by REG5::LOCK_EN. Mirrors CTRL3::LOCKED. |
| Line-fault mon 0 | 18 | MFP6/LMN0/GPIO6 | DES_ISP_MFP6 | — | Line-fault monitor 0 (status in REG26::LF_0). |
| Line-fault mon 1 | 19 | MFP7/LMN1/GPIO7 | DES_ISP_MFP7 | — | Line-fault monitor 1 (status in REG26::LF_1). |
| Primary I²C SDA | 4 | MFP3/SDA/RX | ISP_DES_I2C.DAT | — | Primary control-channel I²C data. No dedicated TP; probe at SoC or IC pin 4. |
| Primary I²C SCL | 6 | MFP4/SCL/TX | ISP_DES_I2C.CLK | — | Primary control-channel I²C clock. Probe at SoC or IC pin 6. |
| Enable / power-down | 31 | PWDNB | DES_EN | — | Active-low power-down. Drive to gate the device on/off in TCs. |
| Reset | 10 | XRES | DES_XRES | — | Hardware reset input. |
| GMSL2 coax link | 13 / 12 | SIOP / SION | DES.SIO_P / _N | TP600 | Serial coax link to remote serializer (PoC). Observe link/fault here. |
| Crystal 25 MHz | 8 / 9 | X1 / X2 | DES.XI / DES.X1 | TP601 / TP602 | Reference clock (Y600). Probe for clock-presence checks. |
| VDD (core 1.0 V) | 15 | VDD | V_DES_VDD | TP604 | Core supply (LDO). Under-voltage flagged in PWR0(0x08). |
| VDD18 (1.8 V) | 11 | VDD18 | V_DES_VDD18 | TP603 | 1.8 V supply. Under-voltage flagged in PWR0(0x08)::CMP_STATUS. |
| VDDIO (1.8 V) | 17 | VDDIO | V_DES_VDDIO | TP605 | I/O supply — measurement point for the VDDIO safety requirement. UV latched in PWR0(0x08). |
| VTERM (PoC) | 20 | VTERM | V_DES_VTERM | TP606 | Coax termination / PoC supply — measurement point for the VTERM safety requirement. Status in REG7(0x07)::CMP_VTERM_STATUS. |
| CAP_VDD | 14 | CAP_VDD | V_DES_CAP_VDD | — | Internal 1 V cap node; UV in PWR0::VDDBAD_STATUS. |

# 5.  How to Read & Write — Primary I²C Access Method

## 5.1  Addressing & transaction format

Device address: 7-bit 0x28 (schematic “0x50” is the 8-bit write byte; read byte = 0x51). Set by CFG0 strap; live value in REG0(0x00)::DEV_ADDR.

Register address: 16-bit, sent as two bytes (high byte first) after the device-write address, followed by the data byte(s).

Write:  START – [0x50] – AddrHi – AddrLo – Data – STOP.

Read:   START – [0x50] – AddrHi – AddrLo – REPEATED START – [0x51] – Data – STOP.

CFG_BLOCK (REG0 bit0) or a set CRC-lock can make registers read-only — a write TC must account for this.

## 5.2  Safety of the primary I²C link — CRC + Message Counter (control channel)

The deserializer can protect the control channel with a CRC and a rolling message counter. These directly implement the “primary-I2C shall use CRC scheme” and “use Message Counter” requirements.

| Register | Addr | Field[bits] | Access | Function |
| --- | --- | --- | --- | --- |
| I2C_UART_CRC7 | 0x300F | CC_CRC_EN[1] | RW | Enable CRC on the control channel (primary I²C). |
| I2C_UART_CRC7 | 0x300F | CC_MSGCNTR_EN[2] | RW | Enable message counter on the control channel. |
| I2C_UART_CRC7 | 0x300F | CC_CRC_MSGCNTR_OVR[0] | RW | Override/behaviour control for CC CRC + msg-counter. |
| I2C_UART_CRC7 | 0x300F | MSGCNTR_PORT_SEL[4:3] | RW | Selects which port the message counter applies to. |
| I2C_UART_CRC1 | 0x3009 | CRC_ERR_THR[4:2] | RW | CRC error-count threshold before flag/interrupt. |
| I2C_UART_CRC1 | 0x3009 | MSGCNTR_ERR_THR[7:5] | RW | Message-counter error threshold. |
| I2C_UART_CRC1 | 0x3009 | RESET_CRC_ERR_CNT[0] / RESET_MSGCNTR_ERR_CNT[1] | RW | Clear the respective error counters. |
| I2C_UART_CRC2 | 0x300A | CRC_VAL[7:0] | RO | Current computed CRC value. |
| I2C_UART_CRC5 | 0x300D | CRC_ERR_CNT[7:0] | RO | Accumulated CRC-error count — read to detect a CRC error on a transfer. |
| I2C_UART_CRC6 | 0x300E | MSGCNTR_ERR_CNT[7:0] | RO | Accumulated message-counter-error count. |
| I2C_UART_CRC3/4 | 0x300B/0C | MSGCNTR_LSB / MSB | RO | Current 16-bit message-counter value. |
| I2C_UART_CRC0 | 0x3008 | RESET_MSGCNTR[0] | RW | Reset the message counter. |
| FS_INTR0 | 0x3010 | I2C_UART_CRC_ERR_OEN[6] / MSGCNTR_ERR_OEN[7] | RW | Route CRC / msg-counter errors to the ERRB pin. |
| FS_INTR1 | 0x3011 | I2C_UART_CRC_ERR_INT[6] / MSGCNTR_ERR_INT[7] | RO | Live CRC / msg-counter error flags (read after ERRB). |

Register-map (configuration) integrity is separately protected by the REGCRC block — see Section 6.6.

# 6. Safety-Relevant Register Reference

## 6.1 LOCK & ERRB — enable and status

| Register | Addr | Field[bits] | Access | Reset | Function / page |
| --- | --- | --- | --- | --- | --- |
| REG5 | 0x05 | LOCK_EN[7] | RW | 0b1 | Enables the LOCK output pin. In the start-up test case, set this bit to 1 if it reads 0. (p.70) |
| REG5 | 0x05 | ERRB_EN[6] | RW | 0b1 | Enable ERRB output pin. Start-up TC: if 0, set to 1. (p.70) |
| REG5 | 0x05 | PU_LF1[1] / PU_LF0[0] | RW | 0b0 | Power-up line-fault monitor 1 / 0. Enable for line-fault reporting. (p.70) |
| REG3 | 0x03 | LOCK_CFG[7] | RW | 0b0 | 0 = LOCK asserts on link lock; 1 = on link lock AND MIPI output started. (p.69) |
| CTRL3 | 0x13 | LOCKED[3] | RO | 0b0 | GMSL2 link locked (bidirectional). Read on LOCK event. (p.73-74) |
| CTRL3 | 0x13 | ERROR[2] | RO | 0b0 | Inverse of ERRB pin: 1 = ERRB asserted. Read on ERRB event. (p.74) |
| CTRL3 | 0x13 | CMU_LOCKED[1] | RO | 0b0 | Main clock / crystal detected & locked. (p.74) |
| INTR0 | 0x18 | LOCK_OEN[4] | RW | 0b0 | Report GMSL2 lock status at ERRB pin. (p.74) |

## 6.2 ERRB interrupt sources — error-enable (OEN) and flags

When ERRB asserts, the SoC reads these flag registers to find the cause; the matching OEN bit decides whether that source is allowed to pull ERRB.

| Register | Addr | Field[bits] | Access | Reset | Function / page |
| --- | --- | --- | --- | --- | --- |
| INTR2 | 0x1A | LFLT_INT_OEN[3] | RW | 0b1 | Enable line-fault interrupt reporting at ERRB. (p.74) |
| INTR2 | 0x1A | DEC_ERR_OEN_A[0] / IDLE_ERR_OEN[2] | RW | 0b1/0b0 | Enable decode-error / idle-word-error reporting at ERRB. (p.74-75) |
| INTR3 | 0x1B | LFLT_INT[3] | RO | 0b0 | Line-fault interrupt flag (see REG26). (p.75) |
| INTR3 | 0x1B | DEC_ERR_FLAG_A[0] / IDLE_ERR_FLAG[2] | RO | 0b0 | Decode / idle-word error flags. (p.75) |
| INTR3 | 0x1B | REM_ERR_FLAG[5] | RO | 0b0 | Remote-side (serializer) error — inverse of remote ERRB. (p.75) |
| INTR4/5 | 0x1C/1D | MAX_RT_OEN/FLAG, RT_CNT, PKT_CNT, EOM_ERR | RW/RO | — | ARQ max-retransmit, retransmit-count, packet-count, eye-monitor error enables & flags. (p.75-76) |
| INTR6 | 0x1E | VDDCMP_INT_OEN[7], VDDBAD_INT_OEN[5], FSYNC/LCRC/VPRBS/VID_PXL_CRC_OEN | RW | — | Enable supply-comparator, frame-sync, line-CRC, video-PRBS and pixel-CRC error reporting at ERRB. (p.76-77) |
| INTR7 | 0x1F | VDDCMP_INT_FLAG[7], VDDBAD_INT_FLAG[5], FSYNC/LCRC/VPRBS/VID_PXL flags | RO | 0b0 | Live flags for the INTR6 sources. (p.77) |
| INTR9 | 0x21 | ERR_RX_EN[7], ERR_RX_RECVED[6], ERR_RX_ID[4:0] | RW/RO | — | Remote/serializer error received over link. (p.77) |

## 6.3 Line-fault status

| Register | Addr | Field[bits] | Access | Function / page |
| --- | --- | --- | --- | --- |
| REG26 | 0x26 | LF_0[2:0] | RO | Line-fault on wire at LMN0: 000=short-to-batt, 001=short-to-GND, 010=normal, 011=open, 1XX=line-to-line short. (p.71) |
| REG26 | 0x26 | LF_1[6:4] | RO | Line-fault on wire at LMN1 (same decode). (p.71) |
| INTR3 | 0x1B | LFLT_INT[3] | RO | Asserted when either monitor reports a fault (sticky if LFLT_STKY set). (p.75) |

## 6.4 Supply monitoring — VDD / VDD18 / VDDIO / VTERM  (map to voltage-range SafeState requirements)

| Register | Addr | Field[bits] | Access | Reset | Function / page |
| --- | --- | --- | --- | --- | --- |
| PWR0 | 0x08 | CMP_STATUS[4:0] | RO | 0b00000 | Under-voltage comparators. Bit0=VDD18<1.617 V; bit1=VDDIO<1.617 V; bit2=CAP_VDD<0.82 V (latched low; 0bXX111 = all OK). Measure VDDIO at TP605. (p.72) |
| PWR0 | 0x08 | VDDBAD_STATUS[7:5] | RO | 0b000 | Switched-1V (CAP_VDD) under-voltage latch. (p.72) |
| REG7 | 0x07 | CMP_VTERM_STATUS[7] | RO | 0b0 | VTERM comparator: latched low when VTERM < 1 V; read-to-clear when VTERM > 1 V. Measure VTERM at TP606. (p.70) |
| INTR6 | 0x1E | VDDCMP_INT_OEN[7] / VDDBAD_INT_OEN[5] | RW | 0b0 | Route supply-comparator / VDD-bad events to ERRB. (p.76) |
| INTR7 | 0x1F | VDDCMP_INT_FLAG[7] / VDDBAD_INT_FLAG[5] | RO | 0b0 | Live supply-fault flags. (p.77) |
| INTR10 | 0x5010 | VDD18_OV_OEN[4] / VDD_OV_OEN[0] | RW | — | Enable over-voltage reporting for VDD18 / VDD. (p.67) |
| INTR11 | 0x5011 | VDD18_OV_FLAG[4] / VDD_OV_FLAG[0] | RO | — | Over-voltage flags for VDD18 / VDD. (p.67) |

## 6.5 Pass-through I²C & control-channel enables (REG1 / REG3)

| Register | Addr | Field[bits] | Access | Reset | Function / page |
| --- | --- | --- | --- | --- | --- |
| REG1 | 0x01 | IIC_2_EN[7] | RW | 0b0 | Enable pass-through I²C Channel 2 (SDA2/SCL2). Reset = disabled — SW must enable for remote reads. (p.68) |
| REG1 | 0x01 | IIC_1_EN[6] | RW | 0b0 | Enable pass-through I²C Channel 1 (SDA1/SCL1). Reset = disabled. (p.68) |
| REG1 | 0x01 | DIS_LOCAL_CC[5] | RW | 0b0 | 1 = disconnect RX/SDA & TX/SCL from local control channel. (p.69) |
| REG1 | 0x01 | DIS_REM_CC[4] | RW | 0b0 | 1 = disable remote control channel (Link-A) over GMSL2. (p.69) |
| REG1 | 0x01 | TX_RATE[3:2] / RX_RATE[1:0] | RW | 00 / 10 | Link rates; RX_RATE actual value latched from CFG1 = 3 Gbps (0b01) at power-up. (p.69) |
| REG3 | 0x03 | UART_2_EN[5] / UART_1_EN[4] | RW | 0b0/0b1 | Pass-through UART channel enables (I²C variant uses IIC_*). (p.69) |
| REG3 | 0x03 | PT_SWAP[6] | RW | 0b1 | Swap I²C/UART pass-through pin assignments. (p.69) |

## 6.6 Tunnelling mode

| Register | Addr | Field[bits] | Access | Function / page |
| --- | --- | --- | --- | --- |
| MIPI_TX52 | 0x474 | TUN_EN[0] | RW | GMSL2 tunnel-mode enable on the deserializer. Check once per driver cycle that it is still set. Board is strapped to tunnel mode via CFG1. (p.62/156) |
| MIPI_TX52 | 0x474 | TUN_NO_CORR[7] / DESKEW_TUN[6:5] | RW | Tunnel deskew / correction options. (p.62/156) |
| MIPI_TX51 | 0x473 | TUN_WAIT_VS_START[2:0] | RW | Tunnel VS-frame wait / DT memory mapping. (p.62/155) |

Note: GMSL2 tunnelling is primarily configured on the serializer (MAX96717); the deserializer must match. Confirm the serializer-side tunnel enable when writing the “configure serializer in tunnelling mode” test case.

## 6.7 Register-map integrity (Register CRC) & FuSa interrupt aggregation

| Register | Addr | Field[bits] | Access | Function / page |
| --- | --- | --- | --- | --- |
| REGCRC0 | 0x3000 | GEN_ROLLING_CRC[4], I2C_WR_COMPUTE[3], PERIODIC_COMPUTE[2], CHECK_CRC[1], RESET_CRC[0] | RW | Configure/trigger the register-map CRC engine (periodic or on-write). (p.66) |
| REGCRC1 | 0x3001 | CRC_PERIOD[7:0] | RW | Period for periodic register-CRC computation. (p.66) |
| REGCRC2/3 | 0x3002/03 | REGCRC_LSB / MSB | RO | Computed register-map CRC value. (p.66) |
| FS_INTR0 | 0x3010 | REG_CRC_ERR_OEN[0], EFUSE_CRC_ERR_OEN[1], MEM_ECC_ERR1/2_OEN[4/5] | RW | Route register-CRC, eFuse-CRC and memory-ECC errors to ERRB. (p.67) |
| FS_INTR1 | 0x3011 | REG_CRC_ERR_FLAG[0], EFUSE_CRC_ERR_FLAG[1], MEM_ECC_ERR1/2_INT[4/5] | RO | Live FuSa error flags. (p.67) |
| MEM_ECC0 | 0x3016 | MEM_ECC_ERR1/2_THR, RESET_*_CNT | RW | Memory-ECC thresholds & counter resets. (p.67) |
| MEM_ECC1/2 | 0x3017/18 | MEM_ECC_ERR1/2_CNT[7:0] | RO | Memory-ECC error counts. (p.67) |

## 6.8 Power-On Self-Test / BIST (“At start-up the deserializer shall run …”)

| Register | Addr | Field[bits] | Access | Function / page |
| --- | --- | --- | --- | --- |
| REG_POST0 | 0x3020 | POST_DONE[7] | RO | Power-On Self-Test completed. Start-up TC waits for this = 1. (p.67) |
| REG_POST0 | 0x3020 | POST_MBIST_PASSED[6] / POST_LBIST_PASSED[5] | RO | Memory-BIST / Logic-BIST result. Both must read 1 (pass). (p.67) |
| REG_POST0 | 0x3020 | POST_RUN_MBIST[1] / POST_RUN_LBIST[0] | RW | Trigger/select which BIST runs. (p.67) |

# 7.  Requirement → Register / Signal / Test-Point Traceability

Each row maps one of the sample safety requirements to the exact register, signal and (where relevant) test point to use in the test case.

| Safety requirement (sample) | Register / field (address) | Signal / IC pin | Net | Test pt |
| --- | --- | --- | --- | --- |
| When SoC SW receives ERRB → check cause | CTRL3(0x13)::ERROR; then INTR3(0x1B), INTR5(0x1D), INTR7(0x1F), FS_INTR1(0x3011), REG26(0x26) | ERRB pin (MFP8, pin 32) | DES_ISP_MFP8 net | — |
| LOCK asserted → react | CTRL3(0x13)::LOCKED | LOCK pin (MFP5, pin 7) | DES_ISP_MFP5 | — |
| LOCK becomes disabled → react | CTRL3(0x13)::LOCKED = 0 | LOCK pin (MFP5) | DES_ISP_MFP5 | — |
| ERRB becomes disabled → react | REG5(0x05)::ERRB_EN; CTRL3::ERROR | ERRB pin (MFP8) | DES_ISP_MFP8 | — |
| Deserializer detects FuSa problem | FS_INTR1(0x3011) flags; INTR7(0x1F) | ERRB pin | DES_ISP_MFP8 | — |
| Communicate via primary-I2C bus | device @0x50; REG13(0x0D) read-back to confirm | SDA pin4 / SCL pin6 | ISP_DES_I2C.DAT/.CLK | — |
| At start-up deserializer runs self-test | REG_POST0(0x3020)::POST_DONE / MBIST / LBIST | — | — | — |
| Enable line-fault interrupt reporting | INTR2(0x1A)::LFLT_INT_OEN:=1; REG5::PU_LF0/PU_LF1 | LMN0/LMN1 (pins 18/19) | DES_ISP_MFP6/7 | — |
| Read FuSa param; CRC error → re-read ≤ 50 ms | CRC scheme: I2C_UART_CRC5(0x300D)::CRC_ERR_CNT; FS_INTR1::I2C_UART_CRC_ERR_INT | SDA/SCL | ISP_DES_I2C | — |
| Re-read still CRC error → SafeState ≤ 200 ms | same CRC registers | SDA/SCL | ISP_DES_I2C | — |
| Primary-I2C shall use CRC scheme | I2C_UART_CRC7(0x300F)::CC_CRC_EN:=1 | SDA/SCL | ISP_DES_I2C | — |
| Use Message Counter for primary-I2C | 0x300F::CC_MSGCNTR_EN:=1; MSGCNTR_LSB/MSB(0x300B/0C) | SDA/SCL | ISP_DES_I2C | — |
| Start-up: test CRC scheme is working | force/observe CRC_ERR_CNT(0x300D); FS_INTR1(0x3011) | SDA/SCL | ISP_DES_I2C | — |
| Compare results of each read-action (2×) | read same FuSa register twice; SW compare | SDA/SCL | ISP_DES_I2C | — |
| Check once/cycle REG1::IIC_2_EN == 1 | REG1(0x01)::IIC_2_EN (reset 0 → must be enabled) | — | — | — |
| Check once/cycle REG1::IIC_1_EN == 1 | REG1(0x01)::IIC_1_EN | — | — | — |
| Check REG1::DIS_REM_CC / DIS_LOCAL_CC | REG1(0x01)::DIS_REM_CC[4] / DIS_LOCAL_CC[5] | — | — | — |
| Read SOW1 param via pass-through I2C 2× ≤ 50 ms | enable REG1::IIC_1/2_EN; access remote via pass-through | pass-through I2C | MFP1/MFP2 | — |
| Configure serializer in tunnelling mode | serializer side; deser MIPI_TX52(0x474)::TUN_EN | coax link | DES.SIO_P/_N | TP600 |
| Check tunnelling still set once/cycle | MIPI_TX52(0x474)::TUN_EN | — | — | — |
| GMSL2 & I2C selection remain as at start-up | REG6(0x06)::GMSL2_A / I2CSEL | — | — | — |
| Coax cable select | CTRL1(0x11)::CXTP_A | coax link | DES.SIO | TP600 |
| Start-up: check ERRB pin enabled, else set | REG5(0x05)::ERRB_EN := 1 | ERRB pin | DES_ISP_MFP8 | — |
| Start-up: check LOCK pin enabled, else set | REG5(0x05)::LOCK_EN := 1 | LOCK pin | DES_ISP_MFP5 | — |
| Changes to GMSL link | CTRL0(0x10)::RESET_LINK / RESET_ONESHOT | coax link | DES.SIO | TP600 |
| Read back data from deserializer | any RO register at 0x50 | SDA/SCL | ISP_DES_I2C | — |
| Read back data from serializer / imager | pass-through / tunnel via link | coax link | DES.SIO | TP600 |
| Ensure proper VDDIO, else SafeState | PWR0(0x08)::CMP_STATUS (bit1 VDDIO<1.617 V); INTR7 | VDDIO rail | V_DES_VDDIO | TP605 |
| Ensure proper VTERM, else SafeState | REG7(0x07)::CMP_VTERM_STATUS (<1 V latched) | VTERM rail | V_DES_VTERM | TP606 |
| Junction temp within –40…125 °C, else SafeState | No die-temp read-back register in MAX96714 DS — see Open Points (§8) | — | — | — |

# 8.  Notes, Assumptions & Open Points for TC Design

I²C address nuance: the schematic “Address 0x50” is the 8-bit write byte (read = 0x51); the 7-bit address is 0x28. Use the form your I²C master expects.

Pass-through enables (REG1::IIC_1_EN / IIC_2_EN) reset to 0 (disabled). Any test that reads the remote serializer/imager must first enable them — this is why the requirement checks them once per driver cycle.

LOCK_EN / ERRB_EN (REG5) reset to 1 (already enabled). The “check-and-enable-if-not” requirement is still valid as a defensive start-up step.

Line rate: REG1::RX_RATE reset field reads 0b10, but the actual rate is latched from CFG1 at power-up to 3 Gbps (0b01). Read after power-up rather than trusting the tabulated reset.

Junction-temperature requirement: the MAX96714 data sheet section reviewed exposes no die-temperature read-back register (only –40…+125 °C as an operating limit). The temperature SafeState mechanism is most likely served by the imager, SoC or PMIC sensor — confirm the intended source before writing that TC.

Tunnelling is configured primarily on the serializer (MAX96717); the deserializer (MIPI_TX52::TUN_EN) must match. Cross-check both ends.

Test points TP603–TP606 map to VDD18 / VDD / VDDIO / VTERM respectively — use TP605 (VDDIO) and TP606 (VTERM) for the supply-range SafeState requirements; TP600 is the coax link for GMSL/serializer observation.

Page numbers cite deserializer_max96714.pdf as read; verify against the controlled revision, and confirm which BOM02I option populates the PHY footprint if PHY-related TCs are combined.