Repository Browser < 11_P_01467_MEKRA_Vision_Controller

Repository Browser < 11_P_01467_MEKRA_Vision_Controller

FSR:

- SafeState definition

  - The SafeState is represented as NO presence of ‘Video Stream’ on the communication link towards vehicle

- Video Stream definition

  - A sequence of distinct images

- Delayed Video Stream

  - A ‘Video Stream’ consisting of images that spent more than 50ms inside the system

- Frozen Video Stream

  - A sequence of identical images

- Detail Lossy Stream

  - A ‘Video Stream’ featuring artefacts that result in the loss of details (such as:  incorrect contrast/brightness/resolution/frame rate/excessive noise)

- Wrong FieldOfView Stream

  - A ‘Video Stream’ featuring incorrect field of view (zoom, ROI, etc..)

- SafetyRequirement Violation

  - The condition of the system when the SafetyRequirement can no longer be fulfilled.

- Transition into SafeState

  - The system performs actions required to achieve the SafeState

- SG1:

  - The system shall avoid presenting ‘Delayed Video Stream’ at any time

  - The system shall not transmit a ‘Delayed Video Stream’ older than 400ms (Information: FHTI == 400ms )

  - If applicable, the system shall monitor its condition and detect possible ‘SafetyRequirement Violation’:’Delayed Video Stream’

  - If presentation of ‘Video Stream‘ cannot be fulfilled (‘SafetyRequirement Violation’:’Delayed Video Stream’), the system shall ‘Transition into SafeState’

  - Interfaces:

    - N.A.

  - Functionality

    - The system shall monitor the 'local time' (availability)

    - The system shall monitor the 'local time' (too slow)

    - The system shall monitor the 'label correctness'

    - the system shall monitor the 'label position in frame'

    - The system shall monitor the 'label content'

    - The system shall monitor the 'label retrieval - position'

    - The system shall monitor the 'label retrieval - content'

    - The system shall ensure internal communication w.r.t. the 'label'

    - The system shall monitor the 'safe state transition decision'

- SG2:

  - The system shall avoid presenting ‘Frozen Video Stream’ at any time

  - The system shall not transmit a ‘Frozen Video Stream’ older than 400ms (Information: FHTI == 400ms )

  - If applicable, the system shall monitor its condition and detect possible ‘SafetyRequirement Violation’:’Frozen Video Stream’

  - If presentation of ‘Video Stream’ cannot be fulfilled (‘SafetyRequirement Violation’:’Frozen Video Stream’), the system shall ‘Transition into SafeState’

  - Interfaces:

    - The system shall ensure fault free reception of RAWVideoStream from SOW1(camera head)

      - ASIL: B

    - fault free reception of RAWVideoStream shall be ensured

      - ASIL: B

    - The system shall ensure fault free reception of 'VideoStreamConfiguration' from User/Vehicle

      - ASIL: B

    - The system shall ensure fault free reception of 'DiagnosticsInformation' from SOW1(camera head)

      - ASIL: B

  - Functionality

    - The system shall ensure that image is stored in the right buffer position

      - ASIL: B

    - The system shall shall ensure that the same image can be dispatched N (TBD)

times at maximum.

      - ASIL: B

    - The system shall ensure that the sequence of incoming frames is preserved

      - ASIL: B

    - The system shall ensure fault free internal communication

      - ASIL: B

    - The system shall ensure fault free diagnostics

      - ASIL: B

    - the system shall ensure fault free FuSa-diagnostics

      - ASIL: B

    - The system shall ensure fault free  image processing

      - ASIL: B

    - The system shall ensure fault free image encoding

      - ASIL: B

    - The system shall ensure fault free  image processing

      - ASIL: B

    - The system shall ensure fault free image scaling

      - ASIL: B

    - The system shall ensure fault free image dispatchment

      - ASIL: B

    - The system shall ensure fault free configuration of the RAWStream

      - ASIL: B

    - The system shall ensure fault free image stream disabling

      - ASIL: B

    - The system shall ensure fault free invocation of image stream disabling

      - ASIL: B

    - The system shall ensure fault free image dispatchment

      - ASIL: B

    - The system shall ensure fault free configuration of image dispatchment

      - ASIL: B

    - The system shall ensure fault free ‘configuration storage’

      - ASIL: B

    - The system shall ensure fault free configuration of video stream

      - ASIL: B

    - The system shall ensure fault free provision of ImageMetaData

      - ASIL: B

    - The system shall ensure fault free internal provision of configuration

      - ASIL: B

  - Parameters

    - Following paramters shall be treated as SG2 relevant:

      - TBD

  - As ALTERNATIVE to the solution above the following could be considered (“global” Safety Mechanism)

- SG3:

  - The system shall avoid presenting ‘Detail Lossy Stream’ at any time

  - The system shall not transmit a ‘Detail Lossy Stream’ older than 400ms (Information: FHTI == 400ms )

  - If applicable, the system shall monitor its condition and detect possible ‘SafetyRequirement Violation’:‘Detail Lossy Stream’

  - If presentation of ‘Video Stream’ cannot be fulfilled (‘SafetyRequirement Violation’:‘Detail Lossy Stream’), the system shall ‘Transition into SafeState’

  - Interfaces

    - The system shall ensure fault free reception of RAWVideoStream from SOW1(camera head)

      - ASIL: A

    - The system shall ensure SOW1(camera head) related parameter integrity

      - ASIL: A

    - The system shall ensure fault free reception of 'VideoStreamConfiguration' from User/Vehicle

      - ASIL: A

    - The system shall ensure fault free reception of 'DiagnosticsInformation' from SOW1(camera head)

      - ASIL: A

  - Functionality

    - The system shall ensure fault free storage of a RAWStream

      - ASIL: A

    - The system shall ensure fault free retrieval of RAWStream

      - ASIL: A

    - The system shall ensure fault free configuration of the RAWStream

      - ASIL: A

    - The system shall ensure fault free  image processing

      - ASIL: A

    - The system shall ensure fault free configuration of image processing

      - ASIL: A

    - The system shall ensure fault free internal communication

      - ASIL: A

    - The system shall ensure fault free image scaling

      - ASIL: A

    - The system shall ensure fault free configuration of image scaling

      - ASIL: A

    - The system shall ensure fault free provision of ImageMetaData

      - ASIL: A

    - The system shall ensure fault free image encoding

      - ASIL: A

    - The system shall ensure fault free image dispatchment

      - ASIL: A

    - The system shall ensure fault free configuration of image encoding

      - ASIL: A

    - The system shall ensure fault free configuration of image dispatchment

      - ASIL: A

    - The system shall ensure fault free image stream disabling

      - ASIL: A

    - The system shall ensure fault free invocation of image stream disabling

      - ASIL: A

    - The system shall ensure fault free analysis of ‘image meta data’

      - ASIL: A

    - The system shall ensure fault free configuration of video stream

      - ASIL: A

    - The system shall ensure fault free internal provision of configuration

      - ASIL: A

    - The system shall ensure fault free ‘configuration storage’

      - ASIL: A

    - The system shall ensure fault free diagnostics

      - ASIL: A

    - The system shall ensure fault free FuSa-diagnostics

      - ASIL: A

  - Parameters

    - Following paramters shall be treated as SG3 relevant:

      - TBD

- SG4

  - The system shall avoid presenting ‘Wrong FieldOfView Stream’ at any time.

  - The system shall not transmit a ‘Wrong FieldOfView Stream’ older than 400ms (Information: FHTI == 400ms )

  - If applicable, the system shall monitor its condition and detect possible ‘SafetyRequirement Violation’:‘Wrong FieldOfView Stream’

  - If presentation of ‘Video Stream’ with correct field of view (zoom, ROI, etc…) can not be fulfilled (‘SafetyRequirement Violation’:‘Wrong FieldOfView Stream’), the system shall ‘Transition into SafeState’

  - Interfaces

    - The system shall ensure fault free reception of RAWVideoStream from SOW1(camera head)

      - ASIL: B

    - The system shall ensure SOW1(camera head) related parameter integrity

      - ASIL: B

    - The system shall ensure fault free reception of 'VideoStreamConfiguration' from User/Vehicle

      - ASIL: B

    - The system shall ensure fault free reception of 'DiagnosticsInformation' from SOW1(camera head)

      - ASIL: B

  - Functionality

    - The system shall ensure fault free configuration of the RAWStream

      - ASIL: B

    - The system shall ensure fault free 'image processing'

      - ASIL: B

    - The system shall ensure fault free configuration of image processing

      - ASIL: B

    - The system shall ensure fault free image scaling

      - ASIL: B

    - The system shall ensure fault free configuration of image scaling

      - ASIL: B

    - The system shall ensure fault free provision of ImageMetaData

      - ASIL: B

    - The system shall ensure fault free image encoding

      - ASIL: B

    - The system shall ensure fault free configuration of image encoding

      - ASIL: B

    - The system shall ensure fault free image stream disabling

      - ASIL: B

    - The system shall ensure fault free invocation of image stream disabling

      - ASIL: B

    - The system shall ensure fault free analysis of ‘image meta data’

      - ASIL: B

    - The system shall ensure fault free configuration of video stream

      - ASIL: B

    - The system shall ensure fault free internal provision of configuration

      - ASIL: B

    - The system shall ensure fault free ‘configuration storage‘

      - ASIL: B

    - The system shall ensure fault free diagnostics

      - ASIL: B

    - The system shall ensure fault free FuSa-diagnostics

      - ASIL: B

  - Parameters

    - Following paramters shall be treated as SG4 relevant:

      - TBD

Notes:

FTA-SG1:

Preventive Measure – SG2:

- ImageAcquisition

  - ImageAcquisition shall ensure that image is stored in the right buffer position

  - the ImageAcquisition (the indicator function) shall ensure that the same image can be dispatched N (TBD) times at maximum.

  - the ImageAcquisition (the provision function) shall ensure that the same image can be dispatched N (TBD) times at maximum.

  - the ImageAcquisition (reception function) shall ensure that the sequence of incoming frames is preserved

  - fault free (no sequence corruption/falsification) reception shall be ensured

  - fault free reception of RAWVideoStream shall be ensured

  - fault free (frozen image) reception of 'tmpDataImage' in 'temporary storage of RAW Data' shall be ensured

  - fault (integrity loss (frozen image) of 'RAWStreamConfiguration') free 'configureRAWStream' (function) shall be ensured

  - fault (frozen image) free reception of 'RAWStreamConfiguration' in 'configureRAWStream' function shall be ensured

- ImagePocessing

  - fault free reception of newImageIndicator shall be ensured

  - fault free reception of the RAW image shall be ensured

  - the ImageAcquisition (frame reception managing) shall be able to detect the SOW1-SafeState request and generate appropriate internal signal

  - fault free reception of 'Target Resolution Image' shall be ensured

  - fault free (no sequence corruption/falsification) image processing shall be ensured

  - fault free (no sequence corruption/falsification) image scaling shall be ensured

  - fault (frozen image) free reception of 'RAWDataImage' in 'provideImageMetaData' function shall be ensured

  - fault (frozen image) free 'provideImageMetaData' (function) shall be ensured

- ImageOutput

  - fault free reception of 'Encoded Image' shall be ensured

  - fault free reception of  'Full Resolution Processed Image' shall be ensured

  - fault free (no sequence corruption/falsification) image encoding shall be ensured

  - fault free (no sequence corruption/falsification) image dispatching shall be ensured

  - fault (frozen image) free invocation of 'disable stream' function shall be ensured

  - fault (frozen image) free function 'disableStream' shall be ensured

- VideoStreamManager

  - fault (frozen image) free reception of 'RAWDataImage' to 'provison of RAW Data' shall be ensured

  - fault (frozen image) free function 'receiveDiagnosticsInformation' shall be ensured

  - fault (frozen image) free reception of 'DiagnosticsInformation' in 'receiveDiagnosticsInformation' function shall be ensured

  - fault (frozen image) free function 'configureStream' shall be ensured

  - fault (frozen image) free reception of 'VideoStreamConfiguration' in 'configureStream' function shall be ensured

  - fault (frozen image) free reception of 'VideoConfiguration' in 'storeVideoStreamConfiguration' function shall be ensured

  - fault (frozen image) free function 'storeVideoStreamConfiguration' shall be ensured

  - fault (frozen image) free function 'provideConfiguration' shall be ensured

  - fault (frozen image) free function 'acquireDiagnostics' shall be ensured

  - fault (frozen image) free reception of 'FunctionalSafetyDiagnostics' in 'acquireDiagnostics' function shall be ensured

Preventive Measure – SG3:

- ImageAcquisition

  - fault free reception of RAWVideoStream shall be ensured

    - external interface SOW2 –SOW1

  - SOW1 raleted contrast-brightness parameter integrity shall be ensured

    - external interface SOW2 –SOW1

  - fault free reception (functionality) of RAW Stream via Input Interface shall be ensured

  - fault free (contrast-brightness) reception of 'tmpDataImage' in 'temporary storage of RAW Data' shall be ensured

  - fault (contrast-brightness) free storage of 'tmpDataImage' shall be ensured

  - fault (contrast-brightness) free reception of 'RAWDataImage' to 'provison of RAW Data' shall be ensured

  - fault (contrast-brightness) free 'provision of RAW  Data Image' (function) shall be ensured

  - fault (contrast-brightness) free reception of 'RAWStreamConfiguration' in 'configureRAWStream' function shall be ensured

  - fault (integrity loss (contrast-brightness) of 'RAWStreamConfiguration') free 'configureRAWStream' (function) shall be ensured

- ImageProcessing

  - faulty (contrast-brightness) free reception of 'rawImage' in 'receive RAW image' shall be ensured

  - fault (contrast-brightness) free reception of 'raw image' (function) shall be ensured

  - fault  (contrast-brightness) free reception of  'RAWImage' in 'processImage'

  - faulty (contrast-brightness) free reception of  'VideoConfiguration' in processingImage' shall be ensured

  - faulty (contrast-brightness) free 'processing image' (function) shall be ensured

  - fault free (contrast-brightness) reception of 'FullResolutionProcessedImage' in 'scale image' shall be ensured

  - fault free (contrast-brightness) reception of  'VideoConfiguration' in 'scale image' shall be ensured

  - faulty (contrast-brightness) free 'scale image' (function) shall be ensured

  - fault (contrast-brightness) free reception of 'RAWDataImage' in 'provideImageMetaData' function shall be ensured

  - fault (contrast-brightness) free 'provideImageMetaData' (function) shall be ensured

  - fault (contrast-brightness) free reception of 'VideoProcessingConfiguration' in 'configureProcessing' function shall be ensured

  - fault (integrity loss (contrast-brightness) of 'ProcessingConfiguration') free 'configureProcessing' (function) shall be ensured

  - fault (contrast-brightness) free reception of 'ScalingConfiguration' in 'configureScaling' function shall be ensured

  - fault ( integrity loss (contrast-brightness) of 'ScalingConfiguration') free 'configureScaling' (function) shall be ensured

- ImageOutput

  - fault (contrast-brightness) free reception of 'TargetResolutionImage' in 'encode image'

  - fault (contrast-brightness) free  reception of 'VideoConfiguration' in 'encode image' shall be ensured

  - fault (contrast-brightness) free 'encode image' (function) shall be ensured

  - fault (contrast-brightness) free reception of 'encodedImage' in 'dispatch image'

  - fault (contrast-brightness) free 'dispatch image' (function)

  - fault free (contrast-brightness) reception of 'EncodingConfiguration' in 'configureEncoding' function shall be ensured

  - fault (integrity loss (contrast-brightness) of 'EncodingConfiguration') free 'configureEncoding' (function)  shall be ensured

  - fault (contrast-brightness) free reception of 'DispatchmentConfiguration' in 'configureDispatchment' function

  - fault (integrity loss (contrast-brightness) of 'DispatchmentConfiguration') free 'configureDispatchment'

  - fault (contrast-brightness) free function 'disableStream' shall be ensured

  - fault (contrast-brightness) free invocation of 'disable stream' function shall be ensured

- VideoStreamManager

  - fault (contrast-brightness) free reception  of 'ImageMetaData' in 'analyseImageMetaData' function shall be ensured

  - fault (contrast-brightness) free 'analyseImageMetaData' (function) shall be ensured

  - fault (contrast-brightness) free reception of 'VideoStreamConfiguration' in 'configureStream' function shall be ensured

    - external interface SOW2 –Vehicle/User

  - fault (contrast-brightness) free function 'configureStream' shall be ensured

  - fault (contrast-brightness) free function 'provideConfiguration' shall be ensured

  - fault (contrast-brightness) free reception of 'DiagnosticsInformation' in 'receiveDiagnosticsInformation' function shall be ensured

    - external interface SOW2 –SOW1

  - fault (contrast-brightness) free function 'receiveDiagnosticsInformation' shall be ensured

  - fault (contrast-brightness) free reception of 'VideoConfiguration' in 'storeVideoStreamConfiguration' function shall be ensured

  - fault (contrast-brightness) free function 'storeVideoStreamConfiguration' shall be ensured

- FunctionalSafetyManger

  - fault (contrast-brightness) free reception of 'FunctionalSafetyDiagnostics' in 'acquireDiagnostics' function shall be ensured

  - fault (contrast-brightness) free function 'acquireDiagnostics' shall be ensured

Preventive Measures – SG4:

- ImageAcquisition

  - fault free reception of RAWVideoStream shall be ensured

    - external interface SOW2 –SOW1

  - fault free reception (functionality) of RAW Stream via Input Interface shall be ensured

  - SOW1 related FoV parameter integrity shall be ensured

    - external interface SOW2 –SOW1

  - fault (FoV) free reception of 'RAWStreamConfiguration' in 'configureRAWStream' function shall be ensured

  - fault (integrity loss (FoV) of 'RAWStreamConfiguration') free 'configureRAWStream' (function) shall be ensured

- ImageProcessing

  - fault (FoV) free 'image processing' shall be ensured

  - fault (FoV) free  reception of 'VideoConfiguration' shall be ensured (processing)

  - fault (FoV) free  reception of 'VideoConfiguration' shall be ensured (scaler)

  - fault (FoV) free  'scale image' (function) shall be ensured

  - fault (FoV) free 'provideImageMetaData' (function) shall be ensured

  - fault (FoV) free reception of 'VideoProcessingConfiguration' in 'configureProcessing' function shall be esnured

  - fault (integrity loss (FoV) of 'ProcessingConfiguration') free 'configureProcessing' (function) shall be ensured

  - fault (FoV) free reception of 'ScalingConfiguration' in 'configureScaling' function shall be ensured

  - fault ( integrity loss (FoV) of 'ScalingConfiguration') free 'configureScaling' (function) shall be ensured

- ImageOutput

  - fault (FoV) free 'encode image' (function) shall be ensured

  - fault (FoV) free  reception of 'VideoConfiguration' shall be ensured (encoder)

  - fault free (FoV) reception of 'EncodingConfiguration' in 'configureEncoding' function shall be ensured

  - fault (integrity loss (FoV) of 'EncodingConfiguration') free 'configureEncoding' (function)  shall be ensured

  - fault (FoV) free function 'disableStream' shall be ensured

  - fault (FoV) free invocation of 'disable stream' function shall be ensured

- VideoStreamManager

  - fault (FoV) free reception  of 'ImageMetaData' in 'analyseImageMetaData' function shall be ensured

  - fault (FoV) free 'analyseImageMetaData' (function) shall be ensured

  - fault (FoV) free reception of 'VideoStreamConfiguration' in 'configureStream' function shall be ensured

    - external interface SOW2 –Vehicle/User

  - fault (FoV) free function 'configureStream' shall be ensured

  - fault (FoV) free function 'provideConfiguration' shall be ensured

  - fault (FoV) free reception of 'DiagnosticsInformation' in 'receiveDiagnosticsInformation' function shall be ensured

    - external interface SOW2 –SOW1

  - fault (FoV) free function 'receiveDiagnosticsInformation' shall be ensured

  - fault (FoV) free reception of 'VideoConfiguration' in 'storeVideoStreamConfiguration' function shall be ensured

  - fault (FoV) free function 'storeVideoStreamConfiguration' shall be ensured

- FunctionalSafetyManger:

  - fault (FoV) free reception of 'FunctionalSafetyDiagnostics' in 'acquireDiagnostics' function shall be ensured

  - fault (FoV) free function 'acquireDiagnostics' shall be ensured