Repository Browser < 11_P_01467_MEKRA_Vision_Controller

Repository Browser < 11_P_01467_MEKRA_Vision_Controller

FSR:

- Definitions:

  - SafeState definition

    - The SafeState is represented as NO output of ‘Video Stream’ on the communication link.

  - Video Stream definition

    - A sequence of distinct images

  - Delayed Video Stream

    - A ‘Video Stream’ consisting of images older than 50ms (compared to time of reception).

  - Frozen Video Stream

    - A sequence of identical images

  - VideoStreamConfiguration

    - tbd

  - DiagnosticsInformation

    - tbd

  - Detail Lossy Stream

    - A ‘Video Stream’ featuring artefacts that result in the loss of details (such as:  incorrect contrast/brightness/resolution/frame rate/excessive noise)

  - Wrong FieldOfView Stream

    - A ‘Video Stream’ featuring incorrect field of view (zoom, ROI, etc..)

  - SafetyRequirement Violation

    - The condition of the system when the SafetyRequirement can no longer be fulfilled.

  - RAWVideoStream

    - tbd

  - Transition into SafeState

    - The system performs actions required to achieve the SafeState

- SG1:

  - The system shall avoid presenting ‘Delayed Video Stream’ at any time

  - The system shall not transmit a ‘Delayed Video Stream’ older than 300ms (Information: FHTI = 400ms minus 50ms max. delay at SOW1 and minus 50ms delay in SOW2)

  - The system shall monitor its condition and detect possible ‘SafetyRequirement Violation’ with respect to ’Delayed Video Stream’.

  - If output of ‘Video Stream‘ cannot be provided (‘SafetyRequirement Violation’ with respect to ’Delayed Video Stream’), the system shall perform ‘Transition into SafeState’.

  - Interfaces:

    - N.A.

  - Functionality

    - The system shall monitor the 'local time' (availability)

    - The system shall monitor the 'local time' (too slow)

    - Image acquisition shall add a label, which contains the actual local time, to each frame.

    - Image output shall read the time label of each frame and compare it to the actual local time.

    - If the difference between the frame’s time label and the actual local time is higher than 50ms, the System shall perform ‘Transition into SafeState’.

    - The system shall ensure fault free internal communication between the separate function blocks.

      - ASIL: A ->valid for all SGs!!!

- SG2:

  - The system shall avoid presenting ‘Frozen Video Stream’ at any time.

  - The system shall not transmit a ‘Frozen Video Stream’ for more than 300ms (Information: FHTI = 400ms minus 50ms max. delay at SOW1 and minus 50ms delay in SOW2)

  - The system shall monitor its condition and detect possible ‘SafetyRequirement Violation’ with respect to 'Frozen Video Stream'.

  - If output of ‘Video Stream’ cannot be fulfilled (‘SafetyRequirement Violation’ with respect to ’Frozen Video Stream’), the system shall perform ‘Transition into SafeState’

  - Interfaces:

    - The system shall ensure fault free reception of 'RAWVideoStream' from SOW1(camera head).

      - ASIL: B

    - The system shall ensure fault free reception of 'VideoStreamConfiguration' from User/Vehicle.

      - ASIL: B

    - The system shall ensure fault free reception of 'DiagnosticsInformation' from SOW1(camera head).

      - ASIL: B

  - Functionality

    - ImageOutput shall read out the frame number of each frame and provide it to the FunctionalSafetyManager.

    - The FunctionalSafetyManager shall compare the current frame number with the number of the previous frame.

    - If the same frame number consecutively occurs for more than N times, the FunctionalSafetyManager declares the Video Stream as frozen.

    - If the FunctionalSafetyManager determines a 'Frozen Video Stream', the system shall perform ‘Transition into SafeState’.

    - The system shall ensure fault free internal communication between the separate function blocks.

      - ASIL: A ->valid for all SGs!!!

- SG3:

  - The system shall avoid presenting ‘Detail Lossy Stream’ at any time

  - The system shall not transmit a ‘Detail Lossy Stream’ older than 300ms (Information: FHTI == 300ms )

  - The system shall monitor its condition and detect possible ‘SafetyRequirement Violation’ with respect to ‘Detail Lossy Stream’

  - If output of ‘Video Stream’ cannot be provided (‘SafetyRequirement Violation’ with respect to ‘Detail Lossy Stream’), the system shall perform ‘Transition into SafeState’

  - Interfaces

    - The system shall ensure fault free reception of RAWVideoStream from SOW1(camera head)

      - ASIL: A

    - The system shall ensure fault free configuration of RAWVideoStream in SOW1.

      - ASIL: A

    - The system shall ensure SOW1(camera head) related parameter integrity

      - ASIL: A

    - The system shall ensure fault free reception of 'VideoStreamConfiguration' from User/Vehicle

      - ASIL: A

    - The system shall ensure fault free reception of 'DiagnosticsInformation' from SOW1(camera head)

      - ASIL: A

  - Functionality

    - ImageAcquisition shall ensure fault free storage of the separate images of the RAWStream.

      - ASIL: A

    - The System shall ensure fault free configuration of the VideoStream.

      - ASIL: A

    - The system shall ensure fault free image processing with respect to ‘Detail Lossy Stream’.

      - ASIL: A

    - The system shall ensure fault free internal communication between the separate function blocks.

      - ASIL: A ->valid for all SGs!!!

    - The system shall ensure fault free image scaling with respect to ‘Detail Lossy Stream’.

      - ASIL: A

    - The system shall ensure fault free image encoding

      - ASIL: A

    - The system shall ensure fault free image dispatchment

      - ASIL: A

    - The system shall ensure fault free handling of ‘image meta data’

      - ASIL: A

    - The system shall ensure fault free ‘configuration storage’

      - ASIL: A

  - Parameters

    - Following paramters shall be treated as SG3 relevant:

      - TBD

- SG4

  - The system shall avoid presenting ‘Wrong FieldOfView Stream’ at any time.

  - The system shall not transmit a ‘Wrong FieldOfView Stream’ older than 300ms (Information: FHTI == 300ms )

  - The system shall monitor its condition and detect possible ‘SafetyRequirement Violation’ with respect to ‘Wrong FieldOfView Stream’

  - If output of ‘Video Stream’ with correct field of view (zoom, ROI, etc…) cannot be fulfilled (‘SafetyRequirement Violation’ with respect to ‘Wrong FieldOfView Stream’), the system shall perform ‘Transition into SafeState’

  - Interfaces

    - The system shall ensure fault free reception of RAWVideoStream from SOW1(camera head)

      - ASIL: B

    - The system shall ensure fault free configuration of RAWVideoStream in SOW1.

      - ASIL: B

    - The system shall ensure SOW1(camera head) related parameter integrity

      - ASIL: B

    - The system shall ensure fault free reception of 'VideoStreamConfiguration' from User/Vehicle

      - ASIL: B

    - The system shall ensure fault free reception of 'DiagnosticsInformation' from SOW1(camera head)

      - ASIL: B

  - Functionality

    - The system shall ensure fault free configuration of the Video Stream.

      - ASIL: B

    - The system shall ensure fault free 'image processing' with respect to ‘Wrong FieldOfView Stream’.

      - ASIL: B

    - The system shall ensure fault free image scaling with respect to ‘Wrong FieldOfView Stream’.

      - ASIL: B

    - The system shall ensure fault free image encoding with respect to ‘Wrong FieldOfView Stream’.

      - ASIL: B

    - The system shall ensure fault free handling of ‘image meta data’

      - ASIL: B

    - The system shall ensure fault free ‘configuration storage‘

      - ASIL: B

    - The system shall ensure fault free internal communication between the separate function blocks.

      - ASIL: B ->valid for all SGs!!!

  - Parameters

    - Following paramters shall be treated as SG4 relevant:

      - TBD

Notes:

FTA-SG1:

Preventive Measure – SG2:

- ImageAcquisition

  - ImageAcquisition shall ensure that image is stored in the right buffer position

  - the ImageAcquisition (the indicator function) shall ensure that the same image can be dispatched N (TBD) times at maximum.

  - the ImageAcquisition (the provision function) shall ensure that the same image can be dispatched N (TBD) times at maximum.

  - the ImageAcquisition (reception function) shall ensure that the sequence of incoming frames is preserved

  - fault free (no sequence corruption/falsification) reception shall be ensured

  - fault free reception of RAWVideoStream shall be ensured

  - fault free (frozen image) reception of 'tmpDataImage' in 'temporary storage of RAW Data' shall be ensured

  - fault (integrity loss (frozen image) of 'RAWStreamConfiguration') free 'configureRAWStream' (function) shall be ensured

  - fault (frozen image) free reception of 'RAWStreamConfiguration' in 'configureRAWStream' function shall be ensured

- ImagePocessing

  - fault free reception of newImageIndicator shall be ensured

  - fault free reception of the RAW image shall be ensured

  - the ImageAcquisition (frame reception managing) shall be able to detect the SOW1-SafeState request and generate appropriate internal signal

  - fault free reception of 'Target Resolution Image' shall be ensured

  - fault free (no sequence corruption/falsification) image processing shall be ensured

  - fault free (no sequence corruption/falsification) image scaling shall be ensured

  - fault (frozen image) free reception of 'RAWDataImage' in 'provideImageMetaData' function shall be ensured

  - fault (frozen image) free 'provideImageMetaData' (function) shall be ensured

- ImageOutput

  - fault free reception of 'Encoded Image' shall be ensured

  - fault free reception of  'Full Resolution Processed Image' shall be ensured

  - fault free (no sequence corruption/falsification) image encoding shall be ensured

  - fault free (no sequence corruption/falsification) image dispatching shall be ensured

  - fault (frozen image) free invocation of 'disable stream' function shall be ensured

  - fault (frozen image) free function 'disableStream' shall be ensured

- VideoStreamManager

  - fault (frozen image) free reception of 'RAWDataImage' to 'provison of RAW Data' shall be ensured

  - fault (frozen image) free function 'receiveDiagnosticsInformation' shall be ensured

  - fault (frozen image) free reception of 'DiagnosticsInformation' in 'receiveDiagnosticsInformation' function shall be ensured

  - fault (frozen image) free function 'configureStream' shall be ensured

  - fault (frozen image) free reception of 'VideoStreamConfiguration' in 'configureStream' function shall be ensured

  - fault (frozen image) free reception of 'VideoConfiguration' in 'storeVideoStreamConfiguration' function shall be ensured

  - fault (frozen image) free function 'storeVideoStreamConfiguration' shall be ensured

  - fault (frozen image) free function 'provideConfiguration' shall be ensured

  - fault (frozen image) free function 'acquireDiagnostics' shall be ensured

  - fault (frozen image) free reception of 'FunctionalSafetyDiagnostics' in 'acquireDiagnostics' function shall be ensured

Preventive Measure – SG3:

- ImageAcquisition

  - fault free reception of RAWVideoStream shall be ensured

    - external interface SOW2 –SOW1

  - SOW1 raleted contrast-brightness parameter integrity shall be ensured

    - external interface SOW2 –SOW1

  - fault free reception (functionality) of RAW Stream via Input Interface shall be ensured

  - fault free (contrast-brightness) reception of 'tmpDataImage' in 'temporary storage of RAW Data' shall be ensured

  - fault (contrast-brightness) free storage of 'tmpDataImage' shall be ensured

  - fault (contrast-brightness) free reception of 'RAWDataImage' to 'provison of RAW Data' shall be ensured

  - fault (contrast-brightness) free 'provision of RAW  Data Image' (function) shall be ensured

  - fault (contrast-brightness) free reception of 'RAWStreamConfiguration' in 'configureRAWStream' function shall be ensured

  - fault (integrity loss (contrast-brightness) of 'RAWStreamConfiguration') free 'configureRAWStream' (function) shall be ensured

- ImageProcessing

  - faulty (contrast-brightness) free reception of 'rawImage' in 'receive RAW image' shall be ensured

  - fault (contrast-brightness) free reception of 'raw image' (function) shall be ensured

  - fault  (contrast-brightness) free reception of  'RAWImage' in 'processImage'

  - faulty (contrast-brightness) free reception of  'VideoConfiguration' in processingImage' shall be ensured

  - faulty (contrast-brightness) free 'processing image' (function) shall be ensured

  - fault free (contrast-brightness) reception of 'FullResolutionProcessedImage' in 'scale image' shall be ensured

  - fault free (contrast-brightness) reception of  'VideoConfiguration' in 'scale image' shall be ensured

  - faulty (contrast-brightness) free 'scale image' (function) shall be ensured

  - fault (contrast-brightness) free reception of 'RAWDataImage' in 'provideImageMetaData' function shall be ensured

  - fault (contrast-brightness) free 'provideImageMetaData' (function) shall be ensured

  - fault (contrast-brightness) free reception of 'VideoProcessingConfiguration' in 'configureProcessing' function shall be ensured

  - fault (integrity loss (contrast-brightness) of 'ProcessingConfiguration') free 'configureProcessing' (function) shall be ensured

  - fault (contrast-brightness) free reception of 'ScalingConfiguration' in 'configureScaling' function shall be ensured

  - fault ( integrity loss (contrast-brightness) of 'ScalingConfiguration') free 'configureScaling' (function) shall be ensured

- ImageOutput

  - fault (contrast-brightness) free reception of 'TargetResolutionImage' in 'encode image'

  - fault (contrast-brightness) free  reception of 'VideoConfiguration' in 'encode image' shall be ensured

  - fault (contrast-brightness) free 'encode image' (function) shall be ensured

  - fault (contrast-brightness) free reception of 'encodedImage' in 'dispatch image'

  - fault (contrast-brightness) free 'dispatch image' (function)

  - fault free (contrast-brightness) reception of 'EncodingConfiguration' in 'configureEncoding' function shall be ensured

  - fault (integrity loss (contrast-brightness) of 'EncodingConfiguration') free 'configureEncoding' (function)  shall be ensured

  - fault (contrast-brightness) free reception of 'DispatchmentConfiguration' in 'configureDispatchment' function

  - fault (integrity loss (contrast-brightness) of 'DispatchmentConfiguration') free 'configureDispatchment'

  - fault (contrast-brightness) free function 'disableStream' shall be ensured

  - fault (contrast-brightness) free invocation of 'disable stream' function shall be ensured

- VideoStreamManager

  - fault (contrast-brightness) free reception  of 'ImageMetaData' in 'analyseImageMetaData' function shall be ensured

  - fault (contrast-brightness) free 'analyseImageMetaData' (function) shall be ensured

  - fault (contrast-brightness) free reception of 'VideoStreamConfiguration' in 'configureStream' function shall be ensured

    - external interface SOW2 –Vehicle/User

  - fault (contrast-brightness) free function 'configureStream' shall be ensured

  - fault (contrast-brightness) free function 'provideConfiguration' shall be ensured

  - fault (contrast-brightness) free reception of 'DiagnosticsInformation' in 'receiveDiagnosticsInformation' function shall be ensured

    - external interface SOW2 –SOW1

  - fault (contrast-brightness) free function 'receiveDiagnosticsInformation' shall be ensured

  - fault (contrast-brightness) free reception of 'VideoConfiguration' in 'storeVideoStreamConfiguration' function shall be ensured

  - fault (contrast-brightness) free function 'storeVideoStreamConfiguration' shall be ensured

- FunctionalSafetyManger

  - fault (contrast-brightness) free reception of 'FunctionalSafetyDiagnostics' in 'acquireDiagnostics' function shall be ensured

  - fault (contrast-brightness) free function 'acquireDiagnostics' shall be ensured

Preventive Measures – SG4:

- ImageAcquisition

  - fault free reception of RAWVideoStream shall be ensured

    - external interface SOW2 –SOW1

  - fault free reception (functionality) of RAW Stream via Input Interface shall be ensured

  - SOW1 related FoV parameter integrity shall be ensured

    - external interface SOW2 –SOW1

  - fault (FoV) free reception of 'RAWStreamConfiguration' in 'configureRAWStream' function shall be ensured

  - fault (integrity loss (FoV) of 'RAWStreamConfiguration') free 'configureRAWStream' (function) shall be ensured

- ImageProcessing

  - fault (FoV) free 'image processing' shall be ensured

  - fault (FoV) free  reception of 'VideoConfiguration' shall be ensured (processing)

  - fault (FoV) free  reception of 'VideoConfiguration' shall be ensured (scaler)

  - fault (FoV) free  'scale image' (function) shall be ensured

  - fault (FoV) free 'provideImageMetaData' (function) shall be ensured

  - fault (FoV) free reception of 'VideoProcessingConfiguration' in 'configureProcessing' function shall be esnured

  - fault (integrity loss (FoV) of 'ProcessingConfiguration') free 'configureProcessing' (function) shall be ensured

  - fault (FoV) free reception of 'ScalingConfiguration' in 'configureScaling' function shall be ensured

  - fault ( integrity loss (FoV) of 'ScalingConfiguration') free 'configureScaling' (function) shall be ensured

- ImageOutput

  - fault (FoV) free 'encode image' (function) shall be ensured

  - fault (FoV) free  reception of 'VideoConfiguration' shall be ensured (encoder)

  - fault free (FoV) reception of 'EncodingConfiguration' in 'configureEncoding' function shall be ensured

  - fault (integrity loss (FoV) of 'EncodingConfiguration') free 'configureEncoding' (function)  shall be ensured

  - fault (FoV) free function 'disableStream' shall be ensured

  - fault (FoV) free invocation of 'disable stream' function shall be ensured

- VideoStreamManager

  - fault (FoV) free reception  of 'ImageMetaData' in 'analyseImageMetaData' function shall be ensured

  - fault (FoV) free 'analyseImageMetaData' (function) shall be ensured

  - fault (FoV) free reception of 'VideoStreamConfiguration' in 'configureStream' function shall be ensured

    - external interface SOW2 –Vehicle/User

  - fault (FoV) free function 'configureStream' shall be ensured

  - fault (FoV) free function 'provideConfiguration' shall be ensured

  - fault (FoV) free reception of 'DiagnosticsInformation' in 'receiveDiagnosticsInformation' function shall be ensured

    - external interface SOW2 –SOW1

  - fault (FoV) free function 'receiveDiagnosticsInformation' shall be ensured

  - fault (FoV) free reception of 'VideoConfiguration' in 'storeVideoStreamConfiguration' function shall be ensured

  - fault (FoV) free function 'storeVideoStreamConfiguration' shall be ensured

- FunctionalSafetyManger:

  - fault (FoV) free reception of 'FunctionalSafetyDiagnostics' in 'acquireDiagnostics' function shall be ensured

  - fault (FoV) free function 'acquireDiagnostics' shall be ensured