Repository Browser < 11_P_01467_MEKRA_Vision_Controller

Repository Browser < 11_P_01467_MEKRA_Vision_Controller

FSR:

- Definitions:

  - SafeState definition

    - The SafeState is represented as NO output of any video image on the communication link towards vehicle.

  - Video Stream definition

    - A sequence of consecutive images

  - Delayed Video Stream

    - A ‘Video Stream’ consisting of images older than 50ms (compared to the time of reception).

  - Frozen Video Stream

    - A sequence of more than N (==2) identical images

  - VideoStreamConfiguration

    - All Functional Safety relevant parameters that are needed to guarantee safety of system (e.g.: zoom settings, FoV, max. operating temperature)

  - FS-DiagnosticsInformation

    - All Functional Safety relevant information that is needed to guarantee safety of system.

  - Detail Lossy Stream

    - ‘Video Stream’ that violates SG3

  - Wrong FieldOfView Stream

    - ‘Video Stream’ that violates SG4

  - SafetyRequirement Violation

    - The condition of the system when the SafetyRequirement can no longer be fulfilled.

  - RAWVideoStream

    - Payload of video transmission protocol at the input (from imager side – SOW1) of the system

  - System definition

    - System == SOW2

  - Active Operating Mode

    - System Output is on

  - fault free

    - sufficiently error-free to meet the required ASIL level

  - image meta data

    - properties or attributes of the video stream (derived or provided from SOW1)

- General

  - The system shall meet the functional safety requirements stated in this document for Active Operating Mode with 60fps (respectively 30fps) frame rates unless explicitly stated otherwise (e.g. another operating mode/frame rate is mentioned in the requirement text)

- FSRs for SG1 (The system shall avert a delay of the Video Stream (between its input and output) to more than 50ms.)

  - When the system cannot ensure that for each input video image the respective processed image is output within 50 ms, the system shall not transmit delayed images for longer than 300ms by achieving SafeState (Information: FHTI = 400ms minus 50ms max. delay at SOW1 and minus 50ms delay in SOW2). Delay is measured as follows: from start of a frame at the input to the start of the frame at the output OR equivalent, e.g.: end to end

  - ASIL: B

  - Note: “Lost frame”/“lost video stream” (no frame at the output) is considered safe

- Interfaces:

  - N.A.

- The system shall ensure that time measuring functionality is available

- The system shall ensure that time measuring functionality accuracy lies within +/- X[ms]

- The system shall ensure ‘fault free’ safety relevant internal communication.(e.g: w.r.t time stamp information/image identity)

  - ASIL: B

- FSRs for SG2 (The system shall avert a frozen image. 
Remark 1: The image is considered frozen, when given frame gets repeated.
Remark 2: Due to technical implementation, one (single!) repetition of the frame may be allowed.):

  - If the requirement 3.a is not fulfilled, the system shall perform transition into the SafeState within 300ms – detection_time - N*1/MIN[fpsIn; fpsOut]. (Information/Observation: in case of low frame rates realized by means of frame dropping this should not negatively impact SG2)

    - ASIL: B

  - When the system cannot ensure that at the system output each video image is repeated (N-1)times at maximum, the system shall not transmit same images for longer than 300ms by achieving the SafeState

    - Note1: For definition of ‘N’, please refer to ‘Frozen Video Stream’ definition above;

    - Note2: term ‘repetition’ applies also to same video image sent in the past.

    - ASIL: B

  - Interfaces:

    - The system shall ensure ‘fault free’ reception of 'RAWVideoStream' SG2 relevant attributes (frame counter, integrity) from SOW1(camera head).

      - ASIL: B

    - The system shall check the strict monotonicity (with increment exactly: 1) of the frame number delivered from SOW1 (see: VT_CH3_SafetyOverview_MekraLang_202509010, v. 0.3, chapter 5.2)

      - ASIL: B

    - The system shall ensure ‘fault free’ reception of 'VideoStreamConfiguration' from User/Vehicle.

      - ASIL: QM

    - The transmission line between SOW1 and SOW2 shall not be able to store any data.

- SG3 (The system shall display the environment realistically at any time regarding contrast and brightness, to avoid losing any detail, which could end up in a hazard.
Video frames causing an unclear view (too dim, too low contrast, too much disturbances or noise, wrong resolution, flicker, too low frame rate) shall not be transmitted. ):

  - The system shall detect contrast deterioration occurred between its input and output

    - ASIL: A

  - The system shall detect brightness deterioration that occurred between its input and output

    - ASIL: A

  - The system shall not transmit deteriorated stream for longer than 300ms by achieving the SafeState.

    - ASIL: A

  - Interfaces

    - The system shall ensure ‘fault free’ reception of RAWVideoStream from SOW1(camera head)

      - ASIL: A

    - The system shall ensure ‘fault free’ configuration of RAWVideoStream in SOW1.

      - ASIL: A

    - The system shall ensure ‘fault free’ reception of 'VideoStreamConfiguration' from User/Vehicle

      - ASIL: A

- FSRs for SG4 (The system has to show correct angle of view, zoom factor and visual field at any time, )

  - Interfaces

    - The system shall ensure ‘fault free’ reception of RAWVideoStream from SOW1(camera head)

      - ASIL: B

    - The system shall ensure ‘fault free’ configuration of RAWVideoStream in SOW1.

      - ASIL: B

    - The system shall ensure ‘fault free’ reception of 'VideoStreamConfiguration' from User/Vehicle

      - ASIL: B

- The system shall check that the SG4 relevant 'VideoStreamConfiguration' was correctly set at start-up.

    - ASIL: B

- The system shall check that the SG4 relevant 'VideoStreamConfiguration' was correctly set at each change of the data.

    - ASIL: B

  - When the system cannot ensure data integrity of the SG4 relevant 'VideoStreamConfiguration', the system shall not transmit ‘Video Stream’ for longer than 300ms by achieving the SafeState

    - ASIL: B

  - The system shall ensure ‘fault free’ configuration storage

    - ASIL: B

  - The system shall ensure ‘fault free’ handling of ‘image meta data’

    - ASIL: B

  - The system shall ensure ‘fault free’ safety relevant internal communication

    - ASIL: B

# SOW1 Interface – General requirements

- The system shall check that the 'VideoStreamConfiguration' was correctly written at SOW1 at start-up.

  - ASIL: A(SG3)/B(SG4)

- The system shall check that the 'VideoStreamConfiguration' was correctly written at SOW1 at each change of the data.

  - ASIL: A(SG3) /B(SG4)

- The system shall ensure ‘fault free’ reception of 'FS-DiagnosticsInformation' from SOW1(camera head).

  - ASIL: A(SG3)/B(SG4)

Notes:

FTA-SG1:

Preventive Measure – SG2:

- ImageAcquisition

  - ImageAcquisition shall ensure that image is stored in the right buffer position

  - the ImageAcquisition (the indicator function) shall ensure that the same image can be dispatched N (TBD) times at maximum.

  - the ImageAcquisition (the provision function) shall ensure that the same image can be dispatched N (TBD) times at maximum.

  - the ImageAcquisition (reception function) shall ensure that the sequence of incoming frames is preserved

  - ‘fault free’ (no sequence corruption/falsification) reception shall be ensured

  - ‘fault free’ reception of RAWVideoStream shall be ensured

  - ‘fault free’ (frozen image) reception of 'tmpDataImage' in 'temporary storage of RAW Data' shall be ensured

  - fault (integrity loss (frozen image) of 'RAWStreamConfiguration') free 'configureRAWStream' (function) shall be ensured

  - fault (frozen image) free reception of 'RAWStreamConfiguration' in 'configureRAWStream' function shall be ensured

- ImagePocessing

  - ‘fault free’ reception of newImageIndicator shall be ensured

  - ‘fault free’ reception of the RAW image shall be ensured

  - the ImageAcquisition (frame reception managing) shall be able to detect the SOW1-SafeState request and generate appropriate internal signal

  - ‘fault free’ reception of 'Target Resolution Image' shall be ensured

  - ‘fault free’ (no sequence corruption/falsification) image processing shall be ensured

  - ‘fault free’ (no sequence corruption/falsification) image scaling shall be ensured

  - fault (frozen image) free reception of 'RAWDataImage' in 'provideImageMetaData' function shall be ensured

  - fault (frozen image) free 'provideImageMetaData' (function) shall be ensured

- ImageOutput

  - ‘fault free’ reception of 'Encoded Image' shall be ensured

  - ‘fault free’ reception of  'Full Resolution Processed Image' shall be ensured

  - ‘fault free’ (no sequence corruption/falsification) image encoding shall be ensured

  - ‘fault free’ (no sequence corruption/falsification) image dispatching shall be ensured

  - fault (frozen image) free invocation of 'disable stream' function shall be ensured

  - fault (frozen image) free function 'disableStream' shall be ensured

- VideoStreamManager

  - fault (frozen image) free reception of 'RAWDataImage' to 'provison of RAW Data' shall be ensured

  - fault (frozen image) free function 'receiveFS-DiagnosticsInformation' shall be ensured

  - fault (frozen image) free reception of 'FS-DiagnosticsInformation' in 'receiveFS-DiagnosticsInformation' function shall be ensured

  - fault (frozen image) free function 'configureStream' shall be ensured

  - fault (frozen image) free reception of 'VideoStreamConfiguration' in 'configureStream' function shall be ensured

  - fault (frozen image) free reception of 'VideoConfiguration' in 'storeVideoStreamConfiguration' function shall be ensured

  - fault (frozen image) free function 'storeVideoStreamConfiguration' shall be ensured

  - fault (frozen image) free function 'provideConfiguration' shall be ensured

  - fault (frozen image) free function 'acquireDiagnostics' shall be ensured

  - fault (frozen image) free reception of 'FunctionalSafetyDiagnostics' in 'acquireDiagnostics' function shall be ensured

Preventive Measure – SG3:

- ImageAcquisition

  - ‘fault free’ reception of RAWVideoStream shall be ensured

    - external interface SOW2 –SOW1

  - SOW1 raleted contrast-brightness parameter integrity shall be ensured

    - external interface SOW2 –SOW1

  - ‘fault free’ reception (functionality) of RAW Stream via Input Interface shall be ensured

  - ‘fault free’ (contrast-brightness) reception of 'tmpDataImage' in 'temporary storage of RAW Data' shall be ensured

  - fault (contrast-brightness) free storage of 'tmpDataImage' shall be ensured

  - fault (contrast-brightness) free reception of 'RAWDataImage' to 'provison of RAW Data' shall be ensured

  - fault (contrast-brightness) free 'provision of RAW  Data Image' (function) shall be ensured

  - fault (contrast-brightness) free reception of 'RAWStreamConfiguration' in 'configureRAWStream' function shall be ensured

  - fault (integrity loss (contrast-brightness) of 'RAWStreamConfiguration') free 'configureRAWStream' (function) shall be ensured

- ImageProcessing

  - faulty (contrast-brightness) free reception of 'rawImage' in 'receive RAW image' shall be ensured

  - fault (contrast-brightness) free reception of 'raw image' (function) shall be ensured

  - fault  (contrast-brightness) free reception of  'RAWImage' in 'processImage'

  - faulty (contrast-brightness) free reception of  'VideoConfiguration' in processingImage' shall be ensured

  - faulty (contrast-brightness) free 'processing image' (function) shall be ensured

  - ‘fault free’ (contrast-brightness) reception of 'FullResolutionProcessedImage' in 'scale image' shall be ensured

  - ‘fault free’ (contrast-brightness) reception of  'VideoConfiguration' in 'scale image' shall be ensured

  - faulty (contrast-brightness) free 'scale image' (function) shall be ensured

  - fault (contrast-brightness) free reception of 'RAWDataImage' in 'provideImageMetaData' function shall be ensured

  - fault (contrast-brightness) free 'provideImageMetaData' (function) shall be ensured

  - fault (contrast-brightness) free reception of 'VideoProcessingConfiguration' in 'configureProcessing' function shall be ensured

  - fault (integrity loss (contrast-brightness) of 'ProcessingConfiguration') free 'configureProcessing' (function) shall be ensured

  - fault (contrast-brightness) free reception of 'ScalingConfiguration' in 'configureScaling' function shall be ensured

  - fault ( integrity loss (contrast-brightness) of 'ScalingConfiguration') free 'configureScaling' (function) shall be ensured

- ImageOutput

  - fault (contrast-brightness) free reception of 'TargetResolutionImage' in 'encode image'

  - fault (contrast-brightness) free  reception of 'VideoConfiguration' in 'encode image' shall be ensured

  - fault (contrast-brightness) free 'encode image' (function) shall be ensured

  - fault (contrast-brightness) free reception of 'encodedImage' in 'dispatch image'

  - fault (contrast-brightness) free 'dispatch image' (function)

  - ‘fault free’ (contrast-brightness) reception of 'EncodingConfiguration' in 'configureEncoding' function shall be ensured

  - fault (integrity loss (contrast-brightness) of 'EncodingConfiguration') free 'configureEncoding' (function)  shall be ensured

  - fault (contrast-brightness) free reception of 'DispatchmentConfiguration' in 'configureDispatchment' function

  - fault (integrity loss (contrast-brightness) of 'DispatchmentConfiguration') free 'configureDispatchment'

  - fault (contrast-brightness) free function 'disableStream' shall be ensured

  - fault (contrast-brightness) free invocation of 'disable stream' function shall be ensured

- VideoStreamManager

  - fault (contrast-brightness) free reception  of 'ImageMetaData' in 'analyseImageMetaData' function shall be ensured

  - fault (contrast-brightness) free 'analyseImageMetaData' (function) shall be ensured

  - fault (contrast-brightness) free reception of 'VideoStreamConfiguration' in 'configureStream' function shall be ensured

    - external interface SOW2 –Vehicle/User

  - fault (contrast-brightness) free function 'configureStream' shall be ensured

  - fault (contrast-brightness) free function 'provideConfiguration' shall be ensured

  - fault (contrast-brightness) free reception of 'FS-DiagnosticsInformation' in 'receiveFS-DiagnosticsInformation' function shall be ensured

    - external interface SOW2 –SOW1

  - fault (contrast-brightness) free function 'receiveFS-DiagnosticsInformation' shall be ensured

  - fault (contrast-brightness) free reception of 'VideoConfiguration' in 'storeVideoStreamConfiguration' function shall be ensured

  - fault (contrast-brightness) free function 'storeVideoStreamConfiguration' shall be ensured

- FunctionalSafetyManger

  - fault (contrast-brightness) free reception of 'FunctionalSafetyDiagnostics' in 'acquireDiagnostics' function shall be ensured

  - fault (contrast-brightness) free function 'acquireDiagnostics' shall be ensured

Preventive Measures – SG4:

- ImageAcquisition

  - ‘fault free’ reception of RAWVideoStream shall be ensured

    - external interface SOW2 –SOW1

  - ‘fault free’ reception (functionality) of RAW Stream via Input Interface shall be ensured

  - SOW1 related FoV parameter integrity shall be ensured

    - external interface SOW2 –SOW1

  - fault (FoV) free reception of 'RAWStreamConfiguration' in 'configureRAWStream' function shall be ensured

  - fault (integrity loss (FoV) of 'RAWStreamConfiguration') free 'configureRAWStream' (function) shall be ensured

- ImageProcessing

  - fault (FoV) free 'image processing' shall be ensured

  - fault (FoV) free  reception of 'VideoConfiguration' shall be ensured (processing)

  - fault (FoV) free  reception of 'VideoConfiguration' shall be ensured (scaler)

  - fault (FoV) free  'scale image' (function) shall be ensured

  - fault (FoV) free 'provideImageMetaData' (function) shall be ensured

  - fault (FoV) free reception of 'VideoProcessingConfiguration' in 'configureProcessing' function shall be esnured

  - fault (integrity loss (FoV) of 'ProcessingConfiguration') free 'configureProcessing' (function) shall be ensured

  - fault (FoV) free reception of 'ScalingConfiguration' in 'configureScaling' function shall be ensured

  - fault ( integrity loss (FoV) of 'ScalingConfiguration') free 'configureScaling' (function) shall be ensured

- ImageOutput

  - fault (FoV) free 'encode image' (function) shall be ensured

  - fault (FoV) free  reception of 'VideoConfiguration' shall be ensured (encoder)

  - ‘fault free’ (FoV) reception of 'EncodingConfiguration' in 'configureEncoding' function shall be ensured

  - fault (integrity loss (FoV) of 'EncodingConfiguration') free 'configureEncoding' (function)  shall be ensured

  - fault (FoV) free function 'disableStream' shall be ensured

  - fault (FoV) free invocation of 'disable stream' function shall be ensured

- VideoStreamManager

  - fault (FoV) free reception  of 'ImageMetaData' in 'analyseImageMetaData' function shall be ensured

  - fault (FoV) free 'analyseImageMetaData' (function) shall be ensured

  - fault (FoV) free reception of 'VideoStreamConfiguration' in 'configureStream' function shall be ensured

    - external interface SOW2 –Vehicle/User

  - fault (FoV) free function 'configureStream' shall be ensured

  - fault (FoV) free function 'provideConfiguration' shall be ensured

  - fault (FoV) free reception of 'FS-DiagnosticsInformation' in 'receiveFS-DiagnosticsInformation' function shall be ensured

    - external interface SOW2 –SOW1

  - fault (FoV) free function 'receiveFS-DiagnosticsInformation' shall be ensured

  - fault (FoV) free reception of 'VideoConfiguration' in 'storeVideoStreamConfiguration' function shall be ensured

  - fault (FoV) free function 'storeVideoStreamConfiguration' shall be ensured

- FunctionalSafetyManger:

  - fault (FoV) free reception of 'FunctionalSafetyDiagnostics' in 'acquireDiagnostics' function shall be ensured

  - fault (FoV) free function 'acquireDiagnostics' shall be ensured