# Technical safety regarding DDR memory

## References

These documents can be found in the Polarion repository browser

https://erg-polarion-p1.mekra.net/polarion/svnwebclient/directoryContent.jsp?url=Intern%2F11_P_01467_MEKRA_Vision_Controller%2FProject%20Files%2F02%20Development

| [1] | HW-SM-002_0.8_Ambarella CV2FS CV22FS HW Safety Manual.pdf |
| --- | --- |
| [2] | Datasheet ISSI LPDDR4 8Gbit (16x512) (ECC-Ram) 43-46LQ16512B.pdf |
| [3] | Datasheet ISSI LPDDR4 16Gbit (32x512) (Data-Ram) 43-46LQ32512A.pdf |
| [4] | Safety Manual PMIC MPQ70165FS |
| [5] | MPQ70165FS-xxxx-AEC1 - Published Datasheet (MPS Rev#{3} 0.8) (002).pdf |

## TSRs regarding DDR memory

For SOW2, the system uses 2 LPDDR4 memories:

- ISSI LPDDR4 8Gbit (16x512) (ECC-Ram) [2]

- ISSI LPDDR4 16Gbit (32x512) (Data-Ram) [3]

In an integrated circuit, sources of faults come from a variety of sources: electro-magnetic interference (EMI), radiation, electro migration, shocks, vibrations, and more. In some cases, it is important to know the specific sources so targeted measures can be taken. When this is the case, it is usually reasonable to abstract them to bit flips (transients) and stuck-at faults (permanents). Importantly, this abstraction is allowed by ISO 26262.

The used LPDDR4 components do not provide any special functional safety precautions.

Faults in the DDR memory violate safety goals by loss or corruption of Video Data, Metadata, configuration data or any other information stored.

To ensure the functional safety, the DDR memory shall be secured by using external measures and functions to reveal permanent and transient faults.

The main functional safety feature here is, to store the Hamming ECC for the given data into a second DRAM. During readout, the ECC for the read-data is calculated and compared against the stored ECC. Due to the nature of the ECC (Hamming 298,16), single error correction is possible, and multiple errors can be detected.

### The Power Supply for the DDR shall be monitored

Voltages for the DDR-RAMs are provided and controlled by PMIC MPQ70165FSGUTE-0021-AEC1-Z.

The recommended Voltage for ECC-RAM [2]:

| Voltage | Min | Max |
| --- | --- | --- |
| VDD1 | 1.70 V | 1.95 V |
| VDD2 | 1.06 V | 1.17 V |
| VDDq | 1.06 V | 1.17 V |

Table 1: Recommended Voltages for ECC-RAM

The recommended Voltage for Data-RAM [3]:

| Voltage | Min | Max |
| --- | --- | --- |
| VDD1 | 1.70 V | 1.95 V |
| VDD2 | 1.06 V | 1.17 V |
| VDDq | 1.06 V | 1.17 V |

Table 2: Recommended Voltages for Data-RAM

Ownership: System Integrator

**Note:**

This requirement is checked by the PMIC via PG and ALERT# signaling. In case of failure, the PMIC sets PG and ALERT# to low, which results in a reset of the Microcontroller.

For the min and max conditions to be supervised, the PMIC needs to be configured for under- and over-voltage monitoring [4], SM_8, SM9, with the correct PG_CONFIG (0xCA), bit[0] set according to the recommended voltage ranges:

Buck 3, PG_CONFIG (0xCA), bit[0] set to 1

Reference:

- [4]

  - 5.3.8 PG Over-Voltage (OV) Monitoring (SM_8)

  - 5.3.9 PG Under-Voltage (UV) Monitoring (SM_9)

- TSR PMIC

### [AR-14] Initialization is necessary before safe access to DDR if it’s possible to read from uninitialized DDR address

Ref: [1], chapter 5.3.14

Description

This AoU is to avoid false alarm from ECC Decoder. All data stored in external DDR device are assumed to be protected by ECC. If any DRAM client reads an uninitialized DDR address, the returned read data will be unknown. A DRAM BIST and Scrubbing Engine (DBSE) is implemented in the SEooC. Formatting DDR can be achieved by setting DBSE_MODE to 0 and DBSE_BIST_PATTERN to 0x1 in DBSE module.

ASIL C

Note: It has to be clarified how this works with the boot process. Registers for DBSE should be set already during boot, when there is no data written to DRAM yet.
Maybe this is a task for Boot-ROM (BST) code.

This has to be done before enabling ECC (SM_HW_HECC).

It is recommended to verify this on Hardware and measure the time for scrubbing the DRAM.

### The DRAM subsystem Hardware Watchdog Timer on DRAM for the CV2FS shall be used

Reference: [1], chapter 5.1.22

| Name | Description |
| --- | --- |
| ID | SM_HW_DRAM_WDT |
| Description | Several hardware watchdog timers are implemented to detect hang of specific functional blocks in the DRAM sub-system. System Integrator is responsible for implementing the safety software to configure the related settings. |
| Ownership | Ambarella and System Integrator |
| SM is to against | Hang of specific functional blocks in DRAM sub-system. |
| Software Related | Yes |
| Failure mode coverage | 99% |
| Initialization | Program WATCH DOG_TIMER_0_CONFIG and WATCHDOG_TIMER_1_CONFIG registers with number of cycles to expire/16 |
| Operation | N/A |
| Error indication | Assert safety error signal |
| Error detect time | 3 ms |
| Measure to prevent latent fault | SM_HW_LBIST (if applicable) |
| HW-ASIL stated by Ambarella | ASILC |
| Required ASIL | ASIL-B |

### The DRAM data shall be secured with a Hamming Error correction code

Reference: [1], chapter 5.1.23

- uncorrectable Errors -> safety error

- correctable errors -> counter increment

  - check counter, see chapter 5.1.28 (below)

| Name | Description |
| --- | --- |
| ID | SM_HW_HECC |
| Description | - SM_HW_HECC is implemented with Hamming (298, 16). - Each codeword(32B) is protected by a single error correction and dual error detection (SECDED) - Each codeword consists of a 32B message, 26-bit physical address, and 16-bit parities. - The message part is stored in main DDR device and the parity part is stored in ECC-DDR device. System Integrator is responsible for implementing the software to enable the hardware ECC function in start-up flow. |
| Ownership | Ambarella and System Integrator |
| SM is to against | Any random hardware fault of external DDR device, and data path from ECC engine to the interfaces with DDR devices. |
| Software Related | Yes |
| Failure mode coverage | 100% - for single-bit error 100% - for double-bit error 98% - for multiple-bit error with less than 2% false correction probability |
| Initialization | Software shall enable ECC enabled bit in REG_ECCH_MODE. |
| Operation | N/A |
| Error indication | Assert safety error signal on uncorrectable error detection, increment MAIN_CORRECTABLE_COUNTER or ECC_CORRECTABLE_COUNTER on a single bit error correction |
| Error detect time | 3 ms |
| Measure to prevent latent fault | SM_HW_LBIST |
| HW-ASIL stated by Ambarella | ASIL-C |
| Required ASIL | ASIL-B |

For handling of the counters for correctable errors, see chapter "DRAM ECC Corrected-Error Counter" below.

Note: It has to be clarified how this works with the boot process. Registers for REG_ECCH_MODE should be set already during boot, when there is no data written to DRAM yet. Maybe this is a task for Boot-ROM (BST) code.

### DRAM ECC Corrected-Error Counter

Reference [1], chapter 5.1.28

Note: The threshold mentioned in the safety mechanism below is for correctable errors only. Uncorrectable errors will immediately end up in safe state.

- correctable error -> error is corrected, still safe

- uncorrectable error -> safety error

For this reason SM_HW_DRAM_ECC_CORR_CNT is not used.

| Name | Description |
| --- | --- |
| ID | SM_HW_DRAM_ECC_CORR_CNT |
| Description | - In the SEooC, external DRAM is SECDED ECC protected. - The ECC has: - a counter keeping track of how many single bit errors have been corrected. For every cycle, this counter only counts up to one corrected error; - a configurable threshold indicating at what count the monitor logic will report an error. That is, if the corrected count exceeds the threshold, an error will be reported to CEHU. This counter is 16-bits. The threshold's default value is its maximum value (all 1s). SW can read current corrected error count via debug port. - a mechanism that would prevent an error from being sent to CEHU for corrected single-bit errors, regardless of the count. This is achieved by a disable / mask bit. System Integrator is responsible for implementing the safety software to enable the DRAM ECC function and setup the proper threshold for the safety application usage. |
| Ownership | Ambarella and System Integrator |
| SM is to against | single or dual bit(s) permanent fault in memory data array |
| Software Related | Yes |
| Failure mode coverage | 99% |
| Initialization | Software shall set the threshold on ERROR_STATUS_REG_2/3_Threshold, which 2 for Main-DDR and 3 for ECC-DDR |
| Operation | N/A |
| Error indication | Assert safety error signal on exceeding the threshold |
| Error detect time | 99 ms |
| Measure to prevent latent fault | N/A (this safety mechanism is to detect latent fault) |
| HW-ASIL stated by Ambarella | ASIL-C |
| Required ASIL | ASIL-B |

### DRAM Segmentation shall be used

Reference [1], chapter 5.1.25

Note: Frage: gehört das wirklich hierher?

- Eigentlich werden dadurch SW Fehler abgefangen

- ich denke schon, weil hier alle Sicherheitsmechanismen bzgl. DDR behandelt werden

| Name | Description |
| --- | --- |
| ID | SM_HW_DRAM_SEGMENTATION |
| Description | SM_HW_DRAM_SEGMENTATION is implemented to detect a DRAM client to access illegal DRAM region and a specific client to access secure region. System Integrator is responsible for analyzing the DRAM region/clients configuration and implementing the software to configure the DRAM segmentations settings to avoid interference from QM clients. |
| Ownership | Ambarella and System integrator |
| SM is to against | Illegal activities from QM clients |
| Software Related | Yes |
| Failure mode coverage | 99% |
| Initialization | The unit of segment is 256 KB. Software shall ensure a minimum 256 KB non-overlapped memory region allocated for each DRAM client. |
| Operation | N/A |
| Error indication | Assert corresponding bit in REG_DRAM_SEG_FAULT and an interrupt. Software determines to go to safe state if needed. |
| Error detect time | 99 ms |
| Measure to prevent latent fault | SM_HW_LBIST |
| HW-ASIL stated by Ambarella | ASIL-C |
| Required ASIL | ASIL-B |

Note: It has to be clarified how this works with the boot process. Registers to configure the DRAM segmentations should be set already during boot, when there is no data written to DRAM yet. Maybe this is a task for Boot-ROM (BST) code.

### DRAM Bandwidth Monitoring shall be used

Reference: [1], chapter 5.1.26

| Name | Description |
| --- | --- |
| ID | SM_HW_DRAM_BW_MON |
| Description | The statistic counters are implemented to monitor the traffic of DRAM clients. System Integrator is responsible for implementing the safety software to enable and read out the DRAM statistic counter for the safety application usage. |
| Ownership | Ambarella and System Integrator |
| SM is to against | Illegal activities from QM clients |
| Software Related | Yes |
| Failure mode coverage | 90% |
| Initialization | Software shall enable STAT_CONTROL bit. |
| Operation | Software shall disable STAT_CONTROL bit, check the statistic counters, and then re-enable STAT_CONTROL bit every 99 ms. |
| Error indication | Assert safety error signal by software |
| Error detect time | 99 ms |
| Measure to prevent latent fault | SM_HW_LBIST |
| HW-ASIL stated by Ambarella | ASIL-C |
| Required ASIL | ASIL-B |

Note: In case of, a lack of bus scheduling / prioritization: 
To ensure, that R52 can at least trigger the Safe State, the relevant code should be allocated to R52 TCM. Otherwise, there could be a risk that excessive use of the data bus by A53 prevents R52 from loading its time critical code.

In case of bus prioritization safety / time critical code shall have respective priority.

### Address Stuck Test shall be run at least every DTTI

Reference [1], chapter 5.1.27

| Name | Description |
| --- | --- |
| ID | SM_HW_ASTUCK_TEST |
| Description | - Address stuck test is to periodically measure any stuck-at faults on DDRIO pins, including DDR-IO command, address and data interfaces, when using LPDDR4 DRAM memory. SM_HW_ASTUCK_TEST will write the pre-defined pattern bits to Mode Register 15 and 20 on an external DDR device, read out the Multi-Purpose-Command Read data from it and compare the read data with the expected patterns. Any mismatch on this read data will assert safety error. - The SM_HW_ASTUCK_TEST shall be run at least once every DTTI. - System Integrator is responsible for implementing the safety software to configure the ASTUCK engine test to meet the system safety requirement. |
| Ownership | Ambarella and System Integrator |
| SM is to against | Stuck-at fault on DDR-IO command, address, and data interfaces |
| Software Related | Yes |
| Failure mode coverage | 90% |
| Initialization | At DDRC, DRAM_TYPE has to be 6 (LPDDR4). At DTTE, Configure DTTE_DUAL_DIE, DTTE_DUAL_CHANNEL, DTTE_MR15, DTTE_MR20, DTTE_EXPECTED_PATTERN, DTTE_ASTUCK_PERIOD and DTTE_ASUTCK_PERIOD_MSB, and at last set DTTE_ASTUCK_TEST_EN to 1 to enable SM_HW_ASTUCK_TEST. For more information about configuration, please refer to the DDRC/DTTE Programming Reference Manual. |
| Operation | N/A |
| Error indication | HW asserts safety error signal and sets DTTE_ERR_VLD and DTTE_ASTUCK_ERR_VLD in DTTE Software Programmable Registers. |
| Error detect time | 99 ms |
| Measure to prevent latent fault | None |
| HW-ASIL stated by Ambarella | ASIL-C |
| Required ASIL | ASIL-B |