# TSR on PHY

References:

| [1] | DS754510 TJA1104 DataSheet, NXP, Rev. 1.0 - 17 April 2024 |
| --- | --- |
| [2] | UM12054 TJA1104 safety manual, NXP, Rev. 1 - 28. March 2024 |
| [3] | DS881310 TJA1121 Datasheet, NXP, Rev. 1.0 - 3. December 2024 |
| [4] | UM12165 TJA1121 safety manual, NXP, Rev. 1.0 - 26. November 2024 |
| [5] | Safety Manual PMIC MPQ70165FS |

Table 1: References

Abbreviations:

| FSA | Functional Safety Assumption |
| --- | --- |
| SM | Safety Mechanism |
| WM | Warning Mechanism |

Table 2: Abbreviations

Depending on the Hardware, the used PHYs are

- NXP TJA1104AHN for 100BASE-T1 physical Layer

- NXP TJA1121AHN for 1000BASE-T1 physical layer

For a functional safe operation of the PHY, the datasheets [1], [3] and safety manuals [2], [4] of the device must be taken into account.

## BIST

- automatically at startup

- device and port-level self-test

When both the device and the port self-test checks are completed and pass, the DEVICE_STATUS[FUSA_STATUS] bit is set and the FUSA_PASS_IRQ is triggered automatically. In case a check has not passed successfully, the issue is reported via the related status register.

TSR

- After startup, the SoC shall wait for the FUSA_PASS_IRQ to be set

- Alternatively, the Soc polls the safety status bit

- The SoC shall use the interrupt or poll the ALWAYS_ACCESSIBLE register for failures

- The SoC shall check for Errors regarding SM_3, WM_1, WM_2 or WM_3

  - Refer to safety manuals [2], [4]

Note:

- SM_3: VDD_CORE failure

- WM_1: VDDA_TRX failure warning

- WM_2: VREGA_IN failure warning

- WM_3: VREGD_IN failure warning

Reference: [2], [4] Safety Manual

- SM_10

  - Relates to FSA_37, FSA_38, FSA_74, FSA_75, FSA_76

- SM_5

  - Relates to FSA_14, FSA_15

- SM_3

  - Relates to FSA_46, FSA_47, FSA_48, FSA_49, FSA_50,

## Configuration of the device

The Driver provides protection against unintended changes by reading back and verifying the configuration

TSR

- The Software Driver shall read back and verify every configuration write.

- The Software Driver shall provide frame failure safety mechanism according to [2], [4] SM_7

- After configuration, CONFIG_ENABLE shall be reset, to protect configuration from unintended changes

Reference [2], [4] Safety Manual

chapter 4.3.4.1.11

- SM_7, SM_9

  - Relates to FSA_26, FSA_30, FSA_31, FSA_32, FSA_36, FSA_44, FSA_68, FSA_69, FSA_70, FSA_72

## SG1 related TSRs

The system shall avert a delay of the Video Stream (between its input and output) to more than 50ms while streaming is ‘active’ (i.e. output: on)

- Timestamping is not necessary for SG1, but mandatory to be send to vehicle

- for measuring the time of delay, no global clock is necessary, only a timer.

TSR

The Software Driver shall implement Timestamping integrity safety mechanism according to [2], [4] SM_9

Reference [2], [4] Safety Manual

- SM_9

  - Relates to FSA_20, FSA_36, FSA_72, FSA_73

Reference [1], [3] Datasheet

TJA1104 [1], chapter 6.12 Timestamping

TJA1121 [3], chapter 6.11 Time stamping

## SG2 related TSRs

The system shall avert a frozen image.

Remark 1: The image is considered frozen, when given frame gets repeated.

Remark 2: Due to technical implementation, one (single!) repetition of the frame may be allowed.

TSR

- no TSR apply to the PHY regarding SG2

- MACsec does not include buffers for complete frames.

  - Reference [1] chapter 6.13.1 Introduction page 55.

  - Reference [3] chapter 6.12.1 Introduction page 48.

## SG3 related TSRs

The system shall display the environment realistically at any time regarding contrast and brightness, to avoid losing any detail, which could end up in a hazard.

Video frames causing an unclear view (too dim, too low contrast, too much disturbances or noise, wrong resolution, flicker, too low frame rate) shall not be transmitted.

TSR

- no TSR apply to the PHY regarding SG3

## SG4 related TSRs

The system has to show correct angle of view, zoom factor and visual field at any time

TSR

- no TSR apply to the PHY regarding SG4

## Informational Part

### [SM_1] VDDA_AO failure safety mechanism

Safety mechanism SM_1 is assumed to be implemented externally to TJA1121. It must provide:

- Under- and overvoltage detection of VDDA_AO, the always-on supply of the TJA1121.

SM_1 must be available continuously, because several safety functions need VDDA_AO as their independent supply.

If VDDA_AO is outside its operating range, the system is assumed to assert and hold TJA1121 in reset (RST_N = LOW). The system is assumed to move to a system safe state to handle the communication loss. A supply overvoltage will damage the device or at least reduce its operating lifetime.

Related functional safety assumptions: [FSA_09].

Note: This safety mechanism must be checked by the system, because VDDA_AO is not secured by the PMIC.
VDDA_AO = [3.135V – 3,465V], refer [1], [3], chapter 10 Static characteristics

### [SM_2] VDDIO failure safety mechanism

Safety mechanism SM_2 is assumed to be implemented externally to the TJA1121. It must provide:

- Under- and overvoltage detection of VDDIO, the IO supply shared by the microcontroller, MAC and TJA1121.

SM_2 must be available continuously, because several safety functions make use of VDDIO as their IO supply.

If VDDIO is outside its operating range, the system is assumed to assert and hold TJA1121 in reset (RST_N= LOW). The system is assumed to move to a system safe state to handle the communication loss. A supply overvoltage will damage the device or at least reduce its operating lifetime.

Related functional safety assumptions: [FSA_10].

Note: This safety mechanism is fulfilled externally. VDDIO (+1.8V) is under surveillance of PMIC MPQ70165FS.

VDDIO = [1,65V – 1,95V], refer [1], [3], chapter 10 Static characteristics

For the min and max conditions to be supervised, the PMIC needs to be configured for under- and over-voltage monitoring [4], SM_8, SM9, with the correct PG_CONFIG (0xCA), bit[0] set according to the recommended voltage ranges:

Buck 5, PG_CONFIG (0xCA), bit[0] set to 1
(setting to 0 would be ok, but Deserializer has more tight recommended operating conditions for VDD18, so we need to set it to 1)

Reference:

- [2], [4] SM_2,

- [5]

  - 5.3.8 PG Over-Voltage (OV) Monitoring (SM_8)

  - 5.3.9 PG Under-Voltage (UV) Monitoring (SM_9)

- TSR PMIC

### [SM_3] VDD_CORE failure safety mechanism

Safety mechanism SM_3 is an internal safety measure. It detects and reports VDD_CORE power supply failures. It provides:

- An under- and overvoltage detector [SM_3]

- An independent voltage reference [SM_3]

SM_3 is available continuously when the TJA1121 is powered-up. If a VDD_CORE supply failure is detected:

- CORE_SUPPLY_WARN interrupt in the OP_CONDITIONS_IRQ_SOURCES register (90h) is set.

- INT_N is asserted LOW.

- CORE_SUPPLY_WARN in the ALWAYS_ACCESSIBLE register (801Fh) is set.

- OV_EVENT or UV_EVENT in CORE_SUPPLY_STATUS register (313h) is set.

The system shall use the interrupt or poll the ALWAYS_ACCESSIBLE register to react on the failure. If SM_3 reports a VDD_CORE failure, the system is assumed to assert and hold the TJA1121 in reset (RST_N = LOW). The system is assumed to move to a system safe state to handle the communication loss. In the case of an overvoltage, the supply should be shut off to prevent damage to the TJA1121.

Related functional safety assumptions: [FSA_26, FSA_46, FSA_47].

Note: This safety mechanism is fulfilled internally.

It can be fulfilled externally, too. VDD_core (+1.1V) is under surveillance of PMIC MPQ70165FS. For the min and max conditions to be supervised, the PMIC needs to be configured for under- and over-voltage monitoring [5], SM_8, SM9, with the correct PG_CONFIG (0xCA), bit[0] set according to the recommended voltage ranges.

### [SM_4] Clock failure safety mechanism

Safety mechanism SM_4 is an internal safety measure. It detects and reports core clock failures. It provides:

- A clock failure detector [SM_4]

- An independent frequency reference [SM_4]

SM_4 is available continuously when the TJA1121 is powered-up. If a clock failure is detected:

- OSC_WARN interrupt in the OP_CONDITIONS_IRQ_SOURCES register (90h) is set.

- INT_N is asserted LOW if enabled via OP_CONDITIONS_IRQ_ENABLES register (91h)

- REFCLK_WARN in the ALWAYS_ACCESSIBLE register (801Fh) is set.

- EPHY_CLK_WARN, SGMII_CLK_WARN or TXC_CLK_WARN in the PORT_LEVEL_IRQ_SOURCES register. (8070h) is set to indicate the source of the failure.

The system shall use the interrupt or poll the ALWAYS_ACCESSIBLE register (801Fh) to react to the failure. If SM_4 reports a clock failure, the system is assumed to assert and hold TJA1121 in reset (RST_N = LOW).

Related functional safety assumptions: [FSA_27, FSA_28, FSA_51, FSA_52].

Note: This safety mechanism must be checked by the system.

### [SM_5] 100BASE-T1 link failure safety mechanism

Note: For 1000BASE-T1 refer to [4], chapter 4.3.4.1.5

Safety mechanism SM_5 is an internal safety measure. It detects and reports 100BASE-T1 link failures. It provides:

- An internal link failure detection mechanism [SM_5]

SM_5 is available continuously when TJA1104 is powered-up.

In order to meet a 5 ms detection time (based on the assumed FHTI of 10 ms), assumption FSA_14 must be fulfilled by the link partner. If the indication of loc_rcvr_status = NOK by the link partner takes longer, the detection time of a failing link will be extended as well. If the FHTI of the system is longer, this can be accepted.

SM_5 indicates changes to the 100BASE-T1 link status. The link must be up for running communication, indicated by LINK_AVAILABLE = 1 in the PHY_STATUS register (8102h).

In the case of a link failure:

- LINK_AVAILABLE is set to 0

- LINK_AVAILABLE_EVENT in EPHY_FUNCTIONAL_IRQ_SOURCES (80A0h) is set

- INT_N is asserted LOW if enabled via EPHY_FUNCTIONAL_IRQ_ENABLES register (80A1h)

The system shall use the interrupt or poll the PHY_STATUS register to react to the failure. If SM_5 reports a 100BASE-T1 link failure, the system is assumed to move to a system safe state to handle the communication loss. The system may wait until the status changes and the link becomes available again, or remain in a safe state.

Related functional safety assumptions: [FSA_14, FSA_15].

### [SM_7] Frame failure safety mechanism

Safety mechanism SM_7 is assumed to be implemented externally to TJA1104. It must provide:

- Detection of data path errors

SM_7 can be realized in different ways. It may use dedicated test frames with known content after startup to test the communication path in both directions. It may be covered by a higher layer protocol to detect data path errors or 'heart-beat' frames on the application layer. If an end-2-end protection is implemented on the application layer, related frames can be considered as continuous test frames.

Related functional safety assumptions: [FSA_18, FSA_28, FSA_29, FSA_30].

### [SM_8] Frame integrity failure safety mechanism

Safety mechanism SM_8 is assumed to be implemented externally to TJA1104. It must provide:

- Detection of corrupted frames.

There are several failures that can occur to an Ethernet frame.

100BASE-T1 communication has an intrinsic bit error rate of < 10E-104.

1000BASE-T1 communication has an intrinsic bit error rate of < 10E-10

Additionally, RF disturbances may cause bit flips. Typically, those failures are detected by the receiving MAC, which drops the frame.

The system must be able to handle those missing frames and add additional failure protection on the application layer if the CRC check in the MAC is not sufficient for the application.

Related functional safety assumptions: [FSA_19, FSA_31, FSA_32].

### [SM_9] Timestamping integrity safety mechanism

Safety mechanism SM_9 is assumed to be implemented externally to TJA1104 if the application uses timestamping as a safety relevant function. It must provide:

- Detection of faulty time information from the TJA1104 timestamps.

Timestamp information may become corrupted. The system is, therefore, required to detect and handle incidental timestamp errors and shall safeguard against drastic permanent timestamp changes, such as an extreme increase or decrease in time or in the time constant, or a non-linear increase in time.

TJA1104 enhances the diagnostic coverage of SM_8 for PTP frame integrity failures by means of SM_9. When timestamp information is added to PTP frames, the TJA1104 must apply a new FCS to the frame. If the original PTP frame was detected to have a failing FCS, it will be propagated with a (new) failing FCS, enabling SM_8 to detect the frame as corrupt in the receiving MAC.

Related functional safety assumptions: [FSA_20].

### [SM_10] Safe startup sequence safety mechanism

Safety mechanism SM_10 sequences the latent fault checks of internal safety mechanisms SM_3 and SM_5 (partly), as well as warning mechanisms WM_1, WM_2 and WM_3.

SM_10 is triggered automatically after a power up, release of RST_N or a wake-up from DEVICE_DEEP_SLEEP state. Upon successful completion of the startup latent fault checks, TJA1104 will:

- Set FUSA_STATUS = 1 in the DEVICE_STATUS register (42h)

- Set FUSA_PASS_IRQ in the ALWAYS_ACCESSIBLE register (801Fh)

- Assert INT_N LOW

The system shall use the interrupt or poll the ALWAYS_ACCESSIBLE register to wait for a successful startup before using the link for safety critical communication.

For SM_5, the latent fault test finishes by checking for stuck-LOW failures by ensuring LINK_AVAILABLE (SM_5) is HIGH when a link is available.

In case one or more of the latent fault checks fails:

- FUSA_STATUS will remain at 0

- FUSA_PASS_IRQ will remain at 0

- The related xx_LATENT_FAULT bit will be set

The system cannot use the related safety measure when the latent fault test fails.

### Delay of TJA1104

- [2] chapter 6.12 Timestamping

- [2] Chapter 6.13.10.1 TX Latency, Table 26

- [2] Chapter 6.13.10.2 RX Latency, Table 27

- clock cycle assumed to be either 25MHz or RGMII_TXC

- MACsec does not include buffers for complete frames. [2] chapter 6.13.1 Introduction page 55.