# TSR on PMIC MPQ70165FS

References:

| [1] | Datasheet MPQ70165FS Rev. 0.8 MonolithicPower.com 20.02.2025 |
| --- | --- |
| [2] | MPQ70165FS Safety Manual Rev. 0.3 MonolithicPower.com 07.05.2024 |
| [3] | Datasheet MPQ4572 Rev. 1.0, 1/21/2020 |
| [4] | UM12054 TJA1104 safety manual, NXP, Rev. 1 - 28. March 2024 |
| [5] | UM12165 TJA1121 safety manual, NXP, Rev. 1.0 - 26. November 2024 |

Table 1: References

## Assumptions

The MPQ70165FS is developed as a safety element out of context (SEooC) following the guidelines for SEooC hardware component development according to ISO26262-10, Clause 9. The according assumptions for system-level technical safety requirements are specified and described in this section.

### Assumed Safety Goals

- Rail(s) over-voltage (OV) failure

- Rail(s) under-voltage (UV) failure

### Automotive Safety Integrity Level (ASIL)

- ASIL B

### Assumed Safe State

- Output rails shut off

### PMIC System TSRs

| Metric | Reference |
| --- | --- |
| FTTI: 5 ms | [2] System_TSR_1 and _2 |
| FIT: 10 | [2] System_TSR_3 |
| PMHF: >= 90% | [2] System_TSR_3 |
| SPFM >= 90% | [2] System_TSR_4 |
| LFM => 60% | [2] System_TSR_5 |

Table 2: System TSR Metrics

- The safety mechanisms shall be tested either at MPQ70165FS start-up or shutdown.

- It is recommended to run the test(s) at start-up.

- One power cycle shall be performed every 72 hours or sooner.

## TSRs regarding PMIC MPQ70165FS

### 1. The Operating conditions are not exceeded

Recommended Operating Conditions

| Supply Voltage Vin | 2.7V to 5.5V |
| --- | --- |
| Buck1/2/5/6 Output Voltage | 0.20625V to 0.4 x Vin |
| Buck3/4 Output Voltage | 0.20625V to 3.6V |
| Operating Junction Temp. (Tj) | -40°C to +150°C |

Table 3: Recommended Operating Conditions

Reference: [1], page 7 “Recommended Operating Conditions

If the operating conditions are exceeded, the MPQ70165FS safety mechanisms cannot be guaranteed.

Note:

According to schematic:

- Vin = 5V

- Output Voltage 0.75V .. 1.8V

### 2. The system shall transition into Safe State if Vin is out of range

Reference [2], chapter 5.2.4

| ID | SM_16 |
| --- | --- |
| Description | The system shall monitor the Vin supply voltage (see Section 4.5.1 on page 12). |
| Error Type | MPQ70165FS power supply out of range failure (single-point fault/latent fault). |
| Error Signaling | The system shall define the proper state / reaction at the system level in case of Vin failure. The defined safe state / reaction shall be independent to Vin power source to avoid common cause failure. |
| Fault-Handling Time Interval | To be defined by the system integrator. |
| Self-Test | N-A |

Table 4: External Vin Monitoring

Note:

This requirement is fulfilled by the signal Power Good (PG) of MPQ4572GQBE (generates Vin). This signal is used as PMIC enable.

PG MPQ4572GQBE is driven high, if output voltage is within the given range after 70μs:

| 90% * Vout< Vout < 108% * Vout | (4.5V .. 5.4V) |
| --- | --- |

PG MPQ4572GQBE is driven low, if output voltage is outside the given range after 25μs:

| 84% * Vout < Vout < 116% * Vout | (4.2V .. 5.8V) |
| --- | --- |

Reference: [3], page 22, Power Good (PG) Indicator

Although 5.8V is outside the recommended Supply voltage for the PMIC, but it is still within its absolute maximum rating.

### 3. The Question and Answer Watchdog function shall be used

The MPQ70165FS uses a watchdog function to monitor the integrity of the microcontroller (MCU). If the MCU experiences an internal failure, MPQ70170FS shall detect those failures via the watchdog interface

- The Watchdog is configured in Q-A Mode

  - see MVC-95649 - PMIC- configuration of error input

  - PMIC generates token, uC answers with calculated 32 bit CRC within the watchdog window

### 4. At startup of MPQ70165FS, the device safety path shall be tested

#### 4.1 ALERT# Test

Reference [2], chapter 4.5.3.1, chapter 5.2.2

The safety path from MPQ70165FS’ point of view shall be the connection between ALERT# and system. The safety path shall be tested every drive cycle to ensure that the system can detect the correct state of ALERT#.

The system shall set the dedicated ALERT# register (ALERT_PG_FORCE (0xC0) bit 1) to pull it low at start-up. In case the ALERT# is not being properly pulled low, the system shall maintain at its safe state and no normal operation is allowed.

| ID | SM_19 |
| --- | --- |
| Description | The system shall toggle the ALERT# at power up to ensure the ALERT# function is working properly in case of relevant failures. |
| Error Type | ALERT# stuck at failure (latent fault). |
| Error Signaling | The system shall remain in its safe state. Normal operation is not allowed. |
| Fault-Handling Time Interval | One power cycle. |
| Self-Test | N/A |

Table 5: ALERT# Power-Up Test

#### 4.2 PG# Start-up Test

Reference [2], chapter 4.5.3.2, chapter 5.2.1

The safety path from MPQ70165FS’ point of view shall be the connection between PG# and system. The safety path shall be tested every drive cycle to ensure that the system can detect the correct state of PG#.

Therefore, system shall set the dedicated PG# register (ALERT_PG_FORCE (0xC0) bit 0) to pull PG# low at start-up.

In case the PG# is not being properly pulled low, the system shall maintain at its safe state, and no normal operation is allowed.

| ID | SM_18 |
| --- | --- |
| Description | The system shall toggle the PG# at power up to ensure the PG# function is working properly in case of power good failures. |
| Error Type | PG# stuck at failure (latent fault). |
| Error Signaling | The system shall remain in its safe state. Normal operation is not allowed. |
| Fault-Handling Time Interval | One power cycle. |
| Self-Test | N/A |

Table 6: PG# Power-Up Test

#### 5. Assumption of I2C Communication

Reference [2], chapter 4.5.5, chapter 5.3.1

The I2C message shall contain a CRC-8, which is represented by the polynomial 
C(x) = x8 + x2 + x + 1.

| ID | SM_1 |
| --- | --- |
| Description | A CRC check shall be implemented to detect at least 2 bits of failure in I2C communication. The reaction shall be triggered when a 1-bit error occurs. - Number of bits to check: 32 bits - Polynomial: 0x83 - Hamming Distance: 3 |
| Error Type | I2C communication failure (Single-point fault/Latent fault) |
| Error Signaling | - Pull ALERT# low (can be configured via ALERT_MAPPING (0xC4), bit [2]) (ASIL B) - Set the error register STATUS_CML (0x7E), bit [5] (QM) - Pull PG low (after a certain delay time, PG goes back to high, as configured via PG_MAPPING (0xC6), bit [2]; the PG delay time can be set via PG_ALERT_DELAY, bits [1:0]) (ASIL B) |
| Fault-Handling Time Interval | 50μs |
| Assumptions of Use (AoU) | The system shall use the same CRC polynomial. In case of I2C communication failure, the system may not be able to read the proper error register. However, the ALERT# and PG# still triggered. As a result, the system shall define the proper reaction for the error that only indicated by the ALERT# and PG# when the type of error is uncertain. |
| Self-Test | LBIST(SM_12) |

Table 7: I2C CRC Communication Check

#### 6. Assumption of System Read Back

A register write check shall be implemented by system to detect if written data is the same as the command.

Reference [2], chapter 4.5.6, chapter 5.2.3

| ID | SM_20 |
| --- | --- |
| Description | The SoC or microcontroller (MCU) shall read back the written value from the register and check this value against the commanded value. |
| Error Type | An incorrect register value has been written (single-point fault/latent fault) |
| Error Signaling | The system shall define a proper fault reaction. |
| Fault-Handling Time Interval | To be defined by the system within the FTTI. |
| Self-Test | N/A |

Table 8: Write Message Readback

In case of Write Message Readback failure, the SoC shall try to rewrite the command. In case of failing re-write, the System shall go into Safe State.

#### 7. Internal Safety Mechanisms

During start-up, several tests are performed by the device itself.

Failing build-in self-tests result in Deep Safe State and information in registers on the failing cause.

Reference [2], chapter 5.3

covered by LBIST:

- SM_1 I2C CRC Communication Check

- SM_2 Digital Clock Monitoring

- SM_3 OTP CRC

- SM_4 Register CRC Check

- SM_5 Register Write Check

- SM_22 State Machine Error Detection

covered by ABIST:

- SM_8 PG Over-Voltage (OV) Monitoring

- SM_9 PG Under-Voltage (UV) Monitoring

- SM_10 Reference Voltage Cross Check

- SM_15 Auxiliary Voltage Monitor

#### 7.1 Analog Built-In Self-Test (ABIST)

This test is automatically run at start-up

Reference [2], chapter 5.3.11

| ID | SM_11 |
| --- | --- |
| Description | An analog built-in self-test (ABIST) shall be performed during start-up. The defined test values check the functionality of the comparators as shown below. Fault injection test shall be performed on the following comparators during power up: - DAC reference monitoring comparators 1–12 - MON+/MON- monitoring comparators 1–2 - PG voltage comparators 1–6 |
| Error Type | Analog-related safety mechanism failures (latent fault). |
| Error Signaling | - Stuck in deep safe state during power-up (output rails by default low), and re-run ABIST until ABIST passes (Enter deep safe state via register PG_MAPPING 0xC6 bit 0) - Set error register: STATUS_ MFR_SPECIFIC 0x80 bit 1 (QM) - Pull low ALERT# pin via register ALERT_MAPPING 0xC4 bit 0 (ASIL B) - PG is pulled low (by default) (ASIL B) |
| Fault-Handling Time Interval | One power cycle |
| Assumptions of Use (AoU) | In case of an ABIST failure, the system shall remain in the defined safe state. Normal operation shall not be allowed. |
| Self-Test | N/A |

Table 9: Analog Built-In Self-Test (ABIST)

#### 7.2 Logic Built-In Self-Test (LBIST)

Reference [2], chapter 5.3.12

This test is automatically run at start-up

| ID | SM_12 |
| --- | --- |
| Description | Testing all digital logic during start-up. When LBIST fails, MPQ70165FS enters the DEEP SAFE state. The diagnostic coverage shall be greater than 80%. |
| Error Type | Digital logic gate failures (latent fault). |
| Error Signaling | - Output rails stuck at low. POR fails to proceed. - Set the error register STATUS_MFR_SPECIFIC (0x80), bit [3] (QM) - PG is pulled low (by default) (ASIL B) |
| Fault-Handling Time Interval | One power cycle |
| Assumptions of Use (AoU) | In case of an LBIST failure, the system shall remain in the defined safe state. Normal operation shall not be allowed. |
| Self-Test | N/A |

Table 10: Logic Built-In Self-Test (LBIST)

### 8. Threshold configuration for over- and under voltage monitoring

#### 8.1 VOUT1 & 2 (Buck 1 & 2) configuration

VOUT1 & 2 (0.75V) is used for SoC VDDI

For the min and max conditions to be supervised, the PMIC needs to be configured for under- and over-voltage monitoring [2], SM_8, SM9, with the correct PG_CONFIG (0xCA), bit[0] set according to the recommended voltage ranges.

| PG_CONFIG (0xCA) of Buck 1, bit[0] = 1 | Threshold +4.5% / - 3.5% |
| --- | --- |
| PG_CONFIG (0xCA) of Buck 2, bit[0] = 1 | Threshold +4.5% / - 3.5% |

#### 8.2 VOUT3 (Buck3) configuration

VOUT3 1.1V is used for:

- DDR VDD2, VDDq

- PHY VDD_Core

- SoC MIPI_AVDD18_IO

- SoC DDR_DVDD_VDDQ

- SoC DDR_DVDD_IO

For the min and max conditions to be supervised, the PMIC needs to be configured for under- and over-voltage monitoring [2], SM_8, SM9, with the correct PG_CONFIG (0xCA), bit[0] set according to the recommended voltage ranges.

In the case of an overvoltage, the supply should be shut off to prevent damage to the PHY, c.f. [4], [5], SM_3

| PG_CONFIG (0xCA) of Buck 3, bit[0] = 1 | Threshold +4.5% / - 3.5% |
| --- | --- |
| SAFETY_CONFIG (0xC2) bit[1] (PG_REACTION) = 1 | PG low fault (only for PGUV/OV, VMON OV/UV fault) causes safe state (all outputs off) |

It should be checked from a system perspective, if “power rails off” in case of PG fault is valid reaction.

After discussion, “power rails off” is the preferred solution.

Alternatively:

| PG_CONFIG (0xCA) of Buck 3, bit[0] = 1 | Threshold +4.5% / - 3.5% |
| --- | --- |
| SAFETY_CONFIG (0xC2) bit[1] (PG_REACTION) = 0 (default) | PG low fault (only for PGUV/OV, VMON OV/UV fault) does not move to safe state (Default) |

PG should be checked via signal ISP_RST (POR_L at CV22FS) and handled separately.

While the setting of PG_CONFIG is for each buck separately, SAFETY_CONFIG is for all output rails.

#### 8.3 VOUT4 (Buck4) configuration

VOUT4 (0.8V) is used for SoC AVDD.

For the min and max conditions to be supervised, the PMIC needs to be configured for under- and over-voltage monitoring [2], SM_8, SM9, with the correct PG_CONFIG (0xCA), bit[0] set according to the recommended voltage ranges.

| PG_CONFIG (0xCA) of Buck 4, bit[0] = 1 | Threshold +4.5% / - 3.5% |
| --- | --- |

#### 8.4 VOUT5 (Buck5) configuration

VOUT5 1.8V is used for:

- DDR VDD1

- PHY VDDIO

- Deserializer MAX96714 VDD18

- Deserializer MAX96714 VDDIO

- Flash VCC18

For the min and max conditions to be supervised, the PMIC needs to be configured for under- and over-voltage monitoring [2], SM_8, SM9, with the correct PG_CONFIG (0xCA), bit[0] set according to the recommended voltage ranges.

| PG_CONFIG (0xCA) of Buck 5, bit[0] = 1 | Threshold +4.5% / - 3.5% |
| --- | --- |

#### 8.4 VOUT6 (Buck6) configuration

VOUT6 (1.2V) is used for:

- Deserializer MAX96714 VDD

- Deserializer MAX96714 VTERM

For the min and max conditions to be supervised, the PMIC needs to be configured for under- and over-voltage monitoring [2], SM_8, SM9, with the correct PG_CONFIG (0xCA), bit[0] set according to the recommended voltage ranges.

| PG_CONFIG (0xCA) of Buck 6, bit[0] = 1 | Threshold +4.5% / - 3.5% |
| --- | --- |