Shared Functions Library

v1.2 — Series Stream

Contents

1. About this Document	3

2. Reference Tables	3

2.1 Global SOME/IP Constants	3

2.2 Method ID Master Table (Service 0x433F)	4

2.3 SOME/IP Return Codes (Error Dictionary)	4

2.4 camControlIndex Reference	5

2.5 setCamControl Payload Encoding (5 bytes)	5

2.6 getCamControl Response Encoding (4 bytes)	5

2.7 subscribeROIVideo Payload Encoding (4 bytes)	6

2.8 Glossary of Constants	6

2.9 Bench Pin-Out & HSI	7

3. SOME/IP Commands	7

3.1 getCamIdentification	7

3.2 getDataSheet	8

3.3 getCamStatus	9

3.4 setCamExclusive	10

3.5 eraseCamExclusive	11

3.6 setCamControl	12

3.7 getCamControl	14

3.8 subscribeROIVideo	15

3.9 unSubscribeROIVideo	17

3.10 setCamID	18

3.11 setDataBlock	20

3.12 getDataBlock	22

4. General Functions	24

4.1 Set Power Supply	24

4.2 Set Chamber Temperature	25

4.3 Set PHY Link	26

4.4 Read PHY Link	26

4.5 Send TC10 Request	27

4.6 Measure Power Consumption	28

4.7 Check IP Configuration	29

4.8 Check RTP Stream	30

4.9 Monitor SD OfferService	31

4.10 Open / Close Shutter By Hand	32

4.11 Flash Alternative NVM Parameter Set	33

4.13 Read Imager Temperature	33

4.15 RTP Encoding / GStreamer Pipeline for ROI Verification (OPEN)	34

5. State Procedures	34

5.1 System Startup	34

5.2 Camera Init With Temperature	35

5.3 Goto OPERATIONAL_RUNNING	35

5.4 Goto OPERATIONAL_STREAMING	35

5.5 Goto DEEPSLEEP	36

5.6 Goto PHYLINKCHECK	36

5.7 Goto IP_CONFIGURATION	36

6. Blank Template	37

# 1. About this Document

This library defines reusable test steps (SOME/IP commands, bench actions, and state procedures) referenced by feature-level test case documents. Each function has a stable bookmark so feature TC documents can link to it without copying the steps.

# 2. Reference Tables

## 2.1 Global SOME/IP Constants

| Parameter | Value |
| --- | --- |
| Service ID (camera service) | 0x433F |
| Transport | UDP / IPv4 |
| UDP port (camera service) | NVM parameter cameraServicePort (test-bench default 30501) |
| Byte order | Big-endian (network byte order) |
| Protocol Version | 0x01 |
| Interface Version | 0x01 |
| Message Type – REQUEST | 0x00 |
| Message Type – RESPONSE | 0x80 |
| Message Type – ERROR | 0x81 |
| Return Code – positive | 0x00 (E_OK) |
| Header size (no payload) | 16 bytes (length-field counts last 8 bytes) |
| Request frequency limit | One request at a time per client |
| Max response delay (error) | 100 ms |
| MVC IP (test bench) | 192.168.120.x — bench DHCP convention; actual IP assigned by DHCP |
| OfferService cycle time (main phase) | 100 ms ± 20 % |
| SOME/IP-SD message Service ID | 0xFFFF |
| SOME/IP-SD message Method ID | 0x8100 |
| SOME/IP-SD entries array Service ID | 0x433F (announces camera service) |

## 2.2 Method ID Master Table (Service 0x433F)

| Method | Method ID | Req Len | Resp Len | Description |
| --- | --- | --- | --- | --- |
| getDataSheet | 0x0001 | 0x08 | 0x20D | Returns device identification, ROI count, intrinsic parameters from flash |
| getCamStatus | 0x0002 | 0x08 | 0x0F | Returns lockedBy IP, power status, imager temperature |
| getCamIdentification | 0x0010 | 0x08 | 0x17 | Returns SNPN, serial number, MAC |
| setCamExclusive | 0x0011 | 0x0C | 0x08 | Locks the camera; TTL payload |
| eraseCamExclusive | 0x0019 | 0x08 | 0x08 | Unlocks camera |
| subscribeROIVideo | 0x0131 | 0x0C | 0x08 | Starts RTP stream (positive response carries no payload) |
| unSubscribeROIVideo | 0x0132 | 0x0C | 0x08 | Stops RTP stream for ROI |
| setCamControl | 0x0201 | 0x0D | 0x08 | Sets control mode/value |
| getCamControl | 0x0203 | 0x09 | 0x0C | Reads control mode/value |

## 2.3 SOME/IP Return Codes (Error Dictionary)

| Return Code | Name | Trigger Condition |
| --- | --- | --- |
| 0x00 | E_OK | Positive response |
| 0x01 | E_NOT_OK | Generic negative response (e.g. stream already subscribed by another client) |
| 0x02 | E_UNKNOWN_SERVICE | Service ID is not 0x433F |
| 0x03 | E_UNKNOWN_METHOD | Unsupported method |
| 0x04 | E_NOT_READY | Service not ready |
| 0x05 | E_NOT_REACHABLE | Target unreachable |
| 0x06 | E_TIMEOUT | No response |
| 0x07 | E_WRONG_PROTOCOL_VERSION | Protocol version invalid |
| 0x08 | E_WRONG_INTERFACE_VERSION | Interface version invalid |
| 0x09 | E_MALFORMED_MESSAGE | Length mismatch / padding / header invalid |
| 0x0A | E_WRONG_MESSAGE_TYPE | Unexpected message type |
| 0x20 | E_LOCKED_BY_FOREIGN_INSTANCE | Camera locked by a different client (setCamExclusive / eraseCamExclusive / setCamControl) |
| 0x21 | E_LOCK_EXPIRED | setCamControl: caller has no active lock |
| 0x22 | E_NOT_LOCKED | eraseCamExclusive: no lock currently active |
| 0x30 | E_INVALID_ROI_INDEX | ROI index out of range or invalid Stream Selection bits |
| 0x35 | E_INVALID_CONTROL_INDEX | Unsupported camControlIndex |
| 0x36 | E_INVALID_CONTROL_MODE | controlMode not in {0x02, 0x03, 0x04} |
| 0x37 | E_INVALID_CONTROL_VALUE | Invalid controlValue for given mode |

## 2.4 camControlIndex Reference

| Hex | Control | Description |
| --- | --- | --- |
| 0x09 | Heater | Lens heater |
| 0x50 | Shutter | Mechanical shutter; only on supported variants |
| 0x60 | sensorFramerate | Sensor frame rate (verify support per variant before use) |
| 0x61 | maxExposureTime | Maximum exposure time (verify support per variant) |
| 0x62 | frameSyncTimeRef | Frame sync timing reference (verify support per variant) |
| 0x63 | whiteBalanceTarget | White balance target (verify support per variant) |

## 2.5 setCamControl Payload Encoding (5 bytes)

[camControlIndex 1B] [controlSupportedModes 1B] [controlMode 1B] [controlValue 2B big-endian]

| Field | Example | Meaning |
| --- | --- | --- |
| controlSupportedModes | 0x07 | Ignored on input; relevant on getCamControl output |
| controlMode = 0x02 | Manual | Manual mode |
| controlMode = 0x03 | ManualModified | User override |
| controlMode = 0x04 | Automatic | Firmware control |
| controlValue = 0x0000 | OFF / Close | Heater OFF / Shutter close |
| controlValue = 0x0001 | ON / Open | Heater ON / Shutter open |

## 2.6 getCamControl Response Encoding (4 bytes)

[controlSupportedModes 1B] [controlMode 1B] [controlValue 2B big-endian]

| State | Hex | Decoded |
| --- | --- | --- |
| Heater – Automatic & ON | 0x07040001 | supported=0x07, mode=Auto, value=ON |
| Heater – Automatic & OFF | 0x07040000 | supported=0x07, mode=Auto, value=OFF |
| Heater – Manual & ON | 0x07020001 | supported=0x07, mode=Manual, value=ON |
| Shutter – Automatic & Open | 0x07040001 | open |
| Shutter – Automatic & Closed | 0x07040000 | closed |
| Protection state | 0x07<mode>0000 | mode unchanged, value forced 0 |

## 2.7 subscribeROIVideo Payload Encoding (4 bytes)

| Bit Range | Field | Notes |
| --- | --- | --- |
| bit 31 | flip | 1 = vertical flip |
| bit 30 | mirror | 1 = horizontal mirror |
| bits 29–28 | Stream Selection | 0b00 = Stream 1, 0b01 = Stream 2. 0b10 / 0b11 → 0x30 E_INVALID_ROI_INDEX |
| bits 27–8 | T3 Setting fields | Resolution, Panning, Zoom, Framerate (encoded per variant). Ignored when Setting is not T3 |
| bits 7–0 | ROI index | 0 … numbersOfRegionOfInterest − 1 |

ROI-index-only subset (Stream 1, no T3 fields), used by most non-T3 TCs:

| Effect | Hex (ROI 0) | Hex (ROI 1) |
| --- | --- | --- |
| Default orientation | 0x00000000 | 0x00000001 |
| Flipped | 0x80000000 | 0x80000001 |
| Mirrored | 0x40000000 | 0x40000001 |
| Flipped + Mirrored | 0xC0000000 | 0xC0000001 |

## 2.8 Glossary of Constants

| Symbol | Typical Value | Description |
| --- | --- | --- |
| U_b_min | 9.0 V | Minimum supply voltage |
| U_b_max | 36.0 V | Maximum supply voltage |
| U_nominal | 24.0 V | Nominal supply |
| T_imager_auto_on | 15 °C | Heating ON threshold (MVC-91237) |
| T_imager_auto_off | 17 °C | Heating OFF threshold (MVC-91246, hysteresis ≥ 2 °C) |
| T_imager_protection_on | 40 °C | Protection engages (MVC-100702). NOTE: MVC-100350 mentions 45 °C — treated as a source TC documentation defect. |
| T_imager_protection_off | 35 °C | Protection releases (MVC-100701) |
| T_ser_protection_on | OPEN ISSUE | SAR rephrasing of MVC-92542 = 40 °C; SW-HW-MEC (MVC-92969 Heating modes) = 45 °C. Not yet committed — pending MEKRA system-engineering decision (Tracker §2.1). |
| T_ser_protection_off | OPEN ISSUE | SAR rephrasing of MVC-92541 = 35 °C; SW-HW-MEC MVC-92881 = 40 °C. Not yet committed — pending MEKRA system-engineering decision (Tracker §2.1). |
| t_heater_min_switch_period | 1 s | Minimum switch period (MVC-99304: fLH,cyc > 1/60s) |
| t_imager_temp_sample | ≤16.7 ms (1/60 s) | Imager temperature evaluation period (SAR MVC-99801, Predefined). |
| t_shutter_open | ≤0.7 s | Max shutter open time (MVC-91226) |
| P_deepsleep | ≤100 µA | Max DEEPSLEEP power @24V (MVC-95010) |
| t_wakeup | ≤1000 ms | Wake-up transition time (MVC-93117) |
| numbersOfRegionOfInterest | variant-dependent | From getDataSheet offset 509 |
| cameraServicePort | NVM (test default 30501) | UDP port for SOME/IP camera service |
| MTU (IPv4) | 1500 B | Default IPv4 MTU |

## 2.9 Bench Pin-Out & HSI

| Signal (SAR) | Pin / GPIO | Behaviour |
| --- | --- | --- |
| camHeatingEn | V16 / GPIO1 | HIGH = heater ON; LOW = heater OFF |
| shutterCtl (ISP_SHUTTER.CTL) | C14 / GPIO52 | HIGH = shutter closed; LOW = shutter open |
| shutterEn (POWER_SHUT_EN) | C13 / GPIO55 | LOW = power on for shutter-supported variant; HIGH = power off |

# 3. SOME/IP Commands

### 3.1 getCamIdentification

Objective

Read the camera identification block (SNPN, serial number, MAC address) via Method ID 0x0010.

Steps

| # | Action / Send | Expected Result / Receive |
| --- | --- | --- |
| 1 | Send Service ID: 0x433F Method ID: 0x0010 Length: 0x00000008 Request ID: 0x12345678 Protocol Version: 0x01 Interface Version: 0x01 Message Type: 0x00 (REQUEST) Return Code: 0x00 Payload: None | Receive Service ID: 0x433F Method ID: 0x0010 Length: 0x00000017 Request ID: 0x12345678 Protocol Version: 0x01 Interface Version: 0x01 Message Type: 0x80 (RESPONSE) Return Code: 0x00 Payload: 15 bytes — see Payload Reference Dictionary |
| 2 | Split the received payload into SNPN, SerialNumber BCD, and MAC address. | SNPN, serial number, and MAC address match the production label / expected result list. |

Payload Reference Dictionary

| Payload Type | Parameter / State | Hex Value |
| --- | --- | --- |
| Received_Payload | Total length | 15 bytes (0x0F) |
| Received_Payload | SNPN | 5 bytes, offset 0 |
| Received_Payload | SerialNumber (BCD) | 4 bytes, offset 5 |
| Received_Payload | MAC address | 6 bytes, offset 9, prefix 98:D3:D2:xx:xx:xx |

Negative Response / Error Codes

| Trigger Condition | Return Code | Error Name |
| --- | --- | --- |
| Method called before IP configuration completes | 0x04 | E_NOT_READY |

Source / References

| Requirement / TC ID | Title / Topic |
| --- | --- |
| MVC-92545 | getCamIdentification — Purpose |
| MVC-93025 | getCamIdentification — Positive Response |
| MVC-92869 | getCamIdentification (Systemtest) |
| MVC-102212 | SOME/IP – getCamIdentification – Message Request |

### 3.2 getDataSheet

Objective

Read the camera data sheet (device identification, ROI count, intrinsic parameters) via Method ID 0x0001.

Steps

| # | Action / Send | Expected Result / Receive |
| --- | --- | --- |
| 1 | Send Service ID: 0x433F Method ID: 0x0001 Length: 0x00000008 Request ID: 0x12345678 Protocol Version: 0x01 Interface Version: 0x01 Message Type: 0x00 (REQUEST) Return Code: 0x00 Payload: None | Receive Service ID: 0x433F Method ID: 0x0001 Length: 0x0000020D Request ID: 0x12345678 Protocol Version: 0x01 Interface Version: 0x01 Message Type: 0x80 (RESPONSE) Return Code: 0x00 Payload: 517 bytes — see Payload Reference Dictionary |
| 2 | Extract fields at declared offsets and compare with production database. | All listed fields match expected values. |

Payload Reference Dictionary

| Payload Type | Parameter / State | Hex Value |
| --- | --- | --- |
| Received_Payload | deviceIdentification (67 B) | offset 0: macAddress, batchNumber, partNumber, partRev, serialNumber, timestamp, epShort, hwVer, parVer |
| Received_Payload | standardVersion (4 B) | offset 256: 0x00000001 |
| Received_Payload | intrinsicCamParam (200 B) | offset 305: zero-filled (not supported in SOW1) |
| Received_Payload | numberOfTemperatures (4 B) | offset 505: 0x00000001 |
| Received_Payload | numbersOfRegionOfInterest (4 B) | offset 509: variant-dependent |
| Received_Payload | Unused bytes | 0x00 |

Source / References

| Requirement / TC ID | Title / Topic |
| --- | --- |
| MVC-92928 | getDataSheet (Systemtest) |

### 3.3 getCamStatus

Objective

Read camera status (lockedBy IP, power status, imager temperature) via Method ID 0x0002.

Steps

| # | Action / Send | Expected Result / Receive |
| --- | --- | --- |
| 1 | Send Service ID: 0x433F Method ID: 0x0002 Length: 0x00000008 Request ID: 0x12345678 Protocol Version: 0x01 Interface Version: 0x01 Message Type: 0x00 (REQUEST) Return Code: 0x00 Payload: None | Receive Service ID: 0x433F Method ID: 0x0002 Length: 0x0000000F Request ID: 0x12345678 Protocol Version: 0x01 Interface Version: 0x01 Message Type: 0x80 (RESPONSE) Return Code: 0x00 Payload: 7 bytes (lockedBy 4B + powerStatus 1B + temperature 2B) |
| 2 | Decode payload at fixed offsets. | offset 0–3: lockedBy IPv4 (0x00000000 when unlocked) offset 4: powerStatus (managed by generic device) offset 5–6: imager temperature (uint16). [Value = 0x0000 outside STREAMING.] |

Payload Reference Dictionary

| Payload Type | Parameter / State | Hex Value |
| --- | --- | --- |
| Received_Payload | Not locked, not streaming | 0x00000000 00 0000 |
| Received_Payload | Locked by 192.168.120.1 | 0xC0A87801 00 0000 |
| Received_Payload | Streaming, imager 25.0 °C | 0x<lockedBy> 00 0FA0 |
| Note | powerStatus byte | Reserved for internal use; do not assert exact value |

Source / References

| Requirement / TC ID | Title / Topic |
| --- | --- |
| MVC-92549 | getCamStatus — Purpose |
| MVC-94767 | getCamStatus — Response Content |
| MVC-97848 | getCamStatus — Message Response |
| MVC-93346 | Simple getCamStatus after camera reset |
| MVC-93345 | getCamStatus – camera locked |
| MVC-93348 | getCamStatus – video stream active |

### 3.4 setCamExclusive

Objective

Lock the camera for exclusive control by the requesting client for a given TTL via Method ID 0x0011.

Parameters

| Name | Type / Range | Description |
| --- | --- | --- |
| TTL_seconds | 0x00000000 – 0xFFFFFFFF s (big-endian) | Lock duration. 0 releases lock held by same client. |

Steps

| # | Action / Send | Expected Result / Receive |
| --- | --- | --- |
| 1 | Send Service ID: 0x433F Method ID: 0x0011 Length: 0x0000000C Request ID: 0x12345678 Protocol Version: 0x01 Interface Version: 0x01 Message Type: 0x00 (REQUEST) Return Code: 0x00 Payload: TTL_seconds (4 B) | Receive Service ID: 0x433F Method ID: 0x0011 Length: 0x00000008 Request ID: 0x12345678 Protocol Version: 0x01 Interface Version: 0x01 Message Type: 0x80 (RESPONSE) Return Code: 0x00 Payload: None |
| 2 | Call getCamStatus to verify the lock. | lockedBy equals the requester IPv4 address. |

Payload Reference Dictionary

| Payload Type | Parameter / State | Hex Value |
| --- | --- | --- |
| Sent_Payload | TTL = 0 s | 0x00000000 |
| Sent_Payload | TTL = 1 s | 0x00000001 |
| Sent_Payload | TTL = 2 s | 0x00000002 |
| Sent_Payload | TTL = 60 s | 0x0000003C |
| Sent_Payload | TTL = 600 s (10 min) | 0x00000258 |
| Sent_Payload | TTL = 1800 s (30 min) | 0x00000708 |
| Sent_Payload | TTL = 86400 s (24 h) | 0x00015180 |
| Sent_Payload | TTL = max | 0xFFFFFFFF |

Negative Response / Error Codes

| Trigger Condition | Return Code | Error Name |
| --- | --- | --- |
| Camera already locked by a different client IP | 0x20 | E_LOCKED_BY_FOREIGN_INSTANCE |
| Invalid Length field | 0x09 | E_MALFORMED_MESSAGE |

Source / References

| Requirement / TC ID | Title / Topic |
| --- | --- |
| MVC-92552 | setCamExclusive — Purpose |
| MVC-92938 | setCamExclusive – up to 10 minutes |
| MVC-92937 | setCamExclusive – 24 hours |
| MVC-93334 | Set and eraseCamExclusive cyclic |

### 3.5 eraseCamExclusive

Objective

Release the camera lock held by the requesting client via Method ID 0x0019.

Steps

| # | Action / Send | Expected Result / Receive |
| --- | --- | --- |
| 1 | Send Service ID: 0x433F Method ID: 0x0019 Length: 0x00000008 Request ID: 0x12345678 Protocol Version: 0x01 Interface Version: 0x01 Message Type: 0x00 (REQUEST) Return Code: 0x00 Payload: None | Receive Service ID: 0x433F Method ID: 0x0019 Length: 0x00000008 Request ID: 0x12345678 Protocol Version: 0x01 Interface Version: 0x01 Message Type: 0x80 (RESPONSE) Return Code: 0x00 Payload: None |
| 2 | Call getCamStatus to verify lock release. | lockedBy = 0x00000000. |

Negative Response / Error Codes

| Trigger Condition | Return Code | Error Name |
| --- | --- | --- |
| Lock held by a different client IP | 0x20 | E_LOCKED_BY_FOREIGN_INSTANCE |
| No lock currently active | 0x22 | E_NOT_LOCKED |

Source / References

| Requirement / TC ID | Title / Topic |
| --- | --- |
| MVC-99814 | eraseCamExclusive — Message Request |
| MVC-99815 | eraseCamExclusive — Message Response |
| MVC-99823 | eraseCamExclusive — Positive Response |
| MVC-92566 | eraseCamExclusive — E_LOCKED_BY_FOREIGN_INSTANCE |
| MVC-92567 | eraseCamExclusive — E_NOT_LOCKED |
| MVC-91218 | eraseCamExclusive: device locked by different client |
| MVC-102544 | eraseCamExclusive: not locked |

### 3.6 setCamControl

Objective

Set the mode and value of a camera control (heater, shutter, etc.) via Method ID 0x0201.

Parameters

| Name | Type / Range | Description |
| --- | --- | --- |
| camControlIndex | 1 B | Control to address (see 2.4). |
| controlSupportedModes | 1 B | Ignored on input; typically 0x07. |
| controlMode | 1 B | 0x02 Manual, 0x03 ManualModified, 0x04 Automatic. |
| controlValue | 2 B big-endian | With ManualModified, only 0x0000 and 0x0001 are valid. |

Steps

| # | Action / Send | Expected Result / Receive |
| --- | --- | --- |
| 1 | Pre-step: ensure camera is locked by the requesting client (setCamExclusive). | Lock confirmed via getCamStatus. |
| 2 | Send Service ID: 0x433F Method ID: 0x0201 Length: 0x0000000D Request ID: 0x12345678 Protocol Version: 0x01 Interface Version: 0x01 Message Type: 0x00 (REQUEST) Return Code: 0x00 Payload: 5-byte control block | Receive Service ID: 0x433F Method ID: 0x0201 Length: 0x00000008 Request ID: 0x12345678 Protocol Version: 0x01 Interface Version: 0x01 Message Type: 0x80 (RESPONSE) Return Code: 0x00 Payload: None |

Payload Reference Dictionary

| Payload Type | Parameter / State | Hex Value |
| --- | --- | --- |
| Payload structure | Layout | [camControlIndex][controlSupportedModes][controlMode][controlValue MSB][controlValue LSB] |
| Heater (0x09) | Manual | 0x09 07 02 00 00 |
| Heater (0x09) | ManualModified ON | 0x09 07 03 00 01 |
| Heater (0x09) | ManualModified OFF | 0x09 07 03 00 00 |
| Heater (0x09) | Automatic | 0x09 07 04 00 00 |
| Shutter (0x50) | Manual | 0x50 07 02 00 00 |
| Shutter (0x50) | ManualModified Open | 0x50 07 03 00 01 |
| Shutter (0x50) | ManualModified Close | 0x50 07 03 00 00 |
| Shutter (0x50) | Automatic | 0x50 07 04 00 00 |

Negative Response / Error Codes

| Trigger Condition | Return Code | Error Name |
| --- | --- | --- |
| Caller has no active lock (never locked / lock expired) | 0x21 | E_LOCK_EXPIRED |
| Camera locked by a different client | 0x20 | E_LOCKED_BY_FOREIGN_INSTANCE |
| camControlIndex unsupported | 0x35 | E_INVALID_CONTROL_INDEX |
| Shutter index 0x50 on non-shutter variant | 0x35 | E_INVALID_CONTROL_INDEX |
| controlMode not in {0x02, 0x03, 0x04} | 0x36 | E_INVALID_CONTROL_MODE |
| ManualModified controlValue not 0x0000 / 0x0001 | 0x37 | E_INVALID_CONTROL_VALUE |
| Invalid Length field | 0x09 | E_MALFORMED_MESSAGE |

Source / References

| Requirement / TC ID | Title / Topic |
| --- | --- |
| MVC-92603 | setCamControl — Specific Error Codes |
| MVC-92600 | setCamControl — E_LOCK_EXPIRED |
| MVC-92601 | setCamControl — E_LOCKED_BY_FOREIGN_INSTANCE |
| MVC-92598 | setCamControl — E_INVALID_CONTROL_INDEX |
| MVC-92599 | setCamControl — E_INVALID_CONTROL_MODE |
| MVC-92596 | setCamControl — E_INVALID_CONTROL_VALUE |
| MVC-92947 | setCamControl – invalid camControlIndex |
| MVC-100187 | Heating Control Processing via setCamControl |
| MVC-100390 | setCamControl – shutter |

### 3.7 getCamControl

Objective

Read the current mode and value of a camera control via Method ID 0x0203.

Parameters

| Name | Type / Range | Description |
| --- | --- | --- |
| camControlIndex | 1 B | 0x09 Heater, 0x50 Shutter, 0x60/0x61/0x62/0x63 controls |

Steps

| # | Action / Send | Expected Result / Receive |
| --- | --- | --- |
| 1 | Send Service ID: 0x433F Method ID: 0x0203 Length: 0x00000009 Request ID: 0x12345678 Protocol Version: 0x01 Interface Version: 0x01 Message Type: 0x00 (REQUEST) Return Code: 0x00 Payload: camControlIndex (1 B) | Receive Service ID: 0x433F Method ID: 0x0203 Length: 0x0000000C (8B header + 4B payload) Request ID: 0x12345678 Protocol Version: 0x01 Interface Version: 0x01 Message Type: 0x80 (RESPONSE) Return Code: 0x00 Payload: 4-byte block: [supportedModes][controlMode][controlValue] |

Payload Reference Dictionary

| Payload Type | Parameter / State | Hex Value |
| --- | --- | --- |
| Sent_Payload | Heater | 0x09 |
| Sent_Payload | Shutter | 0x50 |
| Received_Payload (Heater) | Automatic & ON | 0x07040001 |
| Received_Payload (Heater) | Automatic & OFF | 0x07040000 |
| Received_Payload (Heater) | Manual & ON | 0x07020001 |
| Received_Payload (Heater) | Manual & OFF | 0x07020000 |
| Received_Payload (Shutter) | Automatic & Closed | 0x07040000 |
| Received_Payload (Shutter) | Automatic & Open | 0x07040001 |
| Received_Payload (Shutter) | Manual & Closed | 0x07020000 |
| Received_Payload (Shutter) | Manual & Open | 0x07020001 |

Negative Response / Error Codes

| Trigger Condition | Return Code | Error Name |
| --- | --- | --- |
| camControlIndex not supported | 0x35 | E_INVALID_CONTROL_INDEX |

Source / References

| Requirement / TC ID | Title / Topic |
| --- | --- |
| MVC-92946 | getCamControl – invalid camControlIndex |
| MVC-100225 | Heating control retrieval |
| MVC-100384 | getCamControl – shutter |

### 3.8 subscribeROIVideo

Objective

Subscribe to a Region-Of-Interest video stream and start RTP transmission via Method ID 0x0131.

Parameters

| Name | Type / Range | Description |
| --- | --- | --- |
| roi_index | 0 – numbersOfRegionOfInterest − 1 | Low 8 bits of roiIndex (see 2.7). |
| orientation_flip | bit 31 | Set for vertical flip. |
| orientation_mirror | bit 30 | Set for horizontal mirror. |
| stream | bits 29–28 | 0b00 = Stream 1, 0b01 = Stream 2. |
| t3_setting | bits 27–8 (optional) | Resolution/Panning/Zoom/Framerate for T3 Setting; 0 otherwise. |

Steps

| # | Action / Send | Expected Result / Receive |
| --- | --- | --- |
| 1 | Send Service ID: 0x433F Method ID: 0x0131 Length: 0x0000000C Request ID: 0x12345678 Protocol Version: 0x01 Interface Version: 0x01 Message Type: 0x00 (REQUEST) Return Code: 0x00 Payload: roiIndex (4 B, see 2.7) | Receive Service ID: 0x433F Method ID: 0x0131 Length: 0x00000008 Request ID: 0x12345678 Protocol Version: 0x01 Interface Version: 0x01 Message Type: 0x80 (RESPONSE) Return Code: 0x00 Payload: None |
| 2 | Verify the RTP stream using Check RTP Stream (4.8). | RTP stream visible with resolution/framerate for the requested ROI. |

Payload Reference Dictionary

| Payload Type | Parameter / State | Hex Value |
| --- | --- | --- |
| Payload structure | Layout | [bit31 flip][bit30 mirror][bits 29–28 Stream][bits 27–8 T3 fields][bits 7–0 ROI index] |
| Sent_Payload | Stream 1, ROI 0 default | 0x00000000 |
| Sent_Payload | Stream 1, ROI 1 default | 0x00000001 |
| Sent_Payload | Stream 2, ROI 0 default | 0x10000000 |
| Sent_Payload | Stream 1, ROI 0 flipped | 0x80000000 |
| Sent_Payload | Stream 1, ROI 0 mirrored | 0x40000000 |
| Sent_Payload | Stream 1, ROI 0 flipped + mirrored | 0xC0000000 |
| Sent_Payload | Stream 2, ROI 3 flipped + mirrored | 0xD4000003 |
|  |  | 0x5100812F(0b01010001000000001000000100101111) T3 , 30 Hz , zoom 150 % , pos. 1 , 1920x1080 ,Stream 2 flip off, mirror on unused bits: 0001 (random, d.c.) |

Negative Response / Error Codes

| Trigger Condition | Return Code | Error Name |
| --- | --- | --- |
| ROI index >= numbersOfRegionOfInterest | 0x30 | E_INVALID_ROI_INDEX |
| Invalid Stream Selection bits (29–28 = 0b10 or 0b11) | 0x30 | E_INVALID_ROI_INDEX |
| Invalid T3 Resolution / Panning / Zoom / Framerate field | 0x30 | E_INVALID_ROI_INDEX |
| ROI index = 0xFFFFFFFF | 0x30 | E_INVALID_ROI_INDEX |
| Stream already subscribed by another client | 0x01 | E_NOT_OK |
| Invalid Length / Protocol Version / Interface Version | 0x09 / 0x07 / 0x08 | see Error Dictionary |

Notes

Multiple ROIs can be subscribed concurrently. On shutter-equipped variants in Automatic Mode, subscribing opens the shutter within t_shutter_open (≤0.7 s).

Source / References

| Requirement / TC ID | Title / Topic |
| --- | --- |
| MVC-93967 | subscribeROIVideo — Purpose |
| MVC-93359 | subscribeROIVideo – all ROIs |
| MVC-92962 | subscribeROIVideo – flipping and mirroring |
| MVC-92963 | subscribeROIVideo – voltage behavior |
| MVC-96113 | subscribeROIVideo - positive response (T3) |
| MVC-97795 | subscribeROIVideo: invalid Length field |
| MVC-97825 | subscribeROIVideo - validation of client IP (stream 2) |

### 3.9 unSubscribeROIVideo

Objective

Unsubscribe from a ROI video stream via Method ID 0x0132.

Parameters

| Name | Type / Range | Description |
| --- | --- | --- |
| roi_index | 0 – numbersOfRegionOfInterest − 1 | ROI to release |

Steps

| # | Action / Send | Expected Result / Receive |
| --- | --- | --- |
| 1 | Send Service ID: 0x433F Method ID: 0x0132 Length: 0x0000000C Request ID: 0x0000000F Protocol Version: 0x01 Interface Version: 0x01 Message Type: 0x00 (REQUEST) Return Code: 0x00 Payload: roiIndex (4 B) | Receive Service ID: 0x433F Method ID: 0x0132 Length: 0x00000008 Request ID: 0x0000000F Protocol Version: 0x01 Interface Version: 0x01 Message Type: 0x80 (RESPONSE) Return Code: 0x00 Payload: None |
| 2 | Verify RTP stream state via Check RTP Stream (4.8). | If this was the last active subscription, RTP stops; otherwise remaining streams continue. |

Payload Reference Dictionary

| Payload Type | Parameter / State | Hex Value |
| --- | --- | --- |
| Sent_Payload | Unsubscribe Stream 1, ROI 0 | 0x00000000 |
| Sent_Payload | Unsubscribe Stream 1, ROI 1 | 0x00000001 |
| Sent_Payload | Unsubscribe Stream 2, ROI 0 | 0x10000000 |

Negative Response / Error Codes

| Trigger Condition | Return Code | Error Name |
| --- | --- | --- |
| ROI index >= numbersOfRegionOfInterest | 0x30 | E_INVALID_ROI_INDEX |
| ROI index = 0xFFFFFFFF | 0x30 | E_INVALID_ROI_INDEX |
| Unsubscribe by a different client IP than the subscriber | 0x01 | E_NOT_OK |
| Invalid Length field | 0x09 | E_MALFORMED_MESSAGE |

Source / References

| Requirement / TC ID | Title / Topic |
| --- | --- |
| MVC-93359 | subscribeROIVideo / unSubscribeROIVideo – all ROIs |

### 3.10 setCamID

Objective

Set the camera ID transmitted via SEI message to the HV application via Method ID 0x0409.

Parameters

| Name | Type / Range | Description |
| --- | --- | --- |
| numCameras | uint8 — ignored | Accepted but not used or validated by MVC (MVC-102739). |
| cameraID | uint8, 0x00–0x0F | Camera identifier. Default 0x00. |

Steps

| # | Action / Send | Expected Result / Receive |
| --- | --- | --- |
| 1 | Send Service ID: 0x433F Method ID: 0x0409 Length: 0x0000000A Request ID: 0x12345678 Protocol Version: 0x01 Interface Version: 0x01 Message Type: 0x00 (REQUEST) Return Code: 0x00 Payload: 2 bytes — see Payload Reference Dictionary | Receive Service ID: 0x433F Method ID: 0x0409 Length: 0x00000008 Request ID: 0x12345678 Protocol Version: 0x01 Interface Version: 0x01 Message Type: 0x80 (RESPONSE) Return Code: 0x00 Payload: None |

Payload Reference Dictionary

| Payload Type | Parameter / State | Hex Value |
| --- | --- | --- |
| Payload structure | Layout | [numCameras (uint8, offset 0)] [cameraID (uint8, offset 1)] |
| Sent_Payload | numCameras (any value) | ignored by MVC |
| Sent_Payload | cameraID = 0x00 | default camera ID |
| Sent_Payload | cameraID = 0x0F | maximum camera ID |
| Received_Payload | Positive response payload | none (0 bytes) |

Negative Response / Error Codes

| Trigger Condition | Return Code | Error Name |
| --- | --- | --- |
| cameraID outside 0x00–0x0F | TBD (error code not yet published — MVC-102738 lists name only) | E_INVALID_CAMID |
| Wrong Protocol Version | 0x07 | E_WRONG_PROTOCOL_VERSION |
| Length mismatch | 0x09 | E_MALFORMED_MESSAGE |

Source / References

| Requirement / TC ID | Title / Topic |
| --- | --- |
| MVC-102777 | setCamID — Purpose (System Requirements, Draft) |
| MVC-102736 | setCamID — Message Request (SAR-SW, Draft) |
| MVC-102739 | setCamID — Payload evaluation (SAR-SW, Draft) |
| MVC-102738 | setCamID — Specific Error Codes (SAR-SW, Draft) |
| MVC-102740 | setCamID — Message Response (SAR-SW, Draft) |

### 3.11 setDataBlock

Objective

Write a supported data block to the MVC via Method ID 0x0250. Currently the only supported combination is (deviceID 0x0001, blockID 0x0000) — RawBufferEvent of VCM in GVTtime (absolute timestamp in ns).

Parameters

| Name | Type / Range | Description |
| --- | --- | --- |
| deviceID | uint16 — see Payload Reference Dictionary | Device target. |
| blockID | uint16 — see Payload Reference Dictionary | Block identifier. |
| blocksize | uint32 | Block size in bytes (must match expected size for the supported block). |
| block content | byte array, blocksize bytes | Actual data to write. |

Steps

| # | Action / Send | Expected Result / Receive |
| --- | --- | --- |
| 1 | Send Service ID: 0x433F Method ID: 0x0250 Length: 0x00000008 + 0x00000008 + block content size Request ID: 0x12345678 Protocol Version: 0x01 Interface Version: 0x01 Message Type: 0x00 (REQUEST) Return Code: 0x00 Payload: deviceID (2B) + blockID (2B) + blocksize (4B) + block content | Receive Service ID: 0x433F Method ID: 0x0250 Length: 0x00000008 Request ID: 0x12345678 Protocol Version: 0x01 Interface Version: 0x01 Message Type: 0x80 (RESPONSE) Return Code: 0x00 Payload: None |

Payload Reference Dictionary

| Payload Type | Parameter / State | Hex Value |
| --- | --- | --- |
| Payload structure | Layout | [deviceID uint16 offset 0] [blockID uint16 offset 2] [blocksize uint32 offset 4] [block content offset 8] |
| Sent_Payload | deviceID = 0x0001 | SOW2 runtime parameter |
| Sent_Payload | blockID = 0x0000 | RawBufferEvent of VCM in GVTtime |
| Sent_Payload | blocksize = 0x00000008 | 8 bytes (only supported block size) |
| Sent_Payload | block content default | 0x00000000 01D2EB40 |
| Received_Payload | Positive response payload | none (0 bytes) |

Negative Response / Error Codes

| Trigger Condition | Return Code | Error Name |
| --- | --- | --- |
| Unsupported deviceID / blockID / block size combination | 0x50 | E_INVALID_BLOCKDEFINITION |
| Other unsupported input parameters | 0x01 | E_NOT_OK |
| Wrong Protocol Version | 0x07 | E_WRONG_PROTOCOL_VERSION |
| Length mismatch | 0x09 | E_MALFORMED_MESSAGE |

Source / References

| Requirement / TC ID | Title / Topic |
| --- | --- |
| MVC-102433 | setDataBlock — Purpose (System Requirements, Draft) |
| MVC-102132 | setDatablock — Message Request (SAR-SW, Predefined) |
| MVC-102131 | setDatablock — Payload evaluation (SAR-SW, Predefined) |
| MVC-102134 | setDatablock — Supported blocks in devices (SAR-SW, Predefined) |
| MVC-102263 | setDatablock — Specific Error Codes (SAR-SW, Predefined) |
| MVC-102262 | setDatablock — E_INVALID_BLOCKDEFINITION trigger (SAR-SW, Predefined) |
| MVC-102133 | setDatablock — Message Response (SAR-SW, Predefined) |

### 3.12 getDataBlock

Objective

Read a supported data block from the MVC via Method ID 0x0251. Supported blocks: SOW1 Label Block, SOW1 Intrinsic Data Block, current frameSyncTimeRef, serializer die temperature.

Parameters

| Name | Type / Range | Description |
| --- | --- | --- |
| deviceID | uint16 — see Payload Reference Dictionary | Device target. |
| blockID | uint16 — see Payload Reference Dictionary | Block identifier. |

Steps

| # | Action / Send | Expected Result / Receive |
| --- | --- | --- |
| 1 | Send Service ID: 0x433F Method ID: 0x0251 Length: 0x0000000C (deviceID 2B + blockID 2B + 4B padding for 8-byte alignment) Request ID: 0x12345678 Protocol Version: 0x01 Interface Version: 0x01 Message Type: 0x00 (REQUEST) Return Code: 0x00 Payload: deviceID (2B) + blockID (2B) + 4B padding | Receive Service ID: 0x433F Method ID: 0x0251 Length: 0x00000008 + 0x00000004 + block size Request ID: 0x12345678 Protocol Version: 0x01 Interface Version: 0x01 Message Type: 0x80 (RESPONSE) Return Code: 0x00 Payload: blockSize (uint32, 4B) + block content (blockSize bytes) |
| 2 | Verify the returned block content against the expected data. | Block content matches expected values for the requested block. |

Payload Reference Dictionary

| Payload Type | Parameter / State | Hex Value |
| --- | --- | --- |
| Payload structure (request) | Layout | [deviceID uint16 offset 0] [blockID uint16 offset 2] [padding 4B offset 4] |
| Payload structure (response) | Layout | [blockSize uint32 offset 0] [block content offset 4] |
| Sent_Payload | deviceID = 0x0000 | camera EEPROM (SOW1) |
| Sent_Payload, blockID = 0x0000 | Label Block | 224 bytes |
| Sent_Payload, blockID = 0x0001 | Intrinsic Data Block | 352 bytes |
| Sent_Payload | deviceID = 0x0001 | SOW2 runtime parameter |
| Sent_Payload, blockID = 0x0000 | frameSyncTimeRef (absolute timestamp in ns) | 8 bytes |
| Sent_Payload | deviceID = 0x0002 | camera serializer |
| Sent_Payload, blockID = 0x0000 | serializer die temperature (ISO17215-3 §6.6.25 format; padding required) | 2 bytes + padding |
| Received_Payload | blockSize field | uint32, value = block size in bytes per requested block |
| Note | SOW1 blocks if not retrieved at startup | returned filled with nulls (MVC-102146) |

Negative Response / Error Codes

| Trigger Condition | Return Code | Error Name |
| --- | --- | --- |
| Unsupported deviceID / blockID combination | 0x50 | E_INVALID_BLOCKDEFINITION |
| Wrong Protocol Version | 0x07 | E_WRONG_PROTOCOL_VERSION |
| Length mismatch | 0x09 | E_MALFORMED_MESSAGE |

Source / References

| Requirement / TC ID | Title / Topic |
| --- | --- |
| MVC-102434 | getDataBlock — Purpose (System Requirements, Draft) |
| MVC-102306 | SOW1 EEPROM readout (System Requirements, Specified) |
| MVC-102139 | getDatablock — Message Request (SAR-SW, Predefined) |
| MVC-102147 | getDatablock — Payload evaluation + padding note (SAR-SW, Predefined) |
| MVC-102146 | getDatablock — Supported blocks in devices (SAR-SW, Predefined) |
| MVC-102257 | getDatablock — Specific Error Codes (SAR-SW, Predefined) |
| MVC-102256 | getDatablock — E_INVALID_BLOCKDEFINITION trigger (SAR-SW, Predefined) |
| MVC-102156 | getDatablock — Message Response (SAR-SW, Predefined) |
| MVC-102155 | getDatablock — Response payload (SAR-SW, Predefined) |

# 4. General Functions

### 4.1 Set Power Supply

Objective

Set channel-1 lab power supply to a target voltage and enable output.

Parameters

| Name | Type / Range | Description |
| --- | --- | --- |
| voltage_target | 0.0 V – 36.0 V | Setpoint. 0 powers MVC off; ≥ U_b_min powers it on. |
| channel | integer, default 1 | Power supply channel. |

Steps

| # | Action / Send | Expected Result / Receive |
| --- | --- | --- |
| 1 | Set channel voltage to <voltage_target> V. | Setpoint accepted. |
| 2 | Enable channel output. | Output relay closes. |
| 3 | Read actual output voltage. | V_measured = voltage_target ± 0.2 V. |

Payload Reference Dictionary

| Payload Type | Parameter / State | Hex Value |
| --- | --- | --- |
| Setpoint | 0.0 V | MVC powered down |
| Setpoint | 9.0 V | Lower boundary |
| Setpoint | 24.0 V | Nominal |
| Setpoint | 28.0 V | High nominal |
| Setpoint | 36.0 V | Upper boundary |

### 4.2 Set Chamber Temperature

Objective

Set the climate chamber to a target ambient temperature and wait for stabilization.

Parameters

| Name | Type / Range | Description |
| --- | --- | --- |
| T_target | −40 °C – +85 °C | Target ambient temperature. |
| wait_for_stable | bool, default true | Wait for ±0.5 °C for ≥5 min. |
| max_wait_min | integer, default 30 | Timeout. |

Steps

| # | Action / Send | Expected Result / Receive |
| --- | --- | --- |
| 1 | Enter T_target on chamber controller. | Setpoint accepted. |
| 2 | If wait_for_stable: monitor until stable. | Stable at target or timeout flagged. |

### 4.3 Set PHY Link

Objective

Force host PHY link UP or DOWN to simulate cable disconnect/reconnect.

Parameters

| Name | Type / Range | Description |
| --- | --- | --- |
| state | UP / DOWN | Target PHY link state. |

Steps

| # | Action / Send | Expected Result / Receive |
| --- | --- | --- |
| 1 | Open Technica MediaConverter and connect to host PHY. | Current PHY visible. |
| 2 | Open MDIO register access and locate link-enable control. | Register displayed. |
| 3 | Write register to requested state. | Write acknowledged. |
| 4 | Confirm with Read PHY Link (4.4). | Reported state matches. |

Source / References

| Requirement / TC ID | Title / Topic |
| --- | --- |
| MVC-95098 | PHY supports TC10 |
| Bench | Used by state-machine TCs |

### 4.4 Read PHY Link

Objective

Read current PHY link status (UP or DOWN).

Steps

| # | Action / Send | Expected Result / Receive |
| --- | --- | --- |
| 1 | Start up camera system. | N/A |
| 2 | Read PHY register PMA_STATUS1 (0x01) via MDIO using the AmbaShell: Command: svc_net cl45read 1 0x1 Inspect bit 7 and bit 2. | PHY UP: bit 7 = 0 AND bit 2 = 1 PHY DOWN: bit 7 = 1 OR bit 2 = 0 |

Payload Reference Dictionary

| State | Bits Value |
| --- | --- |
| PHY UP | Bit 7 = 0, Bit 2 = 1 |
| PHY DOWN | Bit 7 = 1 OR Bit 2 = 0 (link-up condition not met) |

### 4.5 Send TC10 Request

Objective

Send TC10 Low Power Sleep or Wake-Up Request via Technica MediaConverter.

Parameters

| Name | Type / Range | Description |
| --- | --- | --- |
| request_type | LPS / WUR | LPS sends to DEEPSLEEP; WUR wakes it. |

Steps

| # | Action / Send | Expected Result / Receive |
| --- | --- | --- |
| 1 | Open TC10 / Event command menu. | Interface visible. |
| 2 | Select request_type and send. | LPS: DEEPSLEEP; WUR: wakes within t_wakeup. |

Payload Reference Dictionary

| Payload Type | Parameter / State | Hex Value |
| --- | --- | --- |
| Parameter | request_type = LPS | Low Power Sleep event |
| Parameter | request_type = WUR | Wake-Up Request event |

Source / References

| Requirement / TC ID | Title / Topic |
| --- | --- |
| MVC-95100 | DEEPSLEEP |

### 4.6 Measure Power Consumption

Objective

Measure DC current drawn by MVC using a Siglent SDM3065X.

Parameters

| Name | Type / Range | Description |
| --- | --- | --- |
| averaging_s | integer, default 5 | Settling/averaging time. |

Steps

| # | Action / Send | Expected Result / Receive |
| --- | --- | --- |
| 1 | Wire Siglent SDM3065X in series with the MVC supply line. | Siglent SDM3065X connected. |
| 2 | Configure the DMM for DC current measurement on the appropriate range (µA range for DEEPSLEEP, mA range for active). | DMM ready. |
| 3 | Record reading after averaging_s seconds. | Power consumption visible and stable. |

Payload Reference Dictionary

| Payload Type | Parameter / State | Hex Value |
| --- | --- | --- |
| Threshold | DEEPSLEEP | I ≤ 100 µA |
| Threshold | Active | I > 100 µA, typically 50–500 mA |

Source / References

| Requirement / TC ID | Title / Topic |
| --- | --- |
| MVC-95010 | Power consumption in DEEPSLEEP state |
| MVC-95100 | DEEPSLEEP power consumption |

### 4.7 Check IP Configuration

Objective

Verify MVC completed DHCP IP configuration (ACK Recieved).

Parameters

| Name | Type / Range | Description |
| --- | --- | --- |
| timeout_s | integer, default 10 | Capture timeout. |

Steps

| # | Action / Send | Expected Result / Receive |
| --- | --- | --- |
| 1 | Wireshark filter: bootp or dhcp. | DHCP packets visible. |
| 2 | Confirm DORA: DISCOVER → OFFER → REQUEST → ACK. | All messages consistent for assigned IP. |

Payload Reference Dictionary

| Payload Type | Parameter / State | Hex Value |
| --- | --- | --- |
| Result | IP configuration Finished | DHCPACK observed |
| Result | IP configuration Not Finished | No DHCPACK |
| Frame | DHCPDISCOVER | 0.0.0.0:68 → 255.255.255.255:67 |
| Frame | DHCPOFFER | DHCP_server:67 → broadcast:68 |
| Frame | DHCPREQUEST | Option 50 = requested IP |
| Frame | DHCPACK | ACK for yiaddr |

Source / References

| Requirement / TC ID | Title / Topic |
| --- | --- |
| MVC-102320 | DHCP starts at LINK_UP |
| MVC-102510 | DHCP transition to OPERATIONAL |

### 4.8 Check RTP Stream

Objective

Verify presence/absence and properties of RTP video stream.

Parameters

| Name | Type / Range | Description |
| --- | --- | --- |
| expected_state | Active / Inactive | Whether RTP is expected. |
| expected_width | pixels, optional | Expected width. |
| expected_height | pixels, optional | Expected height. |
| expected_framerate | fps, optional | Expected frame rate. |

Steps

| # | Action / Send | Expected Result / Receive |
| --- | --- | --- |
| 1 | Wireshark filter: rtp; capture ≥2 s. | Active: RTP packets visible; Inactive: none. |
| 2 | decode via GStreamer or similar. | Width/height/framerate match expected. |

Source / References

| Requirement / TC ID | Title / Topic |
| --- | --- |
| MVC-93359 | ROI resolution/framerate table |

### 4.9 Monitor SD OfferService

Objective

Confirm whether MVC publishes cyclic SOME/IP-SD OfferService messages.

Parameters

| Name | Type / Range | Description |
| --- | --- | --- |
| expected_state | Active / Inactive | Whether OfferService should be present. |
| capture_duration_s | integer, default 2 | Capture window. |

Steps

| # | Action / Send | Expected Result / Receive |
| --- | --- | --- |
| 1 | Wireshark filter: someip-sd AND someip.method == 0x8100 (or someip.serviceid == 0xFFFF). | Capture ready. |
| 2 | Capture and count OfferService entries. | Active: cadence 100 ms ±20%; Inactive: zero. |
| 3 | Inspect SD message header and entries-array Service ID. | SD header Service ID = 0xFFFF, Method ID = 0x8100. Entries-array Service ID = 0x433F. |

Payload Reference Dictionary

| Payload Type | Parameter / State | Hex Value |
| --- | --- | --- |
| SD message header | Service ID | 0xFFFF |
| SD message header | Method ID | 0x8100 |
| SD message header | Message Type | 0x02 (Notification) |
| SD message header | Flags | 0xC0 (Reboot + Unicast) |
| Entries array | Type | 0x01 (OfferService) |
| Entries array | Service ID | 0x433F (camera service) |
| Entries array | Instance ID | 0x00 ¦¦ last_byte(MVC_IP) |
| Entries array | TTL | 0x000001 (1 second) |
| Timing | Cycle time (main phase) | 100 ms ± 20 % |

Source / References

| Requirement / TC ID | Title / Topic |
| --- | --- |
| MVC-92749 | OfferService — content |
| MVC-93333 | OfferService — Message header and entries |
| MVC-93064 | OfferService — package fields |
| MVC-92513 | OfferService — timing |

### 4.10 Open / Close Shutter By Hand

Objective

Manually open/close shutter lid to verify mechanical operation and firmware detection.

Parameters

| Name | Type / Range | Description |
| --- | --- | --- |
| action | Open / Close | Mechanical action. |
| powered | true / false | Whether camera is powered. |

Steps

| # | Action / Send | Expected Result / Receive |
| --- | --- | --- |
| 1 | Using bare fingers, gently perform action. | Lid moves smoothly. |
| 2 | If powered, call getCamControl (0x50). | Manual Mode reflects position; Automatic may re-drive. |

Source / References

| Requirement / TC ID | Title / Topic |
| --- | --- |
| MVC-92331 | Lens protection mechanism — Electrical / Mechanical Control |

### 4.11 Flash Alternative NVM Parameter Set

Objective

Write an alternative value to a NVM-resident parameter (e.g. macAddress, serialNumber, numOfRegionOfInterest, cameraServicePort, T_imager_protection_on / off, etc.), then start up the camera so the new value is loaded.

Steps

| # | Action / Send | Expected Result / Receive |
| --- | --- | --- |
| 1 | Open SW Configuration file | SOW2_sw_config_00012759_PAR10C0.bin Opened and Addresses are shown |
| 2 | Modify <Target Paramter> Value from mapping NVM Software Configuration block Memory Map NVM Production Data block Memory Map | <Target Paramter> Value Updated |
| 3 | Flash NVM | MVC , Reboot NVM Updated |

### 4.13 Read Imager Temperature

Objective

Read the imager temperature (Timager) from the test bench, for cross-check against requirements-side thresholds

Steps

| # | Action / Send | Expected Result / Receive |
| --- | --- | --- |
| 1 | Execute System Startup. end subscribeROIVideo with roi_index. Confirm RTP stream active. | MVC RUNNING. Positive response. Stream present. |
|  | Send (getCamStatus) | Check Receive Payload Last 2B Receive Payload: 7 bytes (lockedBy 4B + powerStatus 1B + temperature 2B) |
| 2 | Decode payload at fixed offsets. | offset 5–6: imager temperature (uint16). |
| 3 | Apply conversion formula to convert raw → °C. | Temperature in °C. |

Payload Reference Dictionary

| Payload Type | Parameter / State | Hex Value |
| --- | --- | --- |
| Imager — register | atopif_tmc_cal_top, address 0x602C (per SAR MVC-92489) | RO, 14-bit (conversion formula: TBD with MEKRA) |
| Serializer — interface | serDes I2C (per SAR MVC-99303) | Conversion: T_Ser = T_EST_OUT / 2 − 273 °C |

Notes

Without a documented host-side read path, heating-protection TCs rely on the chamber's ambient as a proxy for component temperature plus a bench-side equilibration assumption. Decision pending from MEKRA: is the chamber-ambient proxy acceptable for series acceptance, or is a debug-interface read mandated?

Source / References

| Requirement / TC ID | Title / Topic |
| --- | --- |
| MVC-92489 | Imager temperature register location (SAR) |

### 4.15 RTP Encoding / GStreamer Pipeline for ROI Verification (OPEN)

Objective

Decode the RTP stream produced by subscribeROIVideo and assert observed resolution, framerate, and codec match the expected values for the subscribed ROI.

Parameters

| Name | Type / Range | Description |
| --- | --- | --- |
| expected_codec | H.264 / JPEG / RAW / TBD | Codec produced by SOW1 — TBD with MEKRA. |
| expected_width | pixels | From ROI definition (variant-dependent). |
| expected_height | pixels | From ROI definition. |
| expected_framerate | fps | From ROI definition. |

Steps

| # | Action / Send | Expected Result / Receive |
| --- | --- | --- |
| 1 | Start an RTP capture (Wireshark filter rtp), or feed UDP socket directly to GStreamer. | Capture ready. |
| 2 | apply the reference GStreamer pipeline | pipeline runs without errors |
| 3 | Read observed codec, resolution, framerate from decoder output. | Values match expected_codec / expected_width / expected_height / expected_framerate. |

Source / References

| Requirement / TC ID | Title / Topic |
| --- | --- |
| MVC-93359 | ROI resolution / framerate table |

# 5. State Procedures

### 5.1 System Startup

Objective

Bring MVC from fully OFF to RUNNING (OPERATIONAL with no active video stream).

Steps

| # | Action / Send | Expected Result / Receive |
| --- | --- | --- |
| 1 | Set host PHY link UP and confirm (4.3). | Host PHY UP. |
| 2 | Set Temperature Champer (5.2) to 20 | Temp is 20 |
| 3 | Apply 24.0 V supply (4.1). | MVC powers up. |
| 4 | Start DHCP server. | Server listening. |
| 5 | Check IP Configuration (4.7) with 10 s timeout. | IP configuration Finished; valid IP assigned. |
| 6 | Monitor SD OfferService (4.9) for 2 s. | OfferService cyclic at 100 ms ±20 %. |
| 7 | Check getCamStatus Heating | Heating is Automatic , OFF |
| 8 | Check getCamStatus Shutter | Shutter is Automatic , OFF |
| 9 | Check Check RTP Stream | No Stream Active |

Source / References

| Requirement / TC ID | Title / Topic |
| --- | --- |
| MVC-102240 | MVC Powerup |

### 5.2 Camera Init With Temperature

Objective

Bring MVC online with chamber stabilized at given ambient temperature.

Parameters

| Name | Type / Range | Description |
| --- | --- | --- |
| T_init | −40 °C – +85 °C | Target ambient. |
| U_test | 9.0 V – 36.0 V, default 24.0 | Supply voltage. |

Steps

| # | Action / Send | Expected Result / Receive |
| --- | --- | --- |
| 1 | Start DHCP server. | Listening. |
| 2 | Apply 0.0 V so chamber transition occurs with MVC OFF. | MVC powered down. |
| 3 | Set chamber to T_init and wait stable. | Stable ±0.5 °C for ≥5 min. |
| 4 | Apply U_test. | MVC powers up. |
| 5 | Check IP Configuration (4.7). | Finished. |
| 6 | Monitor SD OfferService (4.9). | Active. |

### 5.3 Goto OPERATIONAL_RUNNING

Objective

Drive MVC into OPERATIONAL_RUNNING: services available, no active stream.

Steps

| # | Action / Send | Expected Result / Receive |
| --- | --- | --- |
| 1 | Execute System Startup (5.1). | MVC up; OfferService running. |
| 2 | Call getCamStatus (3.3) and Check RTP Stream (4.8) inactive. | Positive status; no RTP traffic. |

### 5.4 Goto OPERATIONAL_STREAMING

Objective

Drive MVC into OPERATIONAL_STREAMING with active RTP stream.

Parameters

| Name | Type / Range | Description |
| --- | --- | --- |
| roi_index | 0 – ROI count−1, default 0 | ROI to subscribe. |

Steps

| # | Action / Send | Expected Result / Receive |
| --- | --- | --- |
| 1 | Execute System Startup (5.1). | MVC RUNNING. |
| 2 | Send subscribeROIVideo (3.8) with roi_index. | Positive response. |
| 3 | Confirm RTP stream active (4.8). | Stream present at expected resolution/framerate. |

### 5.5 Goto DEEPSLEEP

Objective

Drive MVC into DEEPSLEEP via TC10 LPS request.

Steps

| # | Action / Send | Expected Result / Receive |
| --- | --- | --- |
| 1 | Execute System Startup (5.1). | MVC RUNNING. |
| 2 | Send TC10 LPS (4.5). | DEEPSLEEP within ~1 s. |
| 3 | Monitor SD OfferService (4.9). | No messages. |
| 4 | Measure Power Consumption (4.6). | I ≤ 100 µA. |

### 5.6 Goto PHYLINKCHECK

Objective

Drive MVC into PHYLINKCHECK: powered ON with PHY DOWN.

Steps

| # | Action / Send | Expected Result / Receive |
| --- | --- | --- |
| 1 | Apply 0.0 V (4.1). | MVC OFF. |
| 2 | Set host PHY DOWN (4.3). | PHY DOWN confirmed. |
| 3 | Apply 24.0 V (4.1). | MVC powered. |
| 4 | Check IP Configuration (4.7). | Not Finished. |
| 5 | Monitor SD OfferService (4.9). | No messages. |

### 5.7 Goto IP_CONFIGURATION

Objective

Drive MVC into IP_CONFIGURATION: PHY UP, DHCP not completing.

Steps

| # | Action / Send | Expected Result / Receive |
| --- | --- | --- |
| 1 | Stop DHCP server. | No DHCP service. |
| 2 | Apply 0.0 V (4.1). | MVC OFF. |
| 3 | Set host PHY UP (4.3). | PHY UP confirmed. |
| 4 | Apply 24.0 V (4.1). | MVC powered. |
| 5 | Wireshark bootp for 10 s. | DHCPDISCOVER visible, no OFFER/ACK. |
| 6 | Monitor SD OfferService (4.9). | No messages. |

# 6. Blank Template

Copy this skeleton for a new function. Keep the same field order so the document stays consistent. Assign a stable bookmark in the form fn_<short_name> and add it to the index in section 1.2.

Objective

[One sentence: what the function does, and the SOME/IP method or bench action it uses.]

Parameters (optional)

| Name | Type / Range | Description |
| --- | --- | --- |
| [name] | [type / range] | [description] |

Steps

| # | Action / Send | Expected Result / Receive |
| --- | --- | --- |
| 1 | [Action or one-field-per-line Send frame] | [Expected result or one-field-per-line Receive frame] |
| 2 | […] | […] |

Payload Reference Dictionary

| Payload Type | Parameter / State | Hex Value |
| --- | --- | --- |
| [Sent_Payload] | [parameter name] | [hex] |
| [Received_Payload] | [state name] | [hex] |

Negative Response / Error Codes

| Trigger Condition | Return Code | Error Name |
| --- | --- | --- |
| [When the function fails] | [0xXX] | [E_NAME] |

Notes

[Clarifications, related functions, environment dependencies.]

Source / References

| Requirement / TC ID | Title / Topic |
| --- | --- |
| [MVC-xxxxx] | [Polarion title or source feature TC] |