Technical level: what it adds to the concept

SOW1 - SOW2 secure communication. The 30 technical requirements (TECH_CS_CONCEPT) compared with the 25 concept requirements (CS_CONCEPT).

1. In short

The concept level says what must be true. The technical level says how to verify it, with numbered test steps.

18 of the 30 have the same text as their concept requirement. 5 differ slightly. 4 add new detail. 3 are new (row configuration).

Of our 87 open SIQs, 3 are now answered and 15 are partly answered. 69 stay open.

The technical level does not define the registers, frame layout, timers or reactions that we ask about. It also adds new gaps (section 4).

2. The 30 technical requirements

Colour: green = same text, blue = small difference, yellow = adds detail, orange = new. Order as in your list (T1 to T30).

| # | Technical | Concept | Type | What it says or adds |
| --- | --- | --- | --- | --- |
| T1 | MVC-106582 | MVC-119709 | Same text as concept | Same text. Wrong footer tag and changed frame data must both fail; full 128-bit compare. |
| T2 | MVC-106575 | MVC-103031 | Adds detail (new information) | Freshness, ordering and sequence check with the header counter; "counter-validation policy" not defined. Content is closer to MVC-113703. |
| T3 | MVC-106563 | MVC-107642 | Adds detail (new information) | New: RSASSA-PSS with SHA-256 and a 2048-bit key. Concept MVC-107641 has the same topic but no linked technical requirement. |
| T4 | MVC-106560 | MVC-103042 | Same text as concept | Same text. Steps include injecting a TRNG failure (software level). |
| T5 | MVC-106578 | MVC-103034 | Small wording difference | Adds: over I2C, the term Row_info, and no transmission if authentication or encryption fails. Message layout is "specified" but not defined. |
| T6 | MVC-106569 | MVC-103051 | Small wording difference | Concept: configured maximum. Here: fixed three attempts. |
| T7 | MVC-106581 | MVC-103044 | Same text as concept | Same text. SSK not used for protected traffic; a new establishment can start again ("recovery conditions" not defined). |
| T8 | MVC-106562 | MVC-103063 | Same text as concept | Same text and criteria. One step is about Secure Boot, which is another feature. |
| T9 | MVC-106567 | MVC-103046 | Same text as concept | Same text. Counts requests on I2C; step 5 says "configured limit" while the text says three. |
| T10 | MVC-106566 | MVC-103053 | Small wording difference | Concept: retry the process. Here: retransmit the authentication request up to three times. |
| T11 | MVC-106573 | MVC-103036 | Same text as concept | Same text. Detailed steps per trigger; the old SSK must no longer be accepted. |
| T12 | MVC-106561 | MVC-107642 | Adds detail (new information) | New: SOW2 requests Device_n, KID and Sig_device over I2C after reset or before the session. Not in the concept. |
| T13 | MVC-113727 | MVC-113703 | Same text as concept | Same text. Boundary tests; LastAcceptedCounter is updated after the frame is authenticated. |
| T14 | MVC-106564 | MVC-119710 | Same text as concept | Same text. A rejected frame must not reach downstream processing as trusted. |
| T15 | MVC-106571 | MVC-113706 | Same text as concept | Same text. Lists the interfaces to check (diagnostic, logs, debug, communication, memory). |
| T16 | MVC-109120 | MVC-119708 | Same text as concept | Same text. Frames are trusted only after session set-up and the integrity and freshness checks. |
| T17 | MVC-109121 | MVC-113702 | Small wording difference | Says "protected" commands. Adds a counter maximum and rollover policy and the start value after a new session, but defines neither. |
| T18 | MVC-106568 | MVC-103057 | Same text as concept | Same text. New SSK per session; no key generation after a failed authentication. |
| T19 | MVC-113726 | MVC-103031 | Same text as concept | Same text. A malformed or missing header counter must not authorize a frame. |
| T20 | MVC-106572 | MVC-113705 | Same text as concept | Same text. Exactly 128 bits from the footer; a malformed footer is not treated as extracted. |
| T21 | MVC-106574 | MVC-103055 | New: row-information configuration | New: row range limit start + 1 + (interval + 1) x (total - 1) <= image + embedded data lines. |
| T22 | MVC-106580 | MVC-103055 | Same text as concept | Same text as the first part of the concept (the concept adds a Samsung note). Criteria need the private key to decrypt. |
| T23 | MVC-106576 | MVC-103055 | New: row-information configuration | New: row start 0 or 1. Start 0 puts the header line with the frame counter into the CMAC input. |
| T24 | MVC-108741 | MVC-103055 | New: row-information configuration | New: row interval at least 10 (merge modes) or 30 (line reorder RAW12). |
| T25 | MVC-106583 | MVC-103033 | Same text as concept | Same text. Needs a known SSK and an independent reference tag. |
| T26 | MVC-108740 | MVC-113701 | Same text as concept | Same text. Needs a known SSK; "specified authenticated input" is not defined. |
| T27 | MVC-106565 | MVC-107642 | Adds detail (new information) | New: the signature covers Device_n and KID; the root key is RSA-2048. |
| T28 | MVC-106570 | MVC-119707 | Same text as concept | Same text. The tag follows the command; the field order is "specified" but not defined. |
| T29 | MVC-109119 | MVC-119714 | Small wording difference | Says "protected" control commands; the concept says all control commands. |
| T30 | MVC-106577 | MVC-103040 | Same text as concept | Same text. A missing or malformed key must not be used; the "reference key" question returns. |

3. What it gives our open SIQs

3 answered, 15 partly answered. The comment "partly" means the technical requirement helps, but the value or policy is still not defined.

| Concept requirement | SIQ | Status | What the technical level gives |
| --- | --- | --- | --- |
| MVC-107642 | SIQ-2 | Answered | The signed message is Device_n and KID (MVC-106565). |
| MVC-107641 | SIQ-3 | Answered | RSA key size 2048 bits (MVC-106563). |
| MVC-113703 | SIQ-2 | Answered | LastAcceptedCounter is updated after the frame is authenticated (MVC-113727). |
| MVC-103053 | SIQ-1 | Partly | SOW2 retransmits the authentication request (MVC-106566). What counts as failed is still open. |
| MVC-103051 | SIQ-2 | Partly | A new establishment can start again, under undefined "recovery conditions" (MVC-106581). |
| MVC-103051 | SIQ-3 | Partly | The technical level says a fixed three attempts (MVC-106569); the concept says configured. |
| MVC-103044 | SIQ-1 | Partly | The SSK is not used for protected traffic (MVC-106581); deletion is not stated. |
| MVC-119709 | SIQ-3 | Partly | With row start 0 the header line with the frame counter is part of the CMAC input (MVC-106576). |
| MVC-119709 | SIQ-6 | Partly | "Incorrect" = changed footer tag, "modified" = changed frame data (MVC-106582); "invalid" is not covered. |
| MVC-103031 | SIQ-3 | Partly | Invalid counter data must not authorize a frame (MVC-113726, MVC-106575); the policy is not defined. |
| MVC-113705 | SIQ-1 | Partly | An invalid tag is not treated as extracted (MVC-106572); the policy is not defined. |
| MVC-119708 | SIQ-2 | Partly | Trusted only after session set-up plus integrity and freshness checks (MVC-109120); the start of the stream is open. |
| MVC-119714 | SIQ-1 | Partly | The technical level says "protected" commands only (MVC-109119). |
| MVC-119707 | SIQ-1 | Partly | The tag follows the command (MVC-106570); the field format is not defined. |
| MVC-103042 | SIQ-5 | Partly | Steps include injecting a TRNG failure (MVC-106560), so a software-level negative test is expected. |
| MVC-103046 | SIQ-1 | Partly | Retransmissions are counted on I2C with the response suppressed (MVC-106567); the expected response is not defined. |
| MVC-107641 | SIQ-5 | Partly | Inspection of the configuration plus a modified-data test (MVC-106563). |

4. New gaps found at the technical level

Concept MVC-107641 (RSASSA-PSS, SHA-256) has no linked technical requirement. MVC-106563, which covers it, is linked to MVC-107642.

Five technical requirements are linked from the concept but are not in our export: MVC-113724, MVC-106579, MVC-108739, MVC-113723, MVC-113725.

Fixed or configured retry limit: "three" in the text, "configured limit" in MVC-106567 step 5 and in concept MVC-103051. Three attempts (MVC-106569) and three retransmissions (MVC-106566) differ by one.

Three row-configuration technical requirements are linked to the encryption concept MVC-103055. The concept has no row-selection requirement.

MVC-106575 is linked to MVC-103031 (extract counter) but its content is freshness validation (MVC-113703).

Several criteria refer to things that do not exist yet: "approved interface specification" (MVC-106580), "specified message layout" (MVC-106578, MVC-106570), "counter-validation policy" (MVC-106575), "rollover policy" (MVC-109121).

MVC-106563 numbers its criteria 1, 2, 4, 5 (step 3 is missing). MVC-109119 says "protected" commands, the concept says all commands.

5. Next

Review the 30 technical requirements. For those with the same text, the comment is "Similar to concept REQ: MVC-xxxxx". Only differences and new items get their own SIQs.

Send the five missing technical requirements (MVC-113724, MVC-106579, MVC-108739, MVC-113723, MVC-113725).

المستوى التقني: ما يضيفه إلى المفهوم

التواصل الآمن SOW1 - SOW2. مقارنة 30 متطلباً تقنياً (TECH_CS_CONCEPT) مع 25 متطلباً مفاهيمياً (CS_CONCEPT).

1. باختصار

المستوى المفاهيمي يقول ما يجب أن يتحقق. المستوى التقني يقول كيف نتحقق منه، بخطوات اختبار مرقّمة.

18 من 30 لها نفس نص متطلب المفهوم. 5 تختلف قليلاً. 4 تضيف تفاصيل جديدة. 3 جديدة (إعداد الصفوف).

من أسئلتنا الـ 87 المفتوحة، أُجيب 3 وأُجيب 15 جزئياً. ويبقى 69 مفتوحاً.

المستوى التقني لا يعرّف السجلات ولا شكل الإطار ولا المؤقتات ولا ردود الفعل التي نسأل عنها. كما يضيف فجوات جديدة (القسم 4).

2. المتطلبات التقنية الثلاثون

الألوان: أخضر = نفس النص، أزرق = فرق صغير، أصفر = يضيف تفاصيل، برتقالي = جديد. الترتيب كما في قائمتك (T1 إلى T30).

| # | التقني | المفهوم | النوع | ما يقوله أو يضيفه |
| --- | --- | --- | --- | --- |
| T1 | MVC-106582 | MVC-119709 | نفس نص المفهوم | نفس النص. يجب أن يفشل التحقق عند تغيير الـ tag أو بيانات الإطار، مع مقارنة الـ 128 بت كاملة. |
| T2 | MVC-106575 | MVC-103031 | يضيف تفاصيل (معلومة جديدة) | فحص الحداثة والترتيب والتسلسل بعداد الإطار؛ "سياسة التحقق من العداد" غير معرّفة. المحتوى أقرب إلى MVC-113703. |
| T3 | MVC-106563 | MVC-107642 | يضيف تفاصيل (معلومة جديدة) | جديد: RSASSA-PSS مع SHA-256 ومفتاح 2048 بت. متطلب المفهوم MVC-107641 له نفس الموضوع لكن بلا متطلب تقني مرتبط. |
| T4 | MVC-106560 | MVC-103042 | نفس نص المفهوم | نفس النص. الخطوات تتضمن حقن عطل في TRNG (مستوى البرمجيات). |
| T5 | MVC-106578 | MVC-103034 | فرق صغير في الصياغة | يضيف: عبر I2C، ومصطلح Row_info، وعدم الإرسال إذا فشلت المصادقة أو التشفير. شكل الرسالة "محدد" لكنه غير معرّف. |
| T6 | MVC-106569 | MVC-103051 | فرق صغير في الصياغة | المفهوم: حد أقصى قابل للضبط. هنا: ثلاث محاولات ثابتة. |
| T7 | MVC-106581 | MVC-103044 | نفس نص المفهوم | نفس النص. لا يُستخدم الـ SSK للاتصال المحمي؛ يمكن بدء إنشاء جديد ("شروط الاسترداد" غير معرّفة). |
| T8 | MVC-106562 | MVC-103063 | نفس نص المفهوم | نفس النص والمعايير. خطوة واحدة عن Secure Boot وهي ميزة أخرى. |
| T9 | MVC-106567 | MVC-103046 | نفس نص المفهوم | نفس النص. عدّ الطلبات على I2C؛ الخطوة 5 تقول "الحد المضبوط" بينما النص يقول ثلاثاً. |
| T10 | MVC-106566 | MVC-103053 | فرق صغير في الصياغة | المفهوم: إعادة العملية. هنا: إعادة إرسال طلب المصادقة حتى ثلاث مرات. |
| T11 | MVC-106573 | MVC-103036 | نفس نص المفهوم | نفس النص. خطوات مفصلة لكل محفز؛ ويجب ألا يُقبل الـ SSK القديم. |
| T12 | MVC-106561 | MVC-107642 | يضيف تفاصيل (معلومة جديدة) | جديد: SOW2 يطلب Device_n وKID وSig_device عبر I2C بعد إعادة التشغيل أو قبل الجلسة. غير موجود في المفهوم. |
| T13 | MVC-113727 | MVC-113703 | نفس نص المفهوم | نفس النص. اختبارات الحدود؛ ويُحدَّث LastAcceptedCounter بعد مصادقة الإطار. |
| T14 | MVC-106564 | MVC-119710 | نفس نص المفهوم | نفس النص. الإطار المرفوض يجب ألا يصل للمعالجة اللاحقة كموثوق. |
| T15 | MVC-106571 | MVC-113706 | نفس نص المفهوم | نفس النص. يسرد الواجهات المطلوب فحصها (التشخيص، السجلات، التنقيح، الاتصال، الذاكرة). |
| T16 | MVC-109120 | MVC-119708 | نفس نص المفهوم | نفس النص. الإطارات موثوقة فقط بعد إنشاء الجلسة وفحوص السلامة والحداثة. |
| T17 | MVC-109121 | MVC-113702 | فرق صغير في الصياغة | يقول أوامر "محمية". يضيف حداً أقصى للعداد وسياسة الالتفاف وقيمة البدء بعد جلسة جديدة، دون تعريفها. |
| T18 | MVC-106568 | MVC-103057 | نفس نص المفهوم | نفس النص. SSK جديد لكل جلسة؛ ولا يُولَّد مفتاح بعد فشل المصادقة. |
| T19 | MVC-113726 | MVC-103031 | نفس نص المفهوم | نفس النص. عداد الترويسة التالف أو المفقود يجب ألا يسمح بقبول إطار. |
| T20 | MVC-106572 | MVC-113705 | نفس نص المفهوم | نفس النص. 128 بت بالضبط من التذييل؛ والتذييل التالف لا يُعد مستخرجاً. |
| T21 | MVC-106574 | MVC-103055 | جديد: إعداد معلومات الصفوف | جديد: حد نطاق الصفوف start + 1 + (interval + 1) x (total - 1) <= أسطر الصورة + البيانات المضمنة. |
| T22 | MVC-106580 | MVC-103055 | نفس نص المفهوم | نفس نص الجزء الأول من المفهوم (المفهوم يضيف ملاحظة Samsung). المعايير تحتاج المفتاح الخاص لفك التشفير. |
| T23 | MVC-106576 | MVC-103055 | جديد: إعداد معلومات الصفوف | جديد: بداية الصفوف 0 أو 1. البداية 0 تُدخل سطر الترويسة مع عداد الإطار في مدخل الـ CMAC. |
| T24 | MVC-108741 | MVC-103055 | جديد: إعداد معلومات الصفوف | جديد: الفاصل بين الصفوف 10 على الأقل (أوضاع الدمج) أو 30 (إعادة ترتيب الأسطر RAW12). |
| T25 | MVC-106583 | MVC-103033 | نفس نص المفهوم | نفس النص. يحتاج SSK معلوماً وقيمة مرجعية مستقلة. |
| T26 | MVC-108740 | MVC-113701 | نفس نص المفهوم | نفس النص. يحتاج SSK معلوماً؛ و"المدخل الموثّق المحدد" غير معرّف. |
| T27 | MVC-106565 | MVC-107642 | يضيف تفاصيل (معلومة جديدة) | جديد: التوقيع يشمل Device_n وKID؛ والمفتاح الجذر RSA-2048. |
| T28 | MVC-106570 | MVC-119707 | نفس نص المفهوم | نفس النص. الـ tag يلي الأمر؛ وترتيب الحقول "محدد" لكنه غير معرّف. |
| T29 | MVC-109119 | MVC-119714 | فرق صغير في الصياغة | يقول أوامر تحكم "محمية"؛ المفهوم يقول كل أوامر التحكم. |
| T30 | MVC-106577 | MVC-103040 | نفس نص المفهوم | نفس النص. المفتاح المفقود أو التالف لا يُستخدم؛ وسؤال "المفتاح المرجعي" يتكرر. |

3. ماذا يعطي لأسئلتنا المفتوحة

3 أُجيب، 15 أُجيب جزئياً. "جزئياً" تعني أن المتطلب التقني يساعد لكن القيمة أو السياسة ما زالت غير معرّفة.

| متطلب المفهوم | السؤال | الحالة | ما يعطيه المستوى التقني |
| --- | --- | --- | --- |
| MVC-107642 | SIQ-2 | تمت الإجابة | الرسالة الموقّعة هي Device_n وKID (MVC-106565). |
| MVC-107641 | SIQ-3 | تمت الإجابة | حجم مفتاح RSA هو 2048 بت (MVC-106563). |
| MVC-113703 | SIQ-2 | تمت الإجابة | يُحدَّث LastAcceptedCounter بعد مصادقة الإطار (MVC-113727). |
| MVC-103053 | SIQ-1 | جزئية | SOW2 يعيد إرسال طلب المصادقة (MVC-106566). تعريف الفشل ما زال مفتوحاً. |
| MVC-103051 | SIQ-2 | جزئية | يمكن بدء إنشاء جديد بشروط استرداد غير معرّفة (MVC-106581). |
| MVC-103051 | SIQ-3 | جزئية | المستوى التقني يقول ثلاث محاولات ثابتة (MVC-106569)؛ المفهوم يقول قابلة للضبط. |
| MVC-103044 | SIQ-1 | جزئية | لا يُستخدم الـ SSK للاتصال المحمي (MVC-106581)؛ والحذف غير مذكور. |
| MVC-119709 | SIQ-3 | جزئية | مع بداية الصفوف 0 يكون سطر الترويسة مع عداد الإطار جزءاً من مدخل الـ CMAC (MVC-106576). |
| MVC-119709 | SIQ-6 | جزئية | "غير صحيح" = تغيير الـ tag، "معدّل" = تغيير بيانات الإطار (MVC-106582)؛ "غير صالح" غير مغطى. |
| MVC-103031 | SIQ-3 | جزئية | بيانات العداد غير الصالحة يجب ألا تُقبل الإطار (MVC-113726, MVC-106575)؛ والسياسة غير معرّفة. |
| MVC-113705 | SIQ-1 | جزئية | الـ tag غير الصالح لا يُعد مستخرجاً (MVC-106572)؛ والسياسة غير معرّفة. |
| MVC-119708 | SIQ-2 | جزئية | موثوق فقط بعد إنشاء الجلسة وفحوص السلامة والحداثة (MVC-109120)؛ وبداية البث مفتوحة. |
| MVC-119714 | SIQ-1 | جزئية | المستوى التقني يقول الأوامر "المحمية" فقط (MVC-109119). |
| MVC-119707 | SIQ-1 | جزئية | الـ tag يلي الأمر (MVC-106570)؛ وصيغة الحقول غير معرّفة. |
| MVC-103042 | SIQ-5 | جزئية | الخطوات تتضمن حقن عطل في TRNG (MVC-106560)، فاختبار سلبي على مستوى البرمجيات متوقع. |
| MVC-103046 | SIQ-1 | جزئية | تُعدّ إعادة الإرسال على I2C مع منع الاستجابة (MVC-106567)؛ والاستجابة المتوقعة غير معرّفة. |
| MVC-107641 | SIQ-5 | جزئية | فحص الإعدادات مع اختبار البيانات المعدّلة (MVC-106563). |

4. فجوات جديدة في المستوى التقني

المتطلب MVC-107641 (RSASSA-PSS وSHA-256) ليس له متطلب تقني مرتبط. المتطلب MVC-106563 الذي يغطيه مرتبط بـ MVC-107642.

خمسة متطلبات تقنية مرتبطة من المفهوم غير موجودة في ملف التصدير: MVC-113724 وMVC-106579 وMVC-108739 وMVC-113723 وMVC-113725.

حد المحاولات ثابت أم قابل للضبط: "ثلاث" في النص، و"الحد المضبوط" في MVC-106567 الخطوة 5 وفي MVC-103051. ثلاث محاولات (MVC-106569) وثلاث إعادات (MVC-106566) تختلفان بواحد.

ثلاثة متطلبات تقنية لإعداد الصفوف مرتبطة بمتطلب التشفير MVC-103055. المفهوم لا يحتوي متطلب اختيار الصفوف.

MVC-106575 مرتبط بـ MVC-103031 (استخراج العداد) لكن محتواه التحقق من الحداثة (MVC-113703).

معايير عدة تشير إلى ما لم يُعرَّف بعد: "مواصفة الواجهة المعتمدة" (MVC-106580)، و"شكل الرسالة المحدد" (MVC-106578, MVC-106570)، و"سياسة التحقق من العداد" (MVC-106575)، و"سياسة الالتفاف" (MVC-109121).

MVC-106563 يرقّم معاييره 1 و2 و4 و5 (الخطوة 3 مفقودة). MVC-109119 يقول أوامر "محمية" بينما المفهوم يقول كل الأوامر.

5. الخطوة التالية

مراجعة المتطلبات التقنية الثلاثين. للمتطلبات المطابقة يكون التعليق "مشابه لمتطلب المفهوم: MVC-xxxxx". الفروقات والعناصر الجديدة فقط تأخذ أسئلتها الخاصة.

إرسال المتطلبات التقنية الخمسة الناقصة (MVC-113724 وMVC-106579 وMVC-108739 وMVC-113723 وMVC-113725).